HC11

MC68HC11F1

Technical Data

Freescale Semiconductor, Inc...

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TECHNICAL DATA

Paragraph Title Page

SECTION 1INTRODUCTION

1.1 Features ....................................................................................................1-1

SECTION 2 PIN DESCRIPTIONS

2.1 VDD and VSS ..............................................................................................2-2

2.2 Reset (RESET) ..........................................................................................2-3

2.3 E-Clock Output (E) ....................................................................................2-3

2.4 Crystal Driver and External Clock Input (XTAL, EXTAL) ...........................2-3

2.5 Four Times E-Clock Frequency Output (4XOUT) .....................................2-5

2.6 Interrupt Request (IRQ) .............................................................................2-5

2.7 Non-Maskable Interrupt (XIRQ) .................................................................2-5

2.8 MODA and MODB (MODA/LIR and MODB/VSTBY) ..................................2-6

2.9 VRH and VRL ..............................................................................................2-6

2.10 R/W ...........................................................................................................2-6

2.11 Port Signals ...............................................................................................2-6

2.11.1 Port A ................................................................................................2-7

2.11.2 Port B ................................................................................................2-8

2.11.3 Port C ................................................................................................2-8

2.11.4 Port D ................................................................................................2-8

2.11.5 Port E ................................................................................................2-9

2.11.6 Port F .................................................................................................2-9

2.11.7 Port G ................................................................................................2-9

SECTION 3 CENTRAL PROCESSING UNIT

3.1 CPU Registers ...........................................................................................3-1

3.1.1 Accumulators A, B, and D .................................................................3-2

3.1.2 Index Register X (IX) .........................................................................3-3

3.1.3 Index Register Y (IY) .........................................................................3-3

3.1.4 Stack Pointer (SP) .............................................................................3-3

3.1.5 Program Counter (PC) ......................................................................3-5

3.1.6 Condition Code Register (CCR) ........................................................3-5

3.1.6.1 Carry/Borrow (C) .......................................................................3-5

3.1.6.2 Overflow (V) ..............................................................................3-5

3.1.6.3 Zero (Z) .....................................................................................3-6

3.1.6.4 Negative (N) ..............................................................................3-6

3.1.6.5 Interrupt Mask (I) .......................................................................3-6

3.1.6.6 Half Carry (H) ............................................................................3-6

3.1.6.7 X Interrupt Mask (X) ..................................................................3-6

3.1.6.8 Stop Disable (S) ........................................................................3-7

TABLE OF CONTENTS

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TECHNICAL DATA

(Continued)

Paragraph Title Page

TABLE OF CONTENTS

3.2 Data Types ................................................................................................3-7

3.3 Opcodes and Operands ............................................................................3-7

3.4 Addressing Modes .....................................................................................3-7

3.4.1 Immediate ..........................................................................................3-7

3.4.2 Direct .................................................................................................3-8

3.4.3 Extended ...........................................................................................3-8

3.4.4 Indexed ..............................................................................................3-8

3.4.5 Inherent .............................................................................................3-8

3.4.6 Relative .............................................................................................3-8

3.5 Instruction Set ...........................................................................................3-8

SECTION 4OPERATING MODES AND ON-CHIP MEMORY

4.1 Operating Modes .......................................................................................4-1

4.1.1 Single-Chip Operating Mode .............................................................4-1

4.1.2 Expanded Operating Mode ...............................................................4-1

4.1.3 Special Test Mode .............................................................................4-1

4.1.4 Special Bootstrap Mode ....................................................................4-1

4.2 On-Chip Memory .......................................................................................4-2

4.2.1 Mapping Allocations ..........................................................................4-2

4.2.2 Memory Map .....................................................................................4-3

4.2.2.1 RAM ..........................................................................................4-3

4.2.2.2 Bootloader ROM .......................................................................4-4

4.2.2.3 EEPROM ...................................................................................4-4

4.2.3 Registers ...........................................................................................4-4

4.3 System Initialization ...................................................................................4-6

4.3.1 Mode Selection ..................................................................................4-7

4.3.1.1 HPRIO Register ........................................................................4-8

4.3.2 Initialization ........................................................................................4-9

4.3.2.1 CONFIG Register ......................................................................4-9

4.3.2.2 INIT Register ...........................................................................4-10

4.3.2.3 OPTION Register ....................................................................4-11

4.3.2.4 OPT2 Register ........................................................................4-12

4.3.2.5 Block Protect Register (BPROT) .............................................4-13

4.4 EEPROM and CONFIG Register ............................................................4-14

4.4.1 EEPROM .........................................................................................4-14

4.4.1.1 EEPROM Programming ..........................................................4-14

4.4.1.2 EEPROM Bulk Erase ..............................................................4-15

4.4.1.3 EEPROM Row Erase ..............................................................4-15

4.4.1.4 EEPROM Byte Erase ..............................................................4-16

4.4.2 PPROG EEPROM Programming Control Register .........................4-16

4.4.3 CONFIG Register Programming .....................................................4-17

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TECHNICAL DATA

(Continued)

Paragraph Title Page

TABLE OF CONTENTS

4.5 Chip Selects ............................................................................................4-18

4.5.1 Program Chip Select .......................................................................4-18

4.5.2 I/O Chip Selects ..............................................................................4-18

4.5.3 General-Purpose Chip Select ..........................................................4-19

SECTION 5 RESETS AND INTERRUPTS

5.1 Resets .......................................................................................................5-1

5.1.1 Power-On Reset ................................................................................5-1

5.1.2 External Reset (RESET) ...................................................................5-1

5.1.3 Computer Operating Properly (COP) Reset ......................................5-2

5.1.4 Clock Monitor Reset ..........................................................................5-2

5.1.5 OPTION Register ..............................................................................5-3

5.1.6 CONFIG Register ..............................................................................5-4

5.2 Effects of Reset .........................................................................................5-4

5.2.1 Central Processing Unit .....................................................................5-5

5.2.2 Memory Map .....................................................................................5-5

5.2.3 Parallel I/O .........................................................................................5-5

5.2.4 Timer .................................................................................................5-5

5.2.5 Real-Time Interrupt (RTI) ..................................................................5-5

5.2.6 Pulse Accumulator ............................................................................5-6

5.2.7 Computer Operating Properly (COP) ................................................5-6

5.2.8 Serial Communications Interface (SCI) .............................................5-6

5.2.9 Serial Peripheral Interface (SPI) ........................................................5-6

5.2.10 Analog-to-Digital Converter ...............................................................5-6

5.2.11 System ..............................................................................................5-6

5.3 Reset and Interrupt Priority .......................................................................5-6

5.3.1 Highest Priority Interrupt and Miscellaneous Register ......................5-7

5.4 Interrupts ...................................................................................................5-8

5.4.1 Interrupt Recognition and Register Stacking .....................................5-9

5.4.2 Non-Maskable Interrupt Request (XIRQ) ........................................5-10

5.4.3 Illegal Opcode Trap .........................................................................5-10

5.4.4 Software Interrupt ............................................................................5-11

5.4.5 Maskable Interrupts .........................................................................5-11

5.4.6 Reset and Interrupt Processing .......................................................5-11

5.5 Low Power Operation ..............................................................................5-16

5.5.1 WAIT ...............................................................................................5-17

5.5.2 STOP ...............................................................................................5-17

SECTION 6 PARALLEL INPUT/OUTPUT

6.1 Port A ........................................................................................................6-1

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TECHNICAL DATA

(Continued)

Paragraph Title Page

TABLE OF CONTENTS

6.2 Port B ........................................................................................................6-2

6.3 Port C ........................................................................................................6-2

6.4 Port D ........................................................................................................6-3

6.5 Port E ........................................................................................................6-4

6.6 Port F .........................................................................................................6-4

6.7 Port G ........................................................................................................6-5

6.8 System Configuration Options 2 ................................................................6-5

SECTION 7 SERIAL COMMUNICATIONS INTERFACE

7.1 Data Format ..............................................................................................7-1

7.2 Transmit Operation ....................................................................................7-1

7.3 Receive Operation .....................................................................................7-2

7.4 Wakeup Feature ........................................................................................7-4

7.4.1 Idle-Line Wakeup ..............................................................................7-4

7.4.2 Address-Mark Wakeup ......................................................................7-4

7.5 SCI Error Detection ...................................................................................7-5

7.6 SCI Registers ............................................................................................7-5

7.6.1 Serial Communications Data Register ..............................................7-5

7.6.2 Serial Communications Control Register 1 .......................................7-5

7.6.3 Serial Communications Control Register 2 .......................................7-6

7.6.4 Serial Communication Status Register ..............................................7-7

7.6.5 Baud Rate Register ...........................................................................7-8

7.7 Status Flags and Interrupts .....................................................................7-10

7.7.1 Receiver Flags ................................................................................7-11

SECTION 8 SERIAL PERIPHERAL INTERFACE

8.1 Functional Description ...............................................................................8-1

8.2 SPI Transfer Formats ................................................................................8-2

8.2.1 Clock Phase and Polarity Controls ....................................................8-3

8.3 SPI Signals ................................................................................................8-3

8.3.1 Master In Slave Out ...........................................................................8-4

8.3.2 Master Out Slave In ...........................................................................8-4

8.3.3 Serial Clock .......................................................................................8-4

8.3.4 Slave Select ......................................................................................8-4

8.4 SPI System Errors .....................................................................................8-4

8.5 SPI Registers ............................................................................................8-5

8.5.1 Serial Peripheral Control ...................................................................8-5

8.5.2 Serial Peripheral Status .....................................................................8-7

8.5.3 Serial Peripheral Data Register .........................................................8-7

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TECHNICAL DATA

(Continued)

Paragraph Title Page

TABLE OF CONTENTS

SECTION 9 TIMING SYSTEM

9.1 Timer Structure ..........................................................................................9-3

9.2 Input Capture .............................................................................................9-5

9.2.1 Timer Control Register 2 ...................................................................9-5

9.2.2 Timer Input Capture Registers ..........................................................9-6

9.2.3 Timer Input Capture 4/Output Compare 5 Register ..........................9-6

9.3 Output Compare ........................................................................................9-6

9.3.1 Timer Output Compare Registers .....................................................9-7

9.3.2 Timer Compare Force Register .........................................................9-8

9.3.3 Output Compare Mask Registers ......................................................9-8

9.3.4 Output Compare Data Register .........................................................9-9

9.3.5 Timer Counter Register .....................................................................9-9

9.3.6 Timer Control Register 1 ...................................................................9-9

9.3.7 Timer Interrupt Mask Register 1 ......................................................9-10

9.3.8 Timer Interrupt Flag Register 1 .......................................................9-11

9.3.9 Timer Interrupt Mask Register 2 ......................................................9-11

9.3.10 Timer Interrupt Flag Register 2 .......................................................9-12

9.4 Real-Time Interrupt .................................................................................9-12

9.4.1 Timer Interrupt Mask Register 2 ......................................................9-13

9.4.2 Timer Interrupt Flag Register 2 .......................................................9-14

9.4.3 Pulse Accumulator Control Register ...............................................9-14

9.5 Computer Operating Properly Watchdog Function .................................9-15

9.6 Pulse Accumulator ..................................................................................9-15

9.6.1 Pulse Accumulator Control Register ...............................................9-16

9.6.2 Pulse Accumulator Count Register .................................................9-17

9.6.3 Pulse Accumulator Status and Interrupt Bits ...................................9-18

SECTION 10 ANALOG-TO-DIGITAL CONVERTER

10.1 Overview .................................................................................................10-1

10.1.1 Multiplexer .......................................................................................10-1

10.1.2 Analog Converter ............................................................................10-3

10.1.3 Digital Control ..................................................................................10-3

10.1.4 Result Registers ..............................................................................10-3

10.1.5 A/D Converter Clocks ......................................................................10-4

10.1.6 Conversion Sequence .....................................................................10-4

10.2 A/D Converter Power-Up and Clock Select .............................................10-5

10.3 Conversion Process ................................................................................10-5

10.4 Channel Assignments .............................................................................10-6

10.5 Single-Channel Operation .......................................................................10-6

10.6 Multiple-Channel Operation .....................................................................10-6

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TECHNICAL DATA

(Continued)

Paragraph Title Page

TABLE OF CONTENTS

10.7 Operation in STOP and WAIT Modes ....................................................10-7

10.8 A/D Control/Status Registers ..................................................................10-7

10.9 A/D Converter Result Registers ..............................................................10-8

APPENDIX A ELECTRICAL CHARACTERISTICS

APPENDIX BMECHANICAL DATA AND ORDERING INFORMATION

B.1 Pin Assignments ....................................................................................... B-1

B.2 Package Dimensions ................................................................................ B-2

B.3 Ordering Information ................................................................................ B-3

APPENDIX CDEVELOPMENT SUPPORT

C.1 MC68HC11F1 Development Tools .......................................................... C-1

C.2 MC68HC11EVS — Evaluation System .................................................... C-1

C.3 M68MMDS11 — Modular Development System for M68HC11 Devices . C-1

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TECHNICAL DATA

Figure Title Page

1-1 MC68HC11F1 Block Diagram ........................................................................ 1-2

2-1 Pin Assignments for MC68HC11F1 68-Pin PLCC ......................................... 2-1

2-2 Pin Assignments for MC68HC11F1 80-Pin QFP ............................................ 2-2

2-3 External Reset Circuit ..................................................................................... 2-3

2-4 Common Crystal Connections ........................................................................ 2-4

2-5 External Oscillator Connections ..................................................................... 2-4

2-6 One Crystal Driving Two MCUs ..................................................................... 2-4

2-7 4XOUT Signal Driving a Second MCU ........................................................... 2-5

3-1 Programming Model ....................................................................................... 3-2

3-2 Stacking Operations ....................................................................................... 3-4

4-1 MC68HC11F1 Memory Map .......................................................................... 4-3

4-2 RAM Standby MODB/VSTBY Connections ..................................................... 4-4

4-3 Address Map for I/O and Program Chip Selects .......................................... 4-19

4-4 Address Map for General-Purpose Chip Select ........................................... 4-20

5-1 Processing Flow Out of Reset (1 of 2) ......................................................... 5-12

5-2 Processing Flow Out of Reset (2 of 2) ......................................................... 5-13

5-3 Interrupt Priority Resolution (1 of 2) ............................................................. 5-14

5-4 Interrupt Priority Resolution (2 of 2) ............................................................. 5-15

5-5 Interrupt Source Resolution Within SCI ........................................................ 5-16

7-1 SCI Transmitter Block Diagram ...................................................................... 7-2

7-2 SCI Receiver Block Diagram .......................................................................... 7-3

7-3 SCI Baud Rate Generator Block Diagram .................................................... 7-10

7-4 Interrupt Source Resolution Within SCI ........................................................ 7-12

8-1 SPI Block Diagram ......................................................................................... 8-2

8-2 SPI Transfer Format ....................................................................................... 8-3

9-1 Timer Clock Divider Chains ............................................................................ 9-2

9-2 Capture/Compare Block Diagram .................................................................. 9-4

9-3 Pulse Accumulator ....................................................................................... 9-16

10-1 A/D Converter Block Diagram ...................................................................... 10-2

10-2 Electrical Model of an A/D Input Pin (Sample Mode) ................................... 10-3

10-3 A/D Conversion Sequence ........................................................................... 10-4

A-1 Test Methods ..................................................................................................A-4

A-2 Timer Inputs ...................................................................................................A-5

A-3 POR External Reset Timing Diagram .............................................................A-6

A-4 STOP Recovery Timing Diagram ...................................................................A-7

A-5 WAIT Recovery from Interrupt Timing Diagram .............................................A-8

A-6 Interrupt Timing Diagram ................................................................................A-9

A-7 Port Read Timing Diagram ...........................................................................A-10

A-8 Port Write Timing Diagram ...........................................................................A-10

A-9 Expansion Bus Timing ..................................................................................A-13

A-10 SPI Master Timing (CPHA = 0) ....................................................................A-15

LIST OF ILLUSTRATIONS

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TECHNICAL DATA

(Continued)

Figure Title Page

LIST OF ILLUSTRATIONS

A-11 SPI Master Timing (CPHA = 1) ....................................................................A-15

A-12 SPI Slave Timing (CPHA = 0) ......................................................................A-16

A-13 SPI Slave Timing (CPHA = 1) ......................................................................A-16

B-1 MC68HC11F1 68-Pin PLCC ..........................................................................B-1

B-2 MC68HC11F1 80-Pin Quad Flat Pack ...........................................................B-2

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TECHNICAL DATA

Table Title Page

2-1 Port Signal Functions ...................................................................................... 2-7

3-1 Reset Vector Comparison ............................................................................... 3-5

3-2 Instruction Set ................................................................................................. 3-9

4-1 Register and Control Bit Assignments............................................................. 4-5

4-2 Write Access Limited Registers....................................................................... 4-7

4-3 Hardware Mode Select Summary ................................................................... 4-7

4-4 EEPROM Mapping........................................................................................ 4-10

4-5 RAM and Register Mapping .......................................................................... 4-11

4-6 EEPROM Block Protection............................................................................ 4-14

4-7 EEPROM Erase Mode Control...................................................................... 4-17

4-8 Chip Select Clock Stretch Control ................................................................. 4-20

4-9 Program Chip Select Size Control................................................................. 4-21

4-10 General-Purpose Chip Select Starting Address ............................................ 4-22

4-11 General-Purpose Chip Select Size Control ................................................... 4-22

4-12 Chip Select Control Parameter Summary ..................................................... 4-23

5-1 COP Timer Rate Selection .............................................................................. 5-2

5-2 Reset Cause, Operating Mode, and Reset Vector .......................................... 5-4

5-3 Highest Priority Interrupt Selection.................................................................. 5-8

5-4 Interrupt and Reset Vector Assignments......................................................... 5-9

5-5 Stacking Order on Entry to Interrupts............................................................ 5-10

6-1 I/O Port Configuration...................................................................................... 6-1

7-1 Baud Rate Prescaler Selection ....................................................................... 7-8

7-2 Baud Rate Selection........................................................................................ 7-9

8-1 SPI Clock Rates .............................................................................................. 8-6

9-1 Timer Summary............................................................................................... 9-3

9-2 Timer Output Compare Configuration ........................................................... 9-10

9-3 Timer Prescaler Selection ............................................................................. 9-12

9-4 RTI Rate Selection ........................................................................................ 9-13

9-5 Pulse Accumulator Timing............................................................................. 9-16

9-6 Pulse Accumulator Edge Detection Control .................................................. 9-17

10-1 A/D Converter Channel Assignments............................................................ 10-6

10-2 A/D Converter Channel Selection ................................................................. 10-8

A-1 Maximum Ratings............................................................................................A-1

A-2 Thermal Characteristics ..................................................................................A-2

A-3 DC Electrical Characteristics...........................................................................A-3

A-4 Control Timing .................................................................................................A-5

A-5 Peripheral Port Timing...................................................................................A-10

A-6 Analog-To-Digital Converter Characteristics .................................................A-11

A-7 Expansion Bus Timing...................................................................................A-12

A-8 Serial Peripheral Interface Timing .................................................................A-14

A-9 EEPROM Characteristics ..............................................................................A-17

LIST OF TABLES

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TECHNICAL DATA

(Continued)

Table Title Page

LIST OF TABLES

B-1 Device Ordering Information ...........................................................................B-3

C-1 MC68HC11F1 Development Tools .................................................................C-1

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

INTRODUCTION

TECHNICAL DATA 1-1

SECTION 1 INTRODUCTION

The MC68HC11F1 high-performance microcontroller unit (MCU) is an enhanced derivative of the M68HC11 family of microcontrollers and includes many advanced features. This MCU, with a nonmultiplexed expanded bus, is characterized by high speed and low power consumption. The fully static design allows operation at frequencies from 4 MHz to dc.

1.1 Features

• M68HC11 Central Processing Unit (CPU)

• Power Saving STOP and WAIT Modes

• 512 Bytes Electrically Erasable Programmable Read-Only Memory (EEPROM)

• 1024 Bytes RAM, Data Retained During Standby

• Nonmultiplexed Address and Data Buses

• Enhanced 16-Bit Timer

• Three Input Capture (IC) Channels

• Four Output Compare (OC) Channels

• One Additional Channel, Selectable as Fourth IC or Fifth OC

• 8-Bit Pulse Accumulator

• Real-Time Interrupt Circuit

• Computer Operating Properly (COP) Watchdog

• Enhanced Asynchronous Non Return to Zero (NRZ) Serial Communications Interface (SCI)

• Enhanced Synchronous Serial Peripheral Interface (SPI)

• Eight-Channel 8-Bit Analog-to-Digital (A/D) Converter

• Four Chip-Select Signal Outputs with Programmable Clock Stretching

— Two I/O Chip Selects

— One Program Chip Select

— One General-Purpose Chip Select

• Available in 68-Pin Plastic Leaded Chip Carrier (PLCC) and 80-Pin Plastic Quad Flat Pack (QFP)

Figure 1-1 MC68HC11F1 Block Diagram

SECTION 2 PIN DESCRIPTIONS

The MC68HC11F1 MCU is available in a 68-pin plastic leaded chip carrier (PLCC) and an 80-pin plastic quad flat pack (QFP). Most pins on this MCU serve two or more functions, as described in the following paragraphs. Figure 2-1 shows the pin assignments for the PLCC. Figure 2-2 shows the pin assignments for the QFP.

Figure 2-1 Pin Assignments for MC68HC11F1 68-Pin PLCC

Figure 2-2 Pin Assignments for MC68HC11F1 80-Pin QFP

2.1 VDD and VSS

Power is supplied to the MCU through VDD and VSS. VDD is the power supply, and VSS is ground. The MCU operates from a single 5-volt (nominal) power supply. Very fast signal transitions occur on the MCU pins. The short rise and fall times place high, short duration current demands on the power supply. To prevent noise problems, provide good power-supply bypassing at the MCU. Also, use bypass capacitors that have good high-frequency characteristics and situate them as close to the MCU as possible.  Bypass requirements vary, depending on how heavily the MCU pins are loaded.

2.2 Reset (RESET)

An active low bidirectional control signal, RESET, acts as an input to initialize the MCU to a known start-up state. It also acts as an open-drain output to indicate that an internal failure has been detected in either the clock monitor or COP watchdog circuit. The CPU distinguishes between internal and external reset conditions by sensing whether the reset pin rises to a logic one in less than two E-clock cycles after a reset has occurred. It is not advisable to connect an external resistor-capacitor (RC) power-up delay circuit to the reset pin of M68HC11 devices because the circuit charge time constant can cause the device to misinterpret the type of reset that occurred. Refer to SECTION 5 RESETS AND INTERRUPTS for further information.

Figure 2-3 illustrates a reset circuit that uses an external switch. Other circuits can be used, however, it is important to incorporate a low voltage interrupt (LVI) circuit to prevent operation at insufficient voltage levels which could result in erratic behavior or corruption of RAM.

Figure 2-3 External Reset Circuit

2.3 E-Clock Output (E)

E is the output connection for the internally generated E clock. The signal from E is used as a timing reference. The frequency of the E-clock output is one fourth that of the input frequency at the EXTAL pin. When E-clock output is low, an internal process is taking place. When it is high, data is being accessed. All clocks, including the E clock, are halted when the MCU is in STOP mode. The E clock can be turned off in single-chip modes to reduce the effects of radio frequency interference (RFI). Refer to SECTION 9 TIMING SYSTEM.

2.4 Crystal Driver and External Clock Input (XTAL, EXTAL)

These two pins provide the interface for either a crystal or a CMOS-compatible clock to control the internal clock generator circuitry. Either a crystal oscillator or a CMOS compatible clock can be used. The resulting E-clock rate is the input frequency divided by four.

The XTAL pin is normally left unterminated when an external CMOS compatible clock is connected to the EXTAL pin. However, a 10 k to 100 k load resistor connected from the XTAL output to ground can be used to reduce RFI noise emission.

The XTAL output is normally used to drive a crystal. The XTAL output can be buffered with a high-impedance buffer, or it can be used to drive the EXTAL input of another M68HC11 device. Refer to Figure 2-6.

In all cases, use caution when designing circuitry associated with the oscillator pins. Load capacitances shown in the oscillator circuits include all stray layout capacitances. Refer to Figure 2-4, Figure 2-5, and Figure 2-6.

Figure 2-4 Common Crystal Connections

Figure 2-5 External Oscillator Connections

Figure 2-6 One Crystal Driving Two MCUs

2.5 Four Times E-Clock Frequency Output (4XOUT)

Although the circuit shown in Figure 2-6 will work for any M68HC11 MCU, the MC68HC11F1 has an additional clock output that is four times the E-clock frequency. This output (4XOUT) can be used to directly drive the EXTAL input of another M68HC11 MCU. Refer to Figure 2-7. The 4XOUT output is enabled after reset and can be disabled by clearing the CLK4X bit in the OPT2 register.

Figure 2-7 4XOUT Signal Driving a Second MCU

2.6 Interrupt Request (IRQ)

The IRQ input provides a means of generating asynchronous interrupt requests for the CPU. Either falling-edge triggering or low-level triggering is selected by the IRQE bit in the OPTION register. IRQ is always configured for level-sensitive triggering at reset. Connect an external pull-up resistor, typically 4.7 kW, to VDD when IRQ is used in a level-sensitive wired-OR configuration. Refer to SECTION 5 RESETS AND INTERRUPTS.

2.7 Non-Maskable Interrupt (XIRQ)

The XIRQ input provides a means of requesting a non-maskable interrupt after reset initialization. During reset, the X bit in the condition code register (CCR) is set and any interrupt is masked until MCU software enables it. Because the XIRQ input is level sensitive, it can be connected to a multiple-source wired-OR network with an external pull-up resistor to VDD. XIRQ is often used as a power loss detect interrupt.

Whenever XIRQ or IRQ are used with multiple interrupt sources (IRQ must be configured for level-sensitive operation if there is more than one source of IRQ interrupt), each source must drive the interrupt input with an open-drain type of driver to avoid contention between outputs. There should be a single pull-up resistor near the MCU interrupt input pin (typically 4.7K). There must also be an interlock mechanism at each interrupt source so that the source holds the interrupt line low until the MCU recognizes and acknowledges the interrupt request. If one or more interrupt sources are still pending after the MCU services a request, the interrupt line will still be held low and the MCU will be interrupted again as soon as the interrupt mask bit in the condition code register (CCR) is cleared (normally upon return from an interrupt). Refer to SECTION 5 RESETS AND INTERRUPTS.

2-6 TECHNICAL DATA

2.8 MODA and MODB (MODA/LIR and MODB/VSTBY)

During reset, MODA and MODB select one of the four operating modes. Refer to SECTION 4 OPERATING MODES AND ON-CHIP MEMORY.

After the operating mode has been selected, the LIR pin provides an open-drain output to indicate that execution of an instruction has begun. The LIR pin is configured for wired-OR operation (only pulls low). A series of E-clock cycles occurs during execution of each instruction. The LIR signal is asserted (drives low) during the first E-clock cycle of each instruction (opcode fetch). This output is provided for assistance in program debugging.

The VSTBY pin is used to input RAM standby power. The MCU is powered from the VDD signal unless the difference between the level of VSTBY and Vdd is greater than one MOS threshold (about 0.7 volts). When these voltages differ by more than 0.7 volts, the internal 768-byte RAM and part of the reset logic are powered from VSTBY rather than VDD. This allows RAM contents to be retained without VDD power applied to the MCU. Reset must be driven low before VDD is removed and must remain low until VDD has been restored to a valid level.

2.9 VRH and VRL

These pins provide the reference voltage for the analog-to-digital converter. Bypass capacitors should be used to minimize noise on these signals. Any noise on VRH and VRL will directly affect A/D accuracy.

2.10 R/W

In expanded and test modes, R/W indicates the direction of transfers on the external data bus. A logic level one on this pin indicates that a read cycle is in progress. A logic zero on this pin indicates that a write cycle is in progress and that no external device should drive the data bus.

The E-clock can be used to enable external devices to drive data onto the data bus during the second half of a read bus cycle (E clock high). R/W can then be used to control the direction of data transfers. R/W drives low when data is being written to the external data bus. R/W will remain low during consecutive data bus write cycles, such as when a double-byte store occurs.

2.11 Port Signals

For the MC68HC11F1, 54 pins are arranged into six 8-bit ports: A, B, C, E, F, and G, and one 6-bit port (D). Each of these seven ports serves a purpose other than I/O, depending on the operating mode or peripheral functions selected. Note that ports B, C, and F are available for I/O functions only in single-chip and bootstrap modes. The pins of ports A, C, D, and G are fully bidirectional. Ports B and F are output-only ports. Port E is an input-only port. Refer to Table 2-1 for details about the 54 port signals’ functions within different operating modes.

2.11.1 Port A

Port A is an 8-bit general-purpose I/O port with a data register (PORTA) and a data direction register (DDRA). Port A pins share functions with the 16-bit timer system. PORTA can be read at any time. Inputs return the pin level; outputs return the pin driver input level. If written, PORTA stores the data in internal latches. It drives the pins only if they are configured as outputs. Writes to PORTA do not change the pin state when the pins are configured for timer output compares.

Out of reset, port A pins [7:0] are general-purpose high-impedance inputs. When the timer functions associated with these pins are disabled, the bits in DDRA govern the I/O state of the  associated pin. For further information, refer to SECTION 6 PARALLEL INPUT/OUTPUT.

NOTE

When using the information about port functions, do not confuse pin function with the electrical state of the pin at reset. All general-purpose I/O pins configured as inputs at reset are in a high-impedance state. Port data registers reflect the logic state of the port at reset. The pin function is mode dependent.

Table 2-1 Port Signal Functions

2.11.2 Port B

Port B is an 8-bit output-only port. In single-chip modes, port B pins are general-purpose output pins (PB[7:0]). In expanded modes, port B pins act as the high-order address lines (ADDR[15:8]) of the address bus.

PORTB can be read at any time. Reads of PORTB return the pin driver input level. If PORTB is written, the data is stored in internal latches. It drives the pins only in singlechip or bootstrap mode. In expanded operating modes, port B pins are the high-order address outputs (ADDR[15:8]).

Refer to SECTION 6 PARALLEL INPUT/OUTPUT.

2.11.3 Port C

Port C is an 8-bit general-purpose I/O port with a data register (PORTC) and a data direction register (DDRC). In single-chip modes, port C pins are general-purpose I/O pins (PC[7:0]). In expanded modes, port C pins are configured as data bus pins (DATA[7:0]).

PORTC can be read at any time. Inputs return the pin level; outputs return the pin driver input level. If PORTC is written, the data is stored in internal latches. It drives the pins only if they are configured as outputs in single-chip or bootstrap mode. Port C pins are general-purpose inputs out of reset in single-chip and bootstrap modes. In expanded and test modes, these pins are data bus lines out of reset.

The CWOM control bit in the OPT2 register disables port C’s P-channel output drivers. Because the N-channel driver is not affected by CWOM, setting CWOM causes port C to become an open-drain-type output port suitable for wired-OR operation. In wired-OR mode, (PORTC bits are at logic level zero), pins are actively driven low by the Nchannel driver. When a port C bit is at logic level one, the associated pin is in a high impedance state, as neither the N-channel nor the P-channel devices are active. It is customary to have an external pull-up resistor on lines that are driven by open-drain devices. Port C can only be configured for wired-OR operation when the MCU is in single-chip or bootstrap modes.

Refer to SECTION 6 PARALLEL INPUT/OUTPUT.

2.11.4 Port D

Port D, a 6-bit general-purpose I/O port, has a data register (PORTD) and a data direction

register (DDRD). The six port D lines (D[5:0]) can be used for general-purpose

I/O, for the serial communications interface (SCI) and serial peripheral interface (SPI)

subsystems.

PORTD can be read at any time. Inputs return the pin level; outputs return the pin driver

input level. If PORTD is written, the data is stored in internal latches and can be driven

only if port D is configured for general-purpose output.

The DWOM control bit in the SPCR register disables port D’s P-channel output drivers.

Because the N-channel driver is not affected by DWOM, setting DWOM causes port

D to become an open-drain-type output port suitable for wired-OR operation. In wired-

OR mode, (PORTD bits are at logic level zero), pins are actively driven low by the Nchannel

driver. When a port D bit is at logic level one, the associated pin is in a highimpedance

state, as neither the N-channel nor the P-channel devices are active. It is

customary to have an external pull-up resistor on lines that are driven by open-drain

devices. Port D can be configured for wired-OR operation in any operating mode.

Refer to SECTION 6 PARALLEL INPUT/OUTPUT, SECTION 7 SERIAL COMMUNICATIONS

INTERFACE, and SECTION 8 SERIAL PERIPHERAL INTERFACE.

2.11.5 Port E

Port E is an 8-bit input-only port that is also used as the analog input port for the analog-

to-digital converter. Port E pins that are not used for the A/D system can be used

as general-purpose inputs. However, PORTE should not be read during the sample

portion of an A/D conversion sequence.

Refer to SECTION 10 ANALOG-TO-DIGITAL CONVERTER.

2.11.6 Port F

Port F is an 8-bit output-only port. In single-chip mode, port F pins are general-purpose

output pins (PF[7:0]). In expanded mode, port F pins act as the low-order address outputs

(ADDR[7:0]).

PORTF can be read at any time. Reads of PORTF return the pin driver input level. If

PORTF is written, the data is stored in internal latches. It drives the pins only in singlechip

or bootstrap mode. In expanded operating modes, port F pins are the low-order

address outputs (ADDR[7:0]).

Refer to SECTION 6 PARALLEL INPUT/OUTPUT.

2.11.7 Port G

Port G is an 8-bit general-purpose I/O port. When enabled, four chip select signals are

alternate functions of port G bits [7:4].

PORTG can be read at any time. Inputs return the pin level; outputs return the pin driver

input level. If PORTG is written, the data is stored in internal latches. It drives the

pins only if they are configured as outputs.

The GWOM control bit in the OPT2 register disables port G's P-channel output drivers.

Because the N-channel driver is not affected by GWOM, setting GWOM causes port

G to become an open-drain-type output port suitable for wired-OR operation. In wired-

OR mode, (PORTG bits are at logic level zero), pins are actively driven low by the Nchannel

driver. When a port G bit is at logic level one, the associated pin is in a highimpedance

state, as neither the N-channel nor the P-channel devices are active. It is

customary to have an external pull-up resistor on lines that are driven by open-drain

devices. Port G can be configured for wired-OR operation in any operating mode.

Refer to SECTION 6 PARALLEL INPUT/OUTPUT and SECTION 4 OPERATING

MODES AND ON-CHIP MEMORY.

SECTION 3 CENTRAL PROCESSING UNIT

This section presents information on M68HC11 central processing unit (CPU) architecture.

Data types, addressing modes, the instruction set, and the extended addressing

range required to support this MCU’s memory expansion feature are also included,

as are special operations such as subroutine calls and interrupts.

The CPU is designed to treat all peripheral, I/O, and memory locations identically as

addresses in the 64 Kbyte memory map. This is referred to as memory-mapped I/O.

There are no special instructions for I/O that are separate from those used for memory.

This architecture also allows accessing an operand from an external memory location

with no execution-time penalty.

3.1 CPU Registers

M68HC11 CPU registers are an integral part of the CPU and are not addressed as if

they were memory locations. The seven registers, discussed in the following paragraphs,

are shown in Figure 3-1.

Figure 3-1 Programming Model

3.1.1 Accumulators A, B, and D

Accumulators A and B are general-purpose 8-bit registers that hold operands and results

of arithmetic calculations or data manipulations. For some instructions, these two

accumulators are treated as a single double-byte (16-bit) accumulator called accumulator

D. Although most instructions can use accumulators A or B interchangeably, the

following exceptions apply:

The ABX and ABY instructions add the contents of 8-bit accumulator B to the contents

of 16-bit register X or Y, but there are no equivalent instructions that use A instead of B.

The TAP and TPA instructions transfer data from accumulator A to the condition code

register, or from the condition code register to accumulator A, however, there are no

equivalent instructions that use B rather than A.

The decimal adjust accumulator A (DAA) instruction is used after binary-coded decimal

(BCD) arithmetic operations, but there is no equivalent BCD instruction to adjust

accumulator B.

The add, subtract, and compare instructions associated with both A and B (ABA, SBA,

and CBA) only operate in one direction, making it important to plan ahead to ensure

that the correct operand is in the correct accumulator.

3.1.2 Index Register X (IX)

The IX register provides a 16-bit indexing value that can be added to the 8-bit offset

provided in an instruction to create an effective address. The IX register can also be

used as a counter or as a temporary storage register.

3.1.3 Index Register Y (IY)

The 16-bit IY register performs an indexed mode function similar to that of the IX register.

However, most instructions using the IY register require an extra byte of machine

code and an extra cycle of execution time because of the way the opcode map is implemented.

Refer to 3.3 Opcodes and Operands for further information.

3.1.4 Stack Pointer (SP)

The M68HC11 CPU has an automatic program stack. This stack can be located anywhere

in the address space and can be any size up to the amount of memory available

in the system. Normally the SP is initialized by one of the first instructions in an application

program. The stack is configured as a data structure that grows downward from

high memory to low memory. Each time a new byte is pushed onto the stack, the SP

is decremented. Each time a byte is pulled from the stack, the SP is incremented. At

any given time, the SP holds the 16-bit address of the next free location in the stack.

Figure 3-2 is a summary of SP operations.

Figure 3-2 Stacking Operations

When a subroutine is called by a jump to subroutine (JSR) or branch to subroutine

(BSR) instruction, the address of the instruction after the JSR or BSR is automatically

pushed onto the stack, least significant byte first. When the subroutine is finished, a

return from subroutine (RTS) instruction is executed. The RTS pulls the previously

stacked return address from the stack, and loads it into the program counter. Execution

then continues at this recovered return address.

Address of next instruction in main program to be

executed upon return from subroutine.

Most significant byte of return address.

Least significant byte of return address.

8-bit direct address ($0000-$00FF) (high byte

assumed to be $00).

8-bit positive offset $00 (0) to $FF (256) is added

to index.

High-order byte of 16-bit extended address.

Low-order byte of 16-bit extended address.

Signed-relative offset $80 (-128) to $7F (+127)

(offset relative to the address following the

machine code offset byte).

JSR, JUMP TO SUBROUTINE

Shaded cells show stack pointer position after

operation is complete.

When an interrupt is recognized, the current instruction finishes normally, the return

address (the current value in the program counter) is pushed onto the stack, all of the

CPU registers are pushed onto the stack, and execution continues at the address

specified by the vector for the interrupt. At the end of the interrupt service routine, an

RTI instruction is executed. The RTI instruction causes the saved registers to be pulled

off the stack in reverse order. Program execution resumes at the return address.

There are instructions that push and pull the A and B accumulators and the X and Y

index registers. These instructions are often used to preserve program context. For

example, pushing accumulator A onto the stack when entering a subroutine that uses

accumulator A, and then pulling accumulator A off the stack just before leaving the

subroutine, ensures that the contents of a register will be the same after returning from

the subroutine as it was before starting the subroutine.

3.1.5 Program Counter (PC)

The program counter, a 16-bit register, contains the address of the next instruction to

be executed. After reset, the program counter is initialized from one of six possible

vectors, depending on operating mode and the cause of reset.

3.1.6 Condition Code Register (CCR)

This 8-bit register contains five condition code indicators (C, V, Z, N, and H), two interrupt

masking bits, (I and X) and a stop disable bit (S). In the M68HC11 CPU, condition

codes are automatically updated by most instructions. For example, load accumulator

A (LDAA) and store accumulator A (STAA) instructions automatically set or clear the

N, Z, and V condition code flags. Pushes, pulls, add B to X (ABX), add B to Y (ABY),

and transfer/exchange instructions do not affect the condition codes. Refer to Table

3-2, which shows what condition codes are affected by a particular instruction.

3.1.6.1 Carry/Borrow (C)

The C bit is set if the arithmetic logic unit (ALU) performs a carry or borrow during an

arithmetic operation. The C bit also acts as an error flag for multiply and divide operations.

Shift and rotate instructions operate with and through the carry bit to facilitate

multiple-word shift operations.

3.1.6.2 Overflow (V)

The overflow bit is set if an operation causes an arithmetic overflow. Otherwise, the V

bit is cleared.

Table 3-1 Reset Vector Comparison

3.1.6.3 Zero (Z)

The Z bit is set if the result of an arithmetic, logic, or data manipulation operation is

zero. Otherwise, the Z bit is cleared. Compare instructions do an internal implied subtraction

and the condition codes, including Z, reflect the results of that subtraction. A

few operations (INX, DEX, INY, and DEY) affect the Z bit and no other condition flags.

For these operations, only = and - conditions can be determined.

3.1.6.4 Negative (N)

The N bit is set if the result of an arithmetic, logic, or data manipulation operation is

negative (MSB = 1). Otherwise, the N bit is cleared. A result is said to be negative if

its most significant bit (MSB) is a one. A quick way to test whether the contents of a

memory location has the MSB set is to load it into an accumulator and then check the

status of the N bit.

3.1.6.5 Interrupt Mask (I)

The interrupt request (IRQ) mask (I bit) is a global mask that disables all maskable interrupt

sources. While the I bit is set, interrupts can become pending, but the operation

of the CPU continues uninterrupted until the I bit is cleared. After any reset, the I bit is

set by default and can only be cleared by a software instruction. When an interrupt is

recognized, the I bit is set after the registers are stacked, but before the interrupt vector

is fetched. After the interrupt has been serviced, a return from interrupt instruction is

normally executed, restoring the registers to the values that were present before the

interrupt occurred. Normally, the I bit is zero after a return from interrupt is executed.

Although the I bit can be cleared within an interrupt service routine, “nesting” interrupts

in this way should only be done when there is a clear understanding of latency and of

the arbitration mechanism. Refer to SECTION 5 RESETS AND INTERRUPTS.

3.1.6.6 Half Carry (H)

The H bit is set when a carry occurs between bits 3 and 4 of the arithmetic logic unit

during an ADD, ABA, or ADC instruction. Otherwise, the H bit is cleared. Half carry is

used during BCD operations.

3.1.6.7 X Interrupt Mask (X)

The XIRQ mask (X) bit disables interrupts from the XIRQ pin. After any reset, X is set

by default and must be cleared by a software instruction. When an XIRQ interrupt is

recognized, the X and I bits are set after the registers are stacked, but before the interrupt

vector is fetched. After the interrupt has been serviced, an RTI instruction is

normally executed, causing the registers to be restored to the values that were present

before the interrupt occurred. The X interrupt mask bit is set only by hardware (RESET

or XIRQ acknowledge). X is cleared only by program instruction (TAP, where the associated

bit of A is zero; or RTI, where bit 6 of the value loaded into the CCR from the

stack has been cleared). There is no hardware action for clearing X.

3.1.6.8 Stop Disable (S)

Setting the STOP disable (S) bit prevents the STOP instruction from putting the

M68HC11 into a low-power stop condition. If the CPU encounters a STOP instruction

while the S bit is set, it is treated as a no-operation (NOP) instruction, and processing

continues to the next instruction. S is set by reset — STOP disabled by default.

3.2 Data Types

The M68HC11 CPU supports the following data types:

• Bit data

• 8-bit and 16-bit signed and unsigned integers

• 16-bit unsigned fractions

• 16-bit addresses

A byte is eight bits wide and can be accessed at any byte location. A word is composed

of two consecutive bytes with the most significant byte at the lower value address. Because

the M68HC11 is an 8-bit CPU, there are no special requirements for alignment

of instructions or operands.

3.3 Opcodes and Operands

The M68HC11 family of microcontrollers uses 8-bit opcodes. Each opcode identifies

a particular instruction and associated addressing mode to the CPU. Several opcodes

are required to provide each instruction with a range of addressing capabilities. Only

256 opcodes would be available if the range of values were restricted to the number

able to be expressed in 8-bit binary numbers.

A four-page opcode map has been implemented to expand the number of instructions.

An additional byte, called a prebyte, directs the processor from page 0 of the opcode

map to one of the other three pages. As its name implies, the additional byte precedes

the opcode.

A complete instruction consists of a prebyte, if any, an opcode, and zero, one, two, or

three operands. The operands contain information the CPU needs for executing the

instruction. Complete instructions can be from one to five bytes long.

3.4 Addressing Modes

Six addressing modes can be used to access memory: immediate, direct, extended,

indexed, inherent, and relative. These modes are detailed in the following paragraphs.

All modes except inherent mode use an effective address. The effective address is the

memory address from which the argument is fetched or stored, or the address from

which execution is to proceed. The effective address can be specified within an instruction,

or it can be calculated.

3.4.1 Immediate

In the immediate addressing mode an argument is contained in the byte(s) immediately

following the opcode. The number of bytes following the opcode matches the size

of the register or memory location being operated on. There are two-, three-, and four-

(if prebyte is required) byte immediate instructions. The effective address is the address

of the byte following the instruction.

3.4.2 Direct

In the direct addressing mode, the low-order byte of the operand address is contained

in a single byte following the opcode, and the high-order byte of the address is assumed

to be $00. Addresses $00–$FF are thus accessed directly, using two-byte instructions.

Execution time is reduced by eliminating the additional memory access

required for the high-order address byte. In most applications, this 256-byte area is reserved

for frequently referenced data. In M68HC11 MCUs, the memory map can be

configured for combinations of internal registers, RAM, or external memory to occupy

these addresses.

3.4.3 Extended

In the extended addressing mode, the effective address of the argument is contained

in two bytes following the opcode byte. These are three-byte instructions (or four-byte

instructions if a prebyte is required). One or two bytes are needed for the opcode and

two for the effective address.

3.4.4 Indexed

In the indexed addressing mode, an 8-bit unsigned offset contained in the instruction

is added to the value contained in an index register (IX or IY). The sum is the effective

address. This addressing mode allows referencing any memory location in the 64

Kbyte address space. These are two- to five-byte instructions, depending on whether

or not a prebyte is required.

3.4.5 Inherent

In the inherent addressing mode, all the information necessary to execute the instruction

is contained in the opcode. Operations that use only the index registers or accumulators,

as well as control instructions with no arguments, are included in this

addressing mode. These are one- or two-byte instructions.

3.4.6 Relative

The relative addressing mode is used only for branch instructions. If the branch condition

is true, an 8-bit signed offset included in the instruction is added to the contents

of the program counter to form the effective branch address. Otherwise, control proceeds

to the next instruction. These are usually two-byte instructions.

3.5 Instruction Set

Refer to Table 3-2, which shows all the M68HC11 instructions in all possible addressing

modes. For each instruction, the table shows the operand construction, the number

of machine code bytes, and execution time in CPU E clock cycles.

Table 3-2 Instruction Set (Sheet 1 of 6)

Mnemonic Operation Description Addressing Instruction Condition Codes

Mode Opcode Operand Cycles S X H I N Z V C

ABA Add

Accumulators

A + B Þ A INH 1B — 2 — — D — D D D D

ABX Add B to X IX + (00 : B) Þ IX INH 3A — 3 — — — — — — — —

ABY Add B to Y IY + (00 : B) Þ IY INH 18 3A — 4 — — — — — — — —

ADCA (opr) Add with Carry

to A

A + M + C Þ A A IMM

A DIR

A EXT

A IND,X

A IND,Y

89

99

B9

A9

18 A9

ii

dd

hh ll

ff

ff

23445

— — D — D D D D

ADCB (opr) Add with Carry

to B

B + M + C Þ B B IMM

B DIR

B EXT

B IND,X

B IND,Y

C9

D9

F9

E9

18 E9

ii

dd

hh ll

ff

ff

23445

— — D — D D D D

ADDA (opr) Add Memory

to A

A + M Þ A A IMM

A DIR

A EXT

A IND,X

A IND,Y

8B

9B

BB

AB

18 AB

ii

dd

hh ll

ff

ff

23445

— — D — D D D D

ADDB (opr) Add Memory

to B

B + M Þ B B IMM

B DIR

B EXT

B IND,X

B IND,Y

CB

DB

FB

EB

18 EB

ii

dd

hh ll

ff

ff

23445

— — D — D D D D

ADDD (opr) Add 16-Bit to D D + (M : M + 1) Þ D IMM

DIR

EXT

IND,X

IND,Y

C3

D3

F3

E3

18 E3

jj kk

dd

hh ll

ff

ff

45667

— — — — D D D D

ANDA (opr) AND A with

Memory

A • M Þ A A IMM

A DIR

A EXT

A IND,X

A IND,Y

84

94

B4

A4

18 A4

ii

dd

hh ll

ff

ff

23445

— — — — D D 0 —

ANDB (opr) AND B with

Memory

B • M Þ B B IMM

B DIR

B EXT

B IND,X

B IND,Y

C4

D4

F4

E4

18 E4

ii

dd

hh ll

ff

ff

23445

— — — — D D 0 —

ASL (opr) Arithmetic

Shift Left

EXT

IND,X

IND,Y

78

68

18 68

hh ll

ff

ff

667

— — — — D D D D

ASLA Arithmetic

Shift Left A

A INH 48 — 2 — — — — D D D D

ASLB Arithmetic

Shift Left B

B INH 58 — 2 — — — — D D D D

ASLD Arithmetic

Shift Left D

INH 05 — 3 — — — — D D D D

ASR Arithmetic

Shift Right

EXT

IND,X

IND,Y

77

67

18 67

hh ll

ff

ff

667

— — — — D D D D

ASRA Arithmetic

Shift Right A

A INH 47 — 2 — — — — D D D D

ASRB Arithmetic

Shift Right B

B INH 57 — 2 — — — — D D D D

BCC (rel) Branch if Carry

Clear

? C = 0 REL 24 rr 3 — — — — — — — —

BCLR (opr)

(msk)

Clear Bit(s) M • (mm) Þ M DIR

IND,X

IND,Y

15

1D

18 1D

dd mm

ff mm

ff mm

678

— — — — D D 0 —

BCS (rel) Branch if Carry

Set

? C = 1 REL 25 rr 3 — — — — — — — —

BEQ (rel) Branch if =

Zero

? Z = 1 REL 27 rr 3 — — — — — — — —

BGE (rel) Branch if D

Zero

? N Å V = 0 REL 2C rr 3 — — — — — — — —

C

0

b7 b0

C

0

b7 b0

C

0

b7 b0

C

0

b7 A b0 b7 B b0

b7 b0 C

b7 b0 C

b7 b0 C

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

CENTRAL PROCESSING UNIT MC68HC11F1

3-10 TECHNICAL DATA

BGT (rel) Branch if >

Zero

? Z + (N Å V) = 0 REL 2E rr 3 — — — — — — — —

BHI (rel) Branch if

Higher

? C + Z = 0 REL 22 rr 3 — — — — — — — —

BHS (rel) Branch if

Higher or

Same

? C = 0 REL 24 rr 3 — — — — — — — —

BITA (opr) Bit(s) Test A

with Memory

A • M A IMM

A DIR

A EXT

A IND,X

A IND,Y

85

95

B5

A5

18 A5

ii

dd

hh ll

ff

ff

23445

— — — — D D 0 —

BITB (opr) Bit(s) Test B

with Memory

B • M B IMM

B DIR

B EXT

B IND,X

B IND,Y

C5

D5

F5

E5

18 E5

ii

dd

hh ll

ff

ff

23445

— — — — D D 0 —

BLE (rel) Branch if D

Zero

? Z + (N Å V) = 1 REL 2F rr 3 — — — — — — — —

BLO (rel) Branch if

Lower

? C = 1 REL 25 rr 3 — — — — — — — —

BLS (rel) Branch if

Lower or

Same

? C + Z = 1 REL 23 rr 3 — — — — — — — —

BLT (rel) Branch if <

Zero

? N Å V = 1 REL 2D rr 3 — — — — — — — —

BMI (rel) Branch if

Minus

? N = 1 REL 2B rr 3 — — — — — — — —

BNE (rel) Branch if not =

Zero

? Z = 0 REL 26 rr 3 — — — — — — — —

BPL (rel) Branch if Plus ? N = 0 REL 2A rr 3 — — — — — — — —

BRA (rel) Branch Always ? 1 = 1 REL 20 rr 3 — — — — — — — —

BRCLR(opr)

(msk)

(rel)

Branch if

Bit(s) Clear

? M • mm = 0 DIR

IND,X

IND,Y

13

1F

18 1F

dd mm rr

ff mm rr

ff mm rr

678

— — — — — — — —

BRN (rel) Branch Never ? 1 = 0 REL 21 rr 3 — — — — — — — —

BRSET(opr)

(msk)

(rel)

Branch if Bit(s)

Set

? (M) • mm = 0 DIR

IND,X

IND,Y

12

1E

18 1E

dd mm rr

ff mm rr

ff mm rr

678 —

—

—

—

—

—

—

—

BSET (opr)

(msk)

Set Bit(s) M + mm Þ M DIR

IND,X

IND,Y

14

1C

18 1C

dd mm

ff mm

ff mm

678

— — — — D D 0 —

BSR (rel) Branch to

Subroutine

See Figure 3–2 REL 8D rr 6 — — — — — — — —

BVC (rel) Branch if

Overflow Clear

? V = 0 REL 28 rr 3 — — — — — — — —

BVS (rel) Branch if

Overflow Set

? V = 1 REL 29 rr 3 — — — — — — — —

CBA Compare A to

B

A – B INH 11 — 2 — — — — D D D D

CLC Clear Carry Bit 0 Þ C INH 0C — 2 — — — — — — — 0

CLI Clear Interrupt

Mask

0 Þ I INH 0E — 2 — — — 0 — — — —

CLR (opr) Clear Memory

Byte

0 Þ M EXT

IND,X

IND,Y

7F

6F

18 6F

hh ll

ff

ff

667

— — — — 0 1 0 0

CLRA Clear

Accumulator A

0 Þ A A INH 4F — 2 — — — — 0 1 0 0

CLRB Clear

Accumulator B

0 Þ B B INH 5F — 2 — — — — 0 1 0 0

CLV Clear Overflow

Flag

0 Þ V INH 0A — 2 — — — — — — 0 —

CMPA (opr) Compare A to

Memory

A – M A IMM

A DIR

A EXT

A IND,X

A IND,Y

81

91

B1

A1

18 A1

ii

dd

hh ll

ff

ff

23445

— — — — D D D D

CMPB (opr) Compare B to

Memory

B – M B IMM

B DIR

B EXT

B IND,X

B IND,Y

C1

D1

F1

E1

18 E1

ii

dd

hh ll

ff

ff

23445

— — — — D D D D

Table 3-2 Instruction Set (Sheet 2 of 6)

Mnemonic Operation Description Addressing Instruction Condition Codes

Mode Opcode Operand Cycles S X H I N Z V C

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

CENTRAL PROCESSING UNIT

TECHNICAL DATA 3-11

COM (opr) Ones

Complement

Memory Byte

$FF – M Þ M EXT

IND,X

IND,Y

73

63

18 63

hh ll

ff

ff

667

— — — — D D 0 1

COMA Ones

Complement

A

$FF – A Þ A A INH 43 — 2 — — — — D D 0 1

COMB Ones

Complement

B

$FF – B Þ B B INH 53 — 2 — — — — D D 0 1

CPD (opr) Compare D to

Memory 16-Bit

D – M : M + 1 IMM

DIR

EXT

IND,X

IND,Y

1A 83

1A 93

1A B3

1A A3

CD A3

jj kk

dd

hh ll

ff

ff

56777

— — — — D D D D

CPX (opr) Compare X to

Memory 16-Bit

IX – M : M + 1 IMM

DIR

EXT

IND,X

IND,Y

8C

9C

BC

AC

CD AC

jj kk

dd

hh ll

ff

ff

45667

— — — — D D D D

CPY (opr) Compare Y to

Memory 16-Bit

IY – M : M + 1 IMM

DIR

EXT

IND,X

IND,Y

18 8C

18 9C

18 BC

1A AC

18 AC

jj kk

dd

hh ll

ff

ff

56777

— — — — D D D D

DAA Decimal Adjust

A

Adjust Sum to BCD INH 19 — 2 — — — — D D D D

DEC (opr) Decrement

Memory Byte

M – 1 Þ M EXT

IND,X

IND,Y

7A

6A

18 6A

hh ll

ff

ff

667

— — — — D D D —

DECA Decrement

Accumulator

A

A – 1 Þ A A INH 4A — 2 — — — — D D D —

DECB Decrement

Accumulator

B

B – 1 Þ B B INH 5A — 2 — — — — D D D —

DES Decrement

Stack Pointer

SP – 1 Þ SP INH 34 — 3 — — — — — — — —

DEX Decrement

Index Register

X

IX – 1 Þ IX INH 09 — 3 — — — — — D — —

DEY Decrement

Index Register

Y

IY – 1 Þ IY INH 18 09 — 4 — — — — — D — —

EORA (opr) Exclusive OR

A with Memory

A Å M Þ A A IMM

A DIR

A EXT

A IND,X

A IND,Y

88

98

B8

A8

18 A8

ii

dd

hh ll

ff

ff

23445

— — — — D D 0 —

EORB (opr) Exclusive OR

B with Memory

B Å M Þ B B IMM

B DIR

B EXT

B IND,X

B IND,Y

C8

D8

F8

E8

18 E8

ii

dd

hh ll

ff

ff

23445

— — — — D D 0 —

FDIV Fractional

Divide 16 by

16

D / IX Þ IX; r Þ D INH 03 — 41 — — — — — D D D

IDIV Integer Divide

16 by 16

D / IX Þ IX; r Þ D INH 02 — 41 — — — — — D 0 D

INC (opr) Increment

Memory Byte

M + 1 Þ M EXT

IND,X

IND,Y

7C

6C

18 6C

hh ll

ff

ff

667

— — — — D D D —

INCA Increment

Accumulator

A

A + 1 Þ A A INH 4C — 2 — — — — D D D —

INCB Increment

Accumulator

B

B + 1 Þ B B INH 5C — 2 — — — — D D D —

INS Increment

Stack Pointer

SP + 1 Þ SP INH 31 — 3 — — — — — — — —

INX Increment

Index Register

X

IX + 1 Þ IX INH 08 — 3 — — — — — D — —

Table 3-2 Instruction Set (Sheet 3 of 6)

Mnemonic Operation Description Addressing Instruction Condition Codes

Mode Opcode Operand Cycles S X H I N Z V C

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

CENTRAL PROCESSING UNIT MC68HC11F1

3-12 TECHNICAL DATA

INY Increment

Index Register

Y

IY + 1 Þ IY INH 18 08 — 4 — — — — — D — —

JMP (opr) Jump See Figure 3–2 EXT

IND,X

IND,Y

7E

6E

18 6E

hh ll

ff

ff

334

— — — — — — — —

JSR (opr) Jump to

Subroutine

See Figure 3–2 DIR

EXT

IND,X

IND,Y

9D

BD

AD

18 AD

dd

hh ll

ff

ff

5667

— — — — — — — —

LDAA (opr) Load

Accumulator

A

M Þ A A IMM

A DIR

A EXT

A IND,X

A IND,Y

86

96

B6

A6

18 A6

ii

dd

hh ll

ff

ff

23445

— — — — D D 0 —

LDAB (opr) Load

Accumulator

B

M Þ B B IMM

B DIR

B EXT

B IND,X

B IND,Y

C6

D6

F6

E6

18 E6

ii

dd

hh ll

ff

ff

23445

— — — — D D 0 —

LDD (opr) Load Double

Accumulator

D

M Þ A,M + 1 Þ B IMM

DIR

EXT

IND,X

IND,Y

CC

DC

FC

EC

18 EC

jj kk

dd

hh ll

ff

ff

34556

— — — — D D 0 —

LDS (opr) Load Stack

Pointer

M : M + 1 Þ SP IMM

DIR

EXT

IND,X

IND,Y

8E

9E

BE

AE

18 AE

jj kk

dd

hh ll

ff

ff

34556

— — — — D D 0 —

LDX (opr) Load Index

Register

X

M : M + 1 Þ IX IMM

DIR

EXT

IND,X

IND,Y

CE

DE

FE

EE

CD EE

jj kk

dd

hh ll

ff

ff

34556

— — — — D D 0 —

LDY (opr) Load Index

Register

Y

M : M + 1 Þ IY IMM

DIR

EXT

IND,X

IND,Y

18 CE

18 DE

18 FE

1A EE

18 EE

jj kk

dd

hh ll

ff

ff

45666

— — — — D D 0 —

LSL (opr) Logical Shift

Left

EXT

IND,X

IND,Y

78

68

18 68

hh ll

ff

ff

667

— — — — D D D D

LSLA Logical Shift

Left A

A INH 48 — 2 — — — — D D D D

LSLB Logical Shift

Left B

B INH 58 — 2 — — — — D D D D

LSLD Logical Shift

Left Double

INH 05 — 3 — — — — D D D D

LSR (opr) Logical Shift

Right

EXT

IND,X

IND,Y

74

64

18 64

hh ll

ff

ff

667

— — — — 0 D D D

LSRA Logical Shift

Right A

A INH 44 — 2 — — — — 0 D D D

LSRB Logical Shift

Right B

B INH 54 — 2 — — — — 0 D D D

LSRD Logical Shift

Right Double

INH 04 — 3 — — — — 0 D D D

MUL Multiply 8 by 8 A * B Þ D INH 3D — 10 — — — — — — — D

NEG (opr) Two’s

Complement

Memory Byte

0 – M Þ M EXT

IND,X

IND,Y

70

60

18 60

hh ll

ff

ff

667

— — — — D D D D

NEGA Two’s

Complement

A

0 – A Þ A A INH 40 — 2 — — — — D D D D

NEGB Two’s

Complement

B

0 – B Þ B B INH 50 — 2 — — — — D D D D

Table 3-2 Instruction Set (Sheet 4 of 6)

Mnemonic Operation Description Addressing Instruction Condition Codes

Mode Opcode Operand Cycles S X H I N Z V C

C

0

b7 b0

C

0

b7 b0

C

0

b7 b0

C

0

b7 A b0 b7 B b0

C

0

b7 b0

C

0

b7 b0

C

0

b7 b0

C

0

b7 A b0 b7 B b0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

CENTRAL PROCESSING UNIT

TECHNICAL DATA 3-13

NOP No operation No Operation INH 01 — 2 — — — — — — — —

ORAA (opr) OR

Accumulator

A (Inclusive)

A + M Þ A A IMM

A DIR

A EXT

A IND,X

A IND,Y

8A

9A

BA

AA

18 AA

ii

dd

hh ll

ff

ff

23445

— — — — D D 0 —

ORAB (opr) OR

Accumulator

B (Inclusive)

B + M Þ B B IMM

B DIR

B EXT

B IND,X

B IND,Y

CA

DA

FA

EA

18 EA

ii

dd

hh ll

ff

ff

23445

— — — — D D 0 —

PSHA Push A onto

Stack

A Þ Stk,SP = SP – 1 A INH 36 — 3 — — — — — — — —

PSHB Push B onto

Stack

B Þ Stk,SP = SP – 1 B INH 37 — 3 — — — — — — — —

PSHX Push X onto

Stack (Lo

First)

IX Þ Stk,SP = SP – 2 INH 3C — 4 — — — — — — — —

PSHY Push Y onto

Stack (Lo

First)

IY Þ Stk,SP = SP – 2 INH 18 3C — 5 — — — — — — — —

PULA Pull A from

Stack

SP = SP + 1, A Ü Stk A INH 32 — 4 — — — — — — — —

PULB Pull B from

Stack

SP = SP + 1, B Ü Stk B INH 33 — 4 — — — — — — — —

PULX Pull X From

Stack (Hi

First)

SP = SP + 2, IX Ü

Stk

INH 38 — 5 — — — — — — — —

PULY Pull Y from

Stack (Hi

First)

SP = SP + 2, IY Ü

Stk

INH 18 38 — 6 — — — — — — — —

ROL (opr) Rotate Left EXT

IND,X

IND,Y

79

69

18 69

hh ll

ff

ff

667

— — — — D D D D

ROLA Rotate Left A A INH 49 — 2 — — — — D D D D

ROLB Rotate Left B B INH 59 — 2 — — — — D D D D

ROR (opr) Rotate Right EXT

IND,X

IND,Y

76

66

18 66

hh ll

ff

ff

667

— — — — D D D D

RORA Rotate Right A A INH 46 — 2 — — — — D D D D

RORB Rotate Right B B INH 56 — 2 — — — — D D D D

RTI Return from

Interrupt

See Figure 3–2 INH 3B — 12 D ¯ D D D D D D

RTS Return from

Subroutine

See Figure 3–2 INH 39 — 5 — — — — — — — —

SBA Subtract B

from A

A – B Þ A INH 10 — 2 — — — — D D D D

SBCA (opr) Subtract with

Carry from A

A – M – C Þ A A IMM

A DIR

A EXT

A IND,X

A IND,Y

82

92

B2

A2

18 A2

ii

dd

hh ll

ff

ff

23445

— — — — D D D D

SBCB (opr) Subtract with

Carry from B

B – M – C Þ B B IMM

B DIR

B EXT

B IND,X

B IND,Y

C2

D2

F2

E2

18 E2

ii

dd

hh ll

ff

ff

23445

— — — — D D D D

SEC Set Carry 1 Þ C INH 0D — 2 — — — — — — — 1

SEI Set Interrupt

Mask

1 Þ I INH 0F — 2 — — — 1 — — — —

SEV Set Overflow

Flag

1 Þ V INH 0B — 2 — — — — — — 1 —

STAA (opr) Store

Accumulator

A

A Þ M A DIR

A EXT

A IND,X

A IND,Y

97

B7

A7

18 A7

dd

hh ll

ff

ff

3445

— — — — D D 0 —

Table 3-2 Instruction Set (Sheet 5 of 6)

Mnemonic Operation Description Addressing Instruction Condition Codes

Mode Opcode Operand Cycles S X H I N Z V C

C b7 b0

C b7 b0

C b7 b0

b7 b0 C

b7 b0 C

b7 b0 C

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

CENTRAL PROCESSING UNIT MC68HC11F1

3-14 TECHNICAL DATA

STAB (opr) Store

Accumulator

B

B Þ M B DIR

B EXT

B IND,X

B IND,Y

D7

F7

E7

18 E7

dd

hh ll

ff

ff

3445

— — — — D D 0 —

STD (opr) Store

Accumulator

D

A Þ M, B Þ M + 1 DIR

EXT

IND,X

IND,Y

DD

FD

ED

18 ED

dd

hh ll

ff

ff

4556

— — — — D D 0 —

STOP Stop Internal

Clocks

— INH CF — 2 — — — — — — — —

STS (opr) Store Stack

Pointer

SP Þ M : M + 1 DIR

EXT

IND,X

IND,Y

9F

BF

AF

18 AF

dd

hh ll

ff

ff

4556

— — — — D D 0 —

STX (opr) Store Index

Register X

IX Þ M : M + 1 DIR

EXT

IND,X

IND,Y

DF

FF

EF

CD EF

dd

hh ll

ff

ff

4556

— — — — D D 0 —

STY (opr) Store Index

Register Y

IY Þ M : M + 1 DIR

EXT

IND,X

IND,Y

18 DF

18 FF

1A EF

18 EF

dd

hh ll

ff

ff

5666

— — — — D D 0 —

SUBA (opr) Subtract

Memory from

A

A – M Þ A A IMM

A DIR

A EXT

A IND,X

A IND,Y

80

90

B0

A0

18 A0

ii

dd

hh ll

ff

ff

23445

— — — — D D D D

SUBB (opr) Subtract

Memory from

B

B – M Þ B A IMM

A DIR

A EXT

A IND,X

A IND,Y

C0

D0

F0

E0

18 E0

ii

dd

hh ll

ff

ff

23445

— — — — D D D D

SUBD (opr) Subtract

Memory from

D

D – M : M + 1 Þ D IMM

DIR

EXT

IND,X

IND,Y

83

93

B3

A3

18 A3

jj kk

dd

hh ll

ff

ff

45667

— — — — D D D D

SWI Software

Interrupt

See Figure 3–2 INH 3F — 14 — — — 1 — — — —

TAB Transfer A to B A Þ B INH 16 — 2 — — — — D D 0 —

TAP Transfer A to

CC Register

A Þ CCR INH 06 — 2 D ¯ D D D D D D

TBA Transfer B to A B Þ A INH 17 — 2 — — — — D D 0 —

TEST TEST (Only in

Test Modes)

Address Bus Counts INH 00 — * — — — — — — — —

TPA Transfer CC

Register to A

CCR Þ A INH 07 — 2 — — — — — — — —

TST (opr) Test for Zero

or Minus

M – 0 EXT

IND,X

IND,Y

7D

6D

18 6D

hh ll

ff

ff

667

— — — — D D 0 0

TSTA Test A for Zero

or Minus

A – 0 A INH 4D — 2 — — — — D D 0 0

TSTB Test B for Zero

or Minus

B – 0 B INH 5D — 2 — — — — D D 0 0

TSX Transfer

Stack Pointer

to X

SP + 1 Þ IX INH 30 — 3 — — — — — — — —

TSY Transfer

Stack Pointer

to Y

SP + 1 Þ IY INH 18 30 — 4 — — — — — — — —

TXS Transfer X to

Stack Pointer

IX – 1 Þ SP INH 35 — 3 — — — — — — — —

TYS Transfer Y to

Stack Pointer

IY – 1 Þ SP INH 18 35 — 4 — — — — — — — —

WAI Wait for

Interrupt

Stack Regs & WAIT INH 3E — ** — — — — — — — —

XGDX Exchange D

with X

IX Þ D, D Þ IX INH 8F — 3 — — — — — — — —

XGDY Exchange D

with Y

IY Þ D, D Þ IY INH 18 8F — 4 — — — — — — — —

Table 3-2 Instruction Set (Sheet 6 of 6)

Mnemonic Operation Description Addressing Instruction Condition Codes

Mode Opcode Operand Cycles S X H I N Z V C

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

OPERATING MODES AND ON-CHIP MEMORY

TECHNICAL DATA 4-1

SECTION 4OPERATING MODES AND ON-CHIP MEMORY

This section contains information about the modes that define MC68HC11F1 operating

conditions, and about the on-chip memory that allows the MCU to be configured

for various applications.

4.1 Operating Modes

The values of the mode select inputs MODB and MODA during reset determine the

operating mode. Single chip and expanded modes are the normal modes. In singlechip

mode only on-board resources are available. Expanded mode, however, allows

access to external memory or peripheral devices. Each of these two normal modes is

paired with a special mode. Bootstrap mode, a variation of the single-chip mode, executes

a bootloader program from an internal bootstrap ROM. Test mode allows privileged

access to internal resources.

4.1.1 Single-Chip Operating Mode

In single-chip operating mode, the MC68HC11F1 has no external address or data bus.

Ports B, C, and F are available for general-purpose I/O.

4.1.2 Expanded Operating Mode

In expanded operating mode, the MCU can access a 64-Kbyte physical address

space. The address space includes the same on-chip memory addresses used for single-

chip mode, in addition to external memory and peripheral devices.

The expansion bus is made up of ports B, C, F and the R/W signal. In expanded mode,

high order address bits are output on the port B pins, low order address bits on the port

F pins, and the data bus on port C. The R/W pin indicates the direction of data transfer

on the port C bus.

4.1.3 Special Test Mode

Special test mode, a variation of the expanded mode, is primarily used during Motorola's

internal production testing; however, it is accessible for programming the CONFIG

register, programming calibration data into EEPROM, and supporting emulation

and debugging during development.

4.1.4 Special Bootstrap Mode

Bootstrap mode is a special variation of the single-chip mode. Bootstrap mode allows

special-purpose programs to be entered into internal RAM. When boot mode is selected

at reset, a small bootstrap ROM becomes present in the memory map. Reset and

interrupt vectors are located in bootstrap ROM at $BFC0–$BFFF. The MCU fetches

the reset vector, then executes the bootloader.

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

OPERATING MODES AND ON-CHIP MEMORY MC68HC11F1

4-2 TECHNICAL DATA

The bootstrap ROM contains a small program which initializes the SCI and allows the

user to download a program of up to 1024 bytes into on-chip RAM. After a four-character

delay, or after receiving the character for address $03FF, control passes to the

loaded program at $0000. An external pull-up resistor is required when using the SCI

transmitter pin (TxD) because port D pins are configured for wired-OR operation by

the bootloader. In bootstrap mode, the interrupt vectors point to RAM. This allows the

use of interrupts through a jump table. Refer to Freescale application note AN1060,

M68HC11 Bootstrap Mode.

4.2 On-Chip Memory

The MC68HC11F1 contains 1024 bytes of on-chip RAM and 512 bytes of EEPROM.

The bootloader ROM occupies 256 bytes. The CONFIG register is implemented as a

separate EEPROM byte.

4.2.1 Mapping Allocations

Memory locations for on-chip resources are the same for both expanded and singlechip

modes. The 96-byte register block originates at $1000 after reset and can be

placed at any other 4-Kbyte boundary ($x000) after reset by writing an appropriate value

to the INIT register. Refer to Figure 4-1, which illustrates the memory map.

The on-board 1024-byte RAM is initially located at $0000 after reset. If RAM and registers

are both mapped to the same 4-Kbyte boundary, the first 96 bytes of RAM are

inaccessible (registers have higher priority). Remapping is accomplished by writing

appropriate values to the INIT register.

The 512-byte EEPROM array is initially located at $FE00 after reset when EEPROM

is enabled in the memory map by the CONFIG register. In expanded and special test

modes EEPROM can be placed at any other 4-Kbyte boundary ($xE00) by programming

bits EE[3:0] in the CONFIG register to an appropriate value. In single-chip and

bootstrap modes the EEPROM is forced on and cannot be remapped.

In special bootstrap mode, a bootloader ROM is enabled at locations $BF00–$BFFF.

The vectors for special bootstrap mode are contained in the bootloader program. The

boot ROM fills 256 bytes of the memory map even though not all locations are used.

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

OPERATING MODES AND ON-CHIP MEMORY

TECHNICAL DATA 4-3

4.2.2 Memory Map

Figure 4-1 MC68HC11F1 Memory Map

4.2.2.1 RAM

The MC68HC11F1 microcontroller has 1024 bytes of fully static RAM that can be used

for storing instructions, variables, and temporary data during program execution. RAM

can be placed at any 4-Kbyte boundary in the 64 Kbyte address space by writing an

appropriate value to the INIT register.

RAM is initially located at $0000 in the memory map upon reset. Direct addressing

mode can access the first 256 locations of RAM using a one-byte address operand.

Direct mode accesses save program memory space and execution time.

The on-chip RAM is a fully static memory. RAM contents can be preserved during periods

of processor inactivity by either of two methods, both of which reduce power consumption.

NOTES:

1. RAM can be remapped to any 4-Kbyte boundary ($x000). "x" represents the value contained in

RAM[3:0] in the init register.

2. The register block can be remapped to any 4-Kbyte boundary ($y000). "y" represents the value contained in

reg[3:0] in the init register.

3. Special test mode vectors are externally addressed.

4. In special test mode the address locations $zD00–$zDFF are not externally addressable.

"z" represents the value of bits EE[3:0] in the config register.

5. EEPROM can be remapped to any 4-Kbyte boundary ($z000). "z" represents the value contained in

EE[3:0] in the config register.

zE00

EXPANDED

512 BYTES EEPROM 5

FFC0

FFFF

NORMAL MODE

INTERRUPT

VECTORS

1024 BYTES RAM 1

SINGLE

CHIP

BOOTSTRAP SPECIAL

TEST

EXT

$0000

$1000

$FFFF

x000

y000

y05F

BF00

zFFF

BFC0

BFFF

SPECIAL MODE 3

INTERRUPT

VECTORS

EXT

x3FF

96-BYTE REGISTER BLOCK 2

$FE00

256 BYTES BOOTSTRAP ROM

zD00 256 BYTES RESERVED 4

zDFF (SPECIAL TEST MODE ONLY)

EXT

EXT EXT

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

OPERATING MODES AND ON-CHIP MEMORY MC68HC11F1

4-4 TECHNICAL DATA

During the software-based STOP mode, MCU clocks are stopped, but the MCU continues

to draw power from VDD. Power supply current is directly proportional to operating

frequency in CMOS integrated circuits and there is very little leakage when the

clocks are stopped. These two factors reduce power consumption while the MCU is in

STOP mode.

To reduce power consumption to a minimum, VDD can be turned off, and the MODB/

VSTBY pin can be used to supply RAM power from either a battery back-up or a second

power supply. Although this method requires external hardware, it is very effective.

Refer to SECTION 2 PIN DESCRIPTIONS for information about how to connect

the standby RAM power supply. Refer to SECTION 5 RESETS AND INTERRUPTS

for a description of low power operation.

Figure 4-2 RAM Standby MODB/VSTBY Connections

4.2.2.2 Bootloader ROM

The bootloader ROM is enabled at address $BF00–$BFFF during special bootstrap

mode. The reset vector is fetched from this ROM and the MCU executes the bootloader

firmware. In normal modes, the bootloader ROM is disabled.

4.2.2.3 EEPROM

The MC68HC11F1 contains 512 bytes of electrically erasable programmable readonly

memory (EEPROM). The default location for EEPROM is $FE00–$FFFF. Other

locations can be chosen according to the values written to EE[3:0] in the CONFIG register.

In single-chip and bootstrap modes, the EEPROM is forced on and located at the

default position. In these modes, the EEPROM cannot be remapped. In special test

mode, the EEPROM is disabled initially.

4.2.3 Registers

Table 4-1, a summary of registers and control bits, the registers are shown in ascending

order within the 96-byte register block. The addresses shown are for default block

mapping ($1000–$105F), however, the register block can be remapped to any 4-Kbyte

page ($x000–$x05F) by the INIT register.

VBATT

VDD

TO MODB/VSTBY

OF M68HC11

VOUT

VDD

+

MAX

690

4.8 V

NiCd

4.7 k

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

OPERATING MODES AND ON-CHIP MEMORY

TECHNICAL DATA 4-5

Table 4-1 Register and Control Bit Assignments

The register block can be remapped to any 4-Kbyte boundary.

Bit 7 6 5 4 3 2 1 Bit 0

$1000 PA7 PA6 PA5 PA4 PA3 PA2 PA1 PA0 PORTA

$1001 DDA7 DDA6 DDA5 DDA4 DDA3 DDA2 DDA1 DDA0 DDRA

$1002 PG7 PG6 PG5 PG4 PG3 PG2 PG1 PG0 PORTG

$1003 DDG7 DDG6 DDG5 DDG4 DDG3 DDG2 DDG1 DDG0 DDRG

$1004 PB7 PB6 PB5 PB4 PB3 PB2 PB1 PB0 PORTB

$1005 PF7 PF6 PF5 PF4 PF3 PF2 PF1 PF0 PORTF

$1006 PC7 PC6 PC5 PC4 PC3 PC2 PC1 PC0 PORTC

$1007 DDC7 DDC6 DDC5 DDC4 DDC3 DDC2 DDC1 DDC0 DDRC

$1008 0 0 PD5 PD4 PD3 PD2 PD1 PD0 PORTD

$1009 0 0 DDD5 DDD4 DDD3 DDD2 DDD1 DDD0 DDRD

$100A PE7 PE6 PE5 PE4 PE3 PE2 PE1 PE0 PORTE

$100B FOC1 FOC2 FOC3 FOC4 FOC5 0 0 0 CFORC

$100C OC1M7 OC1M6 OC1M5 OC1M4 OC1M3 0 0 0 OC1M

$100D OC1D7 OC1D6 OC1D5 OC1D4 OC1D3 0 0 0 OC1D

$100E Bit 15 14 13 12 11 10 9 Bit 8 TCNT (High)

$100F Bit 7 6 5 4 3 2 1 Bit 0 TCNT (Low)

$1010 Bit 15 14 13 12 11 10 9 Bit 8 TIC1 (High)

$1011 Bit 7 6 5 4 3 2 1 Bit 0 TIC1 (Low)

$1012 Bit 15 14 13 12 11 10 9 Bit 8 TIC2 (High)

$1013 Bit 7 6 5 4 3 2 1 Bit 0 TIC2 (Low)

$1014 Bit 15 14 13 12 11 10 9 Bit 8 TIC3 (High)

$1015 Bit 7 6 5 4 3 2 1 Bit 0 TIC3 (Low)

$1016 Bit 15 14 13 12 11 10 9 Bit 8 TOC1 (High)

$1017 Bit 7 6 5 4 3 2 1 Bit 0 TOC1 (Low)

$1018 Bit 15 14 13 12 11 10 9 Bit 8 TOC2 (High)

$1019 Bit 7 6 5 4 3 2 1 Bit 0 TOC2 (Low)

$101A Bit 15 14 13 12 11 10 9 Bit 8 TOC3 (High)

$101B Bit 7 6 5 4 3 2 1 Bit 0 TOC3 (Low)

$101C Bit 15 14 13 12 11 10 9 Bit 8 TOC4 (High)

$101D Bit 7 6 5 4 3 2 1 Bit 0 TOC4 (Low)

$101E Bit 15 14 13 12 11 10 9 Bit 8 TI4/O5 (High)

$101F Bit 7 6 5 4 3 2 1 Bit 0 TI4/O5 (Low)

$1020 OM2 OL2 OM3 OL3 OM4 OL4 OM5 OL5 TCTL1

$1021 EDG4B EDG4A EDG1B EDG1A EDG2B EDG2A EDG3B EDG3A TCTL2

$1022 OC1I OC2I OC3I OC4I I4/O5I IC1I IC2I IC3I TMSK1

$1023 OC1F OC2F OC3F OC4F I4/O5F IC1F IC2F IC3F TFLG1

$1024 TOI RTII PAOVI PAII 0 0 PR1 PR0 TMSK2

$1025 TOF RTIF PAOVF PAIF 0 0 0 0 TFLG2

$1026 0 PAEN PAMOD PEDGE 0 I4/O5 RTR1 RTR0 PACTL

$1027 Bit 7 6 5 4 3 2 1 Bit 0 PACNT

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

OPERATING MODES AND ON-CHIP MEMORY MC68HC11F1

4-6 TECHNICAL DATA

4.3 System Initialization

Registers and bits that control initialization and the basic operation of the MCU are protected

against writes except under special circumstances. The following table lists registers

that can be written only once after reset or that must be written within the first 64

cycles after reset.

$1028 SPIE SPE DWOM MSTR CPOL CPHA SPR1 SPR0 SPCR

$1029 SPIF WCOL 0 MODF 0 0 0 Bit 0 SPSR

$102A Bit 7 6 5 4 3 2 1 Bit 0 SPDR

$102B TCLR 0 SCP1 SCP0 RCKB SCR2 SCR1 SCR0 BAUD

$102C R8 T8 0 M WAKE 0 0 0 SCCR1

$102D TIE TCIE RIE ILIE TE RE RWU SBK SCCR2

$102E TDRE TC RDRF IDLE OR NF FE 0 SCSR

$102F Bit 7 6 5 4 3 2 1 Bit 0 SCDR

$1030 CCF 0 SCAN MULT CD CC CB CA ADCTL

$1031 Bit 7 6 5 4 3 2 1 Bit 0 ADR1

$1032 Bit 7 6 5 4 3 2 1 Bit 0 ADR2

$1033 Bit 7 6 5 4 3 2 1 Bit 0 ADR3

$1034 Bit 7 6 5 4 3 2 1 Bit 0 ADR4

$1035 0 0 0 PTCON BPRT3 BPRT2 BPRT1 BPRT0 BPROT

$1036 Reserved

$1037 Reserved

$1038 GWOM CWOM CLK4X 0 0 0 0 0 OPT2

$1039 ADPU CSEL IRQE DLY CME FCME CR1 CR0 OPTION

$103A Bit 7 6 5 4 3 2 1 Bit 0 COPRST

$103B ODD EVEN 0 BYTE ROW ERASE EELAT EEPGM PPROG

$103C RBOOT SMOD MDA IRV PSEL3 PSEL2 PSEL1 PSEL0 HPRIO

$103D RAM3 RAM2 RAM1 RAM0 REG3 REG2 REG1 REG0 INIT

$103E TILOP 0 OCCR CBYP DISR FCM FCOP 0 TEST1

$103F EE3` EE2 EE1 EE0 1 NOCOP 1 EEON CONFIG

$1040 Reserved

to

$105B Reserved

$105C IO1SA IO1SB IO2SA IO2SB GSTHA GSTHB PSTHA PSTHB CSSTRH

$105D IO1EN IO1PL IO2EN IO2PL GCSPR PCSEN PSIZA PSIZB CSCTL

$105E GA15 GA14 GA13 GA12 GA11 GA10 0 0 CSGADR

$105F IO1AV IO2AV 0 GNPOL GAVLD GSIZA GSIZB GSIZC CSGSIZ

Table 4-1 Register and Control Bit Assignments (Continued)

The register block can be remapped to any 4-Kbyte boundary.

Bit 7 6 5 4 3 2 1 Bit 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

OPERATING MODES AND ON-CHIP MEMORY

TECHNICAL DATA 4-7

Notes:

1. Bits 1 and 0 can be written once only in first 64 cycles. When SMOD = 1, these bits can be written any time. All

other bits can be written at any time.

2. Bits can be written to zero (protection disabled) once only in first 64 cycles or at any time in special modes. Bits

can be set to one at any time.

3. Bits 5, 4, 2, 1, and 0 can be written once only in first 64 cycles. When SMOD = 1, bits 5, 4, 2, 1, and 0 can be

written at any time. All other bits can be written at any time

4. Bit 5 (CLK4X) can be written only one time.

5. Bit 4 (IRV) can be written only one time.

6. Can be written once in first 64 cycles after reset in normal modes or at any time in special modes.

4.3.1 Mode Selection

The four mode variations are selected by the logic levels present on the MODA and

MODB pins at the rising edge of RESET. The MODA and MODB logic levels determine

the logic state of SMOD and MDA control bits in the HPRIO register.

After reset is released, the mode select pins no longer influence the MCU operating

mode. In single-chip operating mode, the MODA pin is connected to a logic level zero.

In expanded mode, MODA should be connected to VDD through a pull-up resistor of

4.7 kW. The MODA pin also functions as the load instruction register (LIR) pin when

the MCU is not in reset. The open-drain active low LIR output pin drives low during the

first E cycle of each instruction (opcode fetch). The MODB pin also functions as standby

power input (VSTBY), which allows RAM contents to be maintained in absence of

VDD. Refer to APPENDIX A ELECTRICAL CHARACTERISTICS for VSTBY voltage

requirements.

Refer to Table 4-3, which is a summary of mode pin operation, the mode control bits,

and the four operating modes.

Table 4-2 Write Access Limited Registers

Register

Address

Register

Name

Must be Written in

First 64 Cycles

Write One Time

Only

$x024 Timer Interrupt Mask 2 (TMSK2) Note 1 —

$x035 Block Protect Register (BPROT) Note 2 —

$x038 System Configuration Options 2 (OPT2) No Note 4

$x039 System Configuration Options (OPTION) Note 3 —

$x03C Highest Priority I-bit and Miscellaneous (HPRIO) No Note 5

$x03D RAM and I/O Map Register (INIT) Yes Note 6

Table 4-3 Hardware Mode Select Summary

Input Levels

at Reset

Mode Control Bits in HPRIO

(Latched at Reset)

MODB MODA RBOOT SMOD MDA

1 0 Single Chip 0 0 0

1 1 Expanded 0 0 1

0 0 Special Bootstrap 1 1 0

0 1 Special Test 0 1 1

A normal mode is selected when MODB is logic one during reset. One of three reset

vectors is fetched from address $FFFA–$FFFF, and program execution begins from

the address indicated by this vector. If MODB is logic zero during reset, the special

mode reset vector is fetched from addresses $BFFA–$BFFF and software has access

to special test features. Refer to SECTION 5 RESETS AND INTERRUPTS for information

regarding reset vectors.

4.3.1.1 HPRIO Register

Bits in the HPRIO register select the highest priority interrupt level, select whether

bootstrap ROM is present, and control visibility of internal reads by the CPU. After reset,

MDA and SMOD select the operating mode.

*Reset states of RBOOT, SMOD, and MDA bits depend on hardware mode selection. Refer to Table 4-3.

RBOOT — Read Bootstrap ROM

Set to one out of reset in bootstrap mode. Valid while in special modes only. Can be

read anytime. Can only be written in special modes.

0 = Bootloader ROM disabled and not in map

1 = Bootloader ROM enabled and in map at $BF00–$BFFF

SMOD and MDA — Special Mode Select and Mode Select A

The initial value of SMOD is the inverse of the logic level present on the MODB pin at

the rising edge of reset. The initial value of MDA equals the logic level present on the

MODA pin at the rising edge of reset. These two bits can be read at any time. They can

be written at any time in special modes. Neither bit can be written is normal modes.

SMOD cannot be set once it has been cleared. Refer to Table 4-3.

IRV — Internal Read Visibility

IRV can be written at any time in special modes (SMOD = 1). In normal modes (SMOD

= 0) IRV can be written only once. In expanded and test modes, IRV determines

whether internal read visibility is on or off. In single-chip and bootstrap modes, IRV has

no meaning or effect.

0 = No internal read visibility on external bus

1 = Data from internal reads is driven out the external data bus.

PSEL[3:0] — Priority Select Bits [3:0]

Refer to 5.3.1 Highest Priority Interrupt and Miscellaneous Register.

HPRIO — Highest Priority I-Bit Interrupt and Miscellaneous $103C

Bit 7 6 5 4 3 2 1 Bit 0

RBOOT* SMOD* MDA* IRV PSEL3 PSEL2 PSEL1 PSEL0

RESET: 0 0 0 0 0 1 1 0 Single Chip

0 0 1 0 0 1 1 0 Expanded

1 1 0 1 0 1 1 0 Bootstrap

0 1 1 1 0 1 1 0 Special Test

4.3.2 Initialization

Because bits in the following registers control the basic configuration of the MCU, an

accidental change of their values could cause serious system problems. The protection

mechanism, overridden in special operating modes, requires a write to the protected

bits only within the first 64 bus cycles after any reset, or only once after each reset.

Table 4-2 summarizes the write access limited registers.

4.3.2.1 CONFIG Register

CONFIG controls the presence and position of the EEPROM in the memory map.

CONFIG also enables the COP watchdog timer.

P indicates a previously programmed bit. P(L) indicates that the bit resets to the logic

level held in the latch prior to reset, but the function of COP is controlled by DISR bit

in TEST1 register.

The CONFIG register consists of an EEPROM byte and static latches that control the

start-up configuration of the MCU. The contents of the EEPROM byte are transferred

into static working latches during reset sequences. The operation of the MCU is controlled

directly by these latches and not by CONFIG itself. In normal modes, changes

to CONFIG do not affect operation of the MCU until after the next reset sequence.

When programming, the CONFIG register itself is accessed. When the CONFIG register

is read, the static latches are accessed.

These bits can be read at any time. The value read is the one latched into the register

from the EEPROM cells during the last reset sequence. A new value programmed into

this register cannot be read until after a subsequent reset sequence. Unused bits always

read as ones.

In special test mode, the static latches can be written directly at any time. In all modes,

CONFIG bits can only be programmed using the EEPROM programming sequence,

and are neither readable nor active until latched via the next reset. Refer to 4.4.3 CONFIG

Register Programming.

EE[3:0] — EEPROM Mapping Control

EE[3:0] select the upper four bits of the EEPROM base address. In single-chip and

bootstrap modes, EEPROM is forced to $FE00–$FFFF regardless of the value of

EE[3:0].

CONFIG — System Configuration Register $103F

Bit 7 6 5 4 3 2 1 Bit 0

EE3 EE2 EE1 EE0 — NOCOP — EEON

RESET: 1 1 1 1 1 P 1 1 Single Chip

1 1 1 1 1 P(L) 1 1 Bootstrap

P P P P 1 P 1 P Expanded

P P P P 1 P(L) 1 0 Special Test

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

OPERATING MODES AND ON-CHIP MEMORY MC68HC11F1

4-10 TECHNICAL DATA

Bit 3 — Not implemented

Always reads one

NOCOP — COP System Disable

0 = COP system enabled (forces reset on time-out)

1 = COP system disabled

Bit 1 — Not implemented

Always reads one

EEON — EEPROM Enable

In single-chip modes EEON is forced to one (EEPROM enabled).

0 = 512 bytes of EEPROM is disabled from the memory map

1 = 512 bytes of EEPROM is present in the memory map

4.3.2.2 INIT Register

The internal registers used to control the operation of the MCU can be relocated on 4-

Kbyte boundaries within the memory space with the use of INIT. This 8-bit special-purpose

register can change the default locations of the RAM and control registers within

the MCU memory map. It can be written only once within the first 64 E-clock cycles

after a reset. It then becomes a read-only register.

Table 4-4 EEPROM Mapping

EE[3:0] EEPROM Position

0 0 0 0 $0E00 – $0FFF

0 0 0 1 $1E00 – $1FFF

0 0 1 0 $2E00 – $2FFF

0 0 1 1 $3E00 – $3FFF

0 1 0 0 $4E00 – $4FFF

0 1 0 1 $5E00 – $5FFF

0 1 1 0 $6E00 – $6FFF

0 1 1 1 $7E00 – $7FFF

1 0 0 0 $8E00 – $8FFF

1 0 0 1 $9E00 – $9FFF

1 0 1 0 $AE00 – $AFFF

1 0 1 1 $BE00 – $BFFF

1 1 0 0 $CE00 – $CFFF

1 1 0 1 $DE00 – $DFFF

1 1 1 0 $EE00 – $EFFF

1 1 1 1 $FE00 – $FFFF

INIT — RAM and I/O Mapping Register $103D

Bit 7 6 5 4 3 2 1 Bit 0

RAM3 RAM2 RAM1 RAM0 REG3 REG2 REG1 REG0

RESET: 0 0 0 0 0 0 0 1

RAM[3:0] — RAM Map Position

These four bits, which specify the upper hexadecimal digit of the RAM address, control

position of RAM in the memory map. RAM can be positioned at the beginning of any

4-Kbyte page in the memory map. Refer to Table 4-5.

REG[3:0] — 128-Byte Register Block Position

These four bits specify the upper hexadecimal digit of the address for the 128-byte

block of internal registers. The register block is positioned at the beginning of any 4-

Kbyte page in the memory map. Refer to Table 4-5.

When the memory map has the 96-byte register block mapped at the same location

as RAM, the registers have priority and the lower 96 bytes of RAM are inaccessible.

No harmful conflicts occur due to a hardware resource priority scheme. On-chip registers

have the highest priority of all on-chip resources, followed by on-chip RAM, bootstrap

ROM, and on-chip EEPROM.

4.3.2.3 OPTION Register

The 8-bit special-purpose OPTION register sets internal system configuration options

during initialization. In single-chip and expanded modes (SMOD = 0), IRQE, DLY, FCME,

and CR[1:0] can be written only once and only in the first 64 cycles after a reset.

This minimizes the possibility of any accidental changes to the system configuration.

In special test and bootstrap modes (SMOD = 1), these bits can be written at any time.

*Can be written only once in first 64 cycles out of reset in normal modes or at any time in special modes.

Table 4-5 RAM and Register Mapping

RAM[3:0] Location REG[3:0] Location

0000 $0000–$03FF 0000 $0000–$005F

0001 $1000–$13FF 0001 $1000–$105F

0010 $2000–$23FF 0010 $2000–$205F

0011 $3000–$33FF 0011 $3000–$305F

0100 $4000–$43FF 0100 $4000–$405F

0101 $5000–$53FF 0101 $5000–$505F

0110 $6000–$63FF 0110 $6000–$605F

0111 $7000–$73FF 0111 $7000–$705F

1000 $8000–$83FF 1000 $8000–$805F

1001 $9000–$93FF 1001 $9000–$905F

1010 $A000–$A3FF 1010 $A000–$A05F

1011 $B000–$B3FF 1011 $B000–$B05F

1100 $C000–$C3FF 1100 $C000–$C05F

1101 $D000–$D3FF 1101 $D000–$D05F

1110 $E000–$E3FF 1110 $E000–$E05F

1111 $F000–$F3FF 1111 $F000–$F05F

OPTION — System Configuration Options $1039

Bit 7 6 5 4 3 2 1 Bit 0

ADPU CSEL IRQE* DLY* CME FCME* CR1* CR0*

RESET: 0 0 0 0 0 0 0 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

OPERATING MODES AND ON-CHIP MEMORY MC68HC11F1

4-12 TECHNICAL DATA

ADPU — A/D Power-Up

Refer to SECTION 10 ANALOG-TO-DIGITAL CONVERTER.

0 = A/D system disabled

1 = A/D system power enabled

CSEL — Clock Select

Selects alternate clock source for on-chip EEPROM and A/D charge pumps. On-chip

RC clock should be used when E clock falls below 1 MHz. Refer to SECTION 10 ANALOG-

TO-DIGITAL CONVERTER.

0 = A/D and EEPROM use system E clock

1 = A/D and EEPROM use internal RC clock

IRQE — Configure IRQ for Falling Edge-Sensitive Operation

Refer to SECTION 5 RESETS AND INTERRUPTS.

0 = Low level-sensitive operation.

1 = Falling edge-sensitive only operation.

DLY — Enable Oscillator Start-up Delay

Refer to SECTION 5 RESETS AND INTERRUPTS.

0 = The oscillator start-up delay coming out of STOP is bypassed and the MCU resumes

processing within about four bus cycles.

1 = A delay of approximately 4000 E-clock cycles is imposed as the MCU is started

up from the STOP power-saving mode.

CME — Clock Monitor Enable

In order to use both STOP and clock monitor, the CME bit must be written to zero before

executing STOP, then written to one after recovering from STOP. Refer to SECTION

5 RESETS AND INTERRUPTS.

0 = Clock monitor disabled

1 = Clock monitor enabled

FCME — Force Clock Monitor Enable

When FCME equals one, slow or stopped clocks will cause a clock failure reset. To

use STOP mode, FCME must always equal zero. Refer to SECTION 5 RESETS AND

INTERRUPTS.

0 = Clock monitor follows state of CME bit

1 = Clock monitor enabled and cannot be disabled until next reset

CR[1:0] — COP Timer Rate Select Bits

These control bits determine a scaling factor for the watchdog timer. Refer to SECTION

5 RESETS AND INTERRUPTS.

4.3.2.4 OPT2 Register

The system configuration options 2 register (OPT2) controls three additional system

options.

GWOM — Port G Wired-OR Mode

Refer to SECTION 6 PARALLEL INPUT/OUTPUT.

0 = Port G operates normally.

1 = Port G outputs are open-drain type.

CWOM — Port C Wired-OR Mode

Refer to SECTION 6 PARALLEL INPUT/OUTPUT.

0 = Port C operates normally.

1 = Port C outputs are open-drain type.

CLK4X — 4XOUT Clock Enable

The 4XOUT signal, when enabled, is a buffered XTAL signal and is four times the frequency

of the E-clock. This buffered clock is intended to synchronize external devices

with the MCU. Refer to SECTION 2 PIN DESCRIPTIONS.

0 = The 4XOUT pin is driven low.

1 = The 4XOUT signal is driven on the 4XOUT pin.

Bits [4:0] — Not implemented

Always read zero

4.3.2.5 Block Protect Register (BPROT)

BPROT prevents accidental writes to EEPROM and the CONFIG register. The bits in

this register can be written to zero during the first 64 E-clock cycles after reset in the

normal modes. Once the bits are cleared to zero, the EEPROM array and the CONFIG

register can be programmed or erased. Setting the bits in the BPROT register to logic

one protects the EEPROM and CONFIG register until the next reset. Refer to Table

4-6.

Bits [7:5] — Not implemented

Always read zero

PTCON — Protect for CONFIG

0 = CONFIG register can be programmed or erased normally

1 = CONFIG register cannot be programmed or erased

BPRT[3:0] — Block Protect Bits for EEPROM

0 = Protection disabled for associated block

1 = Protection enabled for associated block

OPT2 — System Configuration Options 2 $1038

Bit 7 6 5 4 3 2 1 Bit 0

GWOM CWOM CLK4X — — — — —

RESET: 0 0 0 0 0 0 0 0

BPROT — Block Protect $1035

Bit 7 6 5 4 3 2 1 Bit 0

— — — PTCON BPRT3 BPRT2 BPRT1 BPRT0

RESET: 0 0 0 1 1 1 1 1

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

OPERATING MODES AND ON-CHIP MEMORY MC68HC11F1

4-14 TECHNICAL DATA

4.4 EEPROM and CONFIG Register

The 512-byte EEPROM array and the single-byte CONFIG register are implemented

with the same type of memory cells. The CONFIG register is a separate address located

within the register block rather than in the EEPROM array. Unlike other registers

within the register block, the CONFIG register can only be altered using the EEPROM

programming procedure.

4.4.1 EEPROM

The 512-byte on-board EEPROM is initially located from $FE00 to $FFFF after reset

in single-chip modes. It can be mapped to any other 4-Kbyte boundary by programming

bits EE[3:0] in the CONFIG register. The EEPROM is enabled by the EEON bit

in the CONFIG register. Programming and erasing is controlled by the PPROG register.

Unlike information stored in ROM, data in the 512 bytes of EEPROM can be erased

and reprogrammed under software control. Because programming and erasing operations

use an on-chip charge pump, a separate external power supply is not required.

Use of the block protect register (BPROT) prevents inadvertent writes to (or erases of)

blocks of EEPROM. The CSEL bit in the OPTION register selects an on-chip oscillator

clock for programming and erasing while operating at frequencies below 1 MHz.

4.4.1.1 EEPROM Programming

An exact register access sequence must be followed to allow successful programming

and erasure of the EEPROM. The following procedures for modifying the EEPROM

and CONFIG register detail the sequence. If an attempt is made to set both EELAT

and EEPGM bits in the same write cycle and this attempt occurs before the required

write cycle with the EELAT bit set, then neither bit is set. If a write to an EEPROM address

is performed while the EEPGM bit is set, the write is ignored, and the programming

operation in progress is not disturbed. If no EEPROM address is written between

the point at which EELAT is set and EEPGM is set, then no program or erase operation

occurs. These safeguards are included to prevent accidental EEPROM changes in

cases of program runaway. If the frequency of the E clock is 1 MHz or less, the CSEL

bit in the OPTION register must be set to select the internal RC clock.

When the EELAT bit in the PPROG register is cleared, the EEPROM can be read as

if it were a ROM. The block protect register has no effect during reads. During EEPROM

programming, the ROW and BYTE bits of PPROG are not used.

Table 4-6 EEPROM Block Protection

Bit Name Block Protected Block Size

BPRT0 $xE00–$xE1F 32 Bytes

BPRT1 $xE20–$xE5F 64 Bytes

BPRT2 $xE60–$xEDF 128 Bytes

BPRT3 $xEE0–$xFFF 288 Bytes

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

OPERATING MODES AND ON-CHIP MEMORY

TECHNICAL DATA 4-15

Recall that zeros must be erased by a separate erase operation before programming.

The following example of how to program an EEPROM byte assumes that the appropriate

bits in BPROT have been cleared and the data to be programmed is present in

accumulator A.

4.4.1.2 EEPROM Bulk Erase

To erase the EEPROM, ensure that the proper bits of the BPROT register are cleared,

then complete the following steps using the PPROG register:

1. Write to PPROG with the ERASE, EELAT, and appropriate BYTE and ROW

bits set.

2. Write to the appropriate EEPROM address with any data. Row erase only requires

a write to any location in the row. Bulk erase is accomplished by writing

to any location in the array.

3. Write to PPROG with ERASE, EELAT, EEPGM, and the appropriate BYTE and

ROW bits set.

4. Delay for 10 ms or more, as appropriate.

5. Clear the EEPGM bit in PPROG to turn off the high voltage.

6. Return to step 1 for next byte or row or proceed to step 7.

7. Clear the PPROG register to reconfigure the EEPROM address and data buses

for normal operation.

The following is an example of how to bulk erase the 512-byte EEPROM. The CONFIG

register is not affected in this example. When bulk erasing the CONFIG register, CONFIG

and the 512-byte array are all erased.

4.4.1.3 EEPROM Row Erase

The following example shows how to perform a fast erase of large sections of EEPROM

and assumes that index register X contains the address of a location in the desired

row.

PROG LDAB #$02 EELAT=1, EEPGM=0

STAB $103B Set EELAT bit

STAA $FE00 Store data to EEPROM address

LDAB #$03 EELAT=1, EEPGM=1

STAB $103B Turn on programming voltage

JSR DLY10 Delay 10 ms

CLR $103B Turn off high voltage and set to READ mode

BULKE LDAB #$06 ERASE=1, EELAT=1, EEPGM=0

STAB $103B Set EELAT bit

STAB $FE00 Store any data to any EEPROM address

LDAB #$07 EELAT=1, EEPGM=1

STAB $103B Turn on programming voltage

JSR DLY10 Delay 10 ms

CLR $103B Turn off high voltage and set to READ mode

4.4.1.4 EEPROM Byte Erase

The following is an example of how to erase a single byte of EEPROM and assumes

that index register X contains the address of the byte to be erased.

4.4.2 PPROG EEPROM Programming Control Register

Bits in PPROG register control parameters associated with EEPROM programming.

ODD — Program Odd Rows in Half of EEPROM (TEST)

EVEN — Program Even Rows in Half of EEPROM (TEST)

Bit 5 — Not implemented

Always reads zero

BYTE — Byte/Other EEPROM Erase Mode

0 = Row or bulk erase mode used

1 = Erase only one byte of EEPROM

ROW — Row/All EEPROM Erase Mode (only valid when BYTE = 0)

0 = All 512 bytes of EEPROM erased

1 = Erase only one 16-byte row of EEPROM

ROWE LDAB #$0E ROW=1, ERASE=1, EELAT=1, EEPGM=0

STAB $103B Set to ROW erase mode

STAB 0,X Store any data to any address in ROW

LDAB #$0F ROW=1, ERASE=1, EELAT=1, EEPGM=1

STAB $103B Turn on high voltage

JSR DLY10 Delay 10 ms

CLR $103B Turn off high voltage and set to READ mode

BYTEE LDAB #$16 BYTE=1, ROW=0, ERASE=1, EELAT=1, EEPGM=0

STAB $103B Set to BYTE erase mode

STAB 0,X Store any data to address to be erased

LDAB #$17 BYTE=1, ROW=0, ERASE=1, EELAT=1, EEPGM=1

STAB $103B Turn on high voltage

JSR DLY10 Delay 10 ms

CLR $103B Turn off high voltage and set to READ mode

PPROG — EEPROM Programming Control $103B

Bit 7 6 5 4 3 2 1 Bit 0

ODD EVEN — BYTE ROW ERASE EELAT EEPGM

RESET: 0 0 0 0 0 0 0 0

ERASE — Erase/Normal Control for EEPROM

Can be read or written any time.

0 = Normal read or program mode

1 = Erase mode

EELAT — EEPROM Latch Control

Can be read or written any time. When EELAT equals one, writes to EEPROM cause

address and data to be latched.

0 = EEPROM address and data bus configured for normal reads

1 = EEPROM address and data bus configured for programming or erasing

EEPGM — EEPROM Program Command

Can be read any time. Can only be written while EELAT = 1.

0 = Program or erase voltage switched off to EEPROM array

1 = Program or erase voltage switched on to EEPROM array

4.4.3 CONFIG Register Programming

Because the CONFIG register is implemented with EEPROM cells, use EEPROM procedures

to erase and program this register. The procedure for programming is the

same as for programming a byte in the EEPROM array, except that the CONFIG register

address is used. CONFIG can be programmed or erased (including byte erase)

while the MCU is operating in any mode, provided that PTCON in BPROT is clear. To

change the value in the CONFIG register, complete the following procedure. Do not

initiate a reset until the procedure is complete. The new value will not take effect until

after the next reset sequence.

1. Erase the CONFIG register.

2. Program the new value to the CONFIG address.

3. Initiate reset.

P indicates a previously programmed bit. P(L) indicates that the bit resets to the logic

level held in the EEPROM bit prior to reset, but the function of COP is controlled by

DISR bit in TEST1 register.

Table 4-7 EEPROM Erase Mode Control

BYTE ROW Action

0 0 Bulk Erase (All 512 Bytes)

0 1 Row Erase (16 Bytes)

1 0 Byte Erase

1 1 Byte Erase

CONFIG — System Configuration Register $103F

Bit 7 6 5 4 3 2 1 Bit 0

EE3 EE2 EE1 EE0 — NOCOP — EEON

RESET: 1 1 1 1 1 P 1 1 Single Chip

1 1 1 1 1 P(L) 1 1 Bootstrap

P P P P 1 P 1 P Expanded

P P P P 1 P(L) 1 0 Special Test

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

OPERATING MODES AND ON-CHIP MEMORY MC68HC11F1

4-18 TECHNICAL DATA

For a description of the bits contained in the CONFIG register refer to 4.3.2.1 CONFIG

Register.

4.5 Chip Selects

The function of the chip selects is to minimize the amount of external glue logic needed

to interface the MCU to external devices. The MC68HC11F1 has four software configured

chip selects that can be enabled in expanded modes. The chip selects for I/O

(CSIO1 and CSIO2) are used for I/O expansion. The program chip select (CSPROG)

is used with an external memory that contains the program code and reset vectors.

The general-purpose chip select (CSGEN) is the most flexible and is used to enable

external devices.

Such factors as polarity, block size, base address and clock stretching can be controlled

using the four chip-select control registers. When a port G pin is not used for

chip select functions it can be used for general-purpose I/O.

When enabled, a chip select signal is asserted whenever the CPU makes an access

to a designated range of addresses. Bus control signals and chip select signals are

synchronous with the external E clock signal. For more information refer to Table A–

7. Expansion Bus Timing in APPENDIX A ELECTRICAL CHARACTERISTICS. The

length of the external E clock cycle to which the external device is synchronized can

be stretched to accommodate devices that are slower than the MCU.

4.5.1 Program Chip Select

The program chip select (CSPROG) is active in the range of memory where the main

program exists. Refer to Figure 4-3.

When enabled, the CSPROG is active during address valid time and is an active-low

signal. Although the general-purpose chip select has priority over the program chip select,

CSPROG can be raised to a higher priority level by setting the GCSPR bit in

CSCTL register. Bits in CSCTL enable the program chip select and determine its address

range and priority level. Bits in CSSTRH select from zero to three clock cycles

of delay.

4.5.2 I/O Chip Selects

The I/O chip selects (CSIO1 and CSIO2) are fixed in size and fill the remainder of the

4-Kbyte block occupied by the register block. CSIO1 is mapped at $x060–$x7FF and

CSIO2 is mapped at $x800–$xFFF, where “x” corresponds to the high-order nibble of

the register block base address, represented by the value contained in REG[3:0] in the

INIT register.

Bits in the CSCTL register determine the polarity of the active state and enable both I/

O chip selects. Bits in CSGSIZ select whether each chip select is active for addressvalid

or E-valid time. Bits in CSSTRH select from zero to three clock cycles of delay.

Refer to Figure 4-3.

Figure 4-3 Address Map for I/O and Program Chip Selects

4.5.3 General-Purpose Chip Select

The general-purpose chip select (CSGEN) is the most flexible and has the most control

bits. Polarity of the active state, E-valid or address-valid timing, size, starting address,

and clock delay are all programmable.

A single bit in CSCTL selects a priority between CSGEN and CSPROG. Bits in CSGSIZ

select between address valid or E-clock valid timing, determine the polarity of the

active state and the address range of CSGEN. The value contained in the CSGADR

register determines the starting address for CSGEN. Depending on the size selected

for CSGEN, some bits in CSGADR will be invalid (don’t cares). Note that CSGEN is

disabled when a size of zero is selected. Refer to Figure 4-4.

$0000

$1000

$FFFF

EXPANDED

MODE

$FE00

FFC0

FFFF

VECTORS

0000

FFFF

E000

C000

8000

x000

x05F

96-BYTE REGISTER

BLOCK

x060

x7FF

I/O CHIP SELECT 1

(CSIO1)

x800

xFFF

I/O CHIP SELECT 2

(CSIO2)

REMAPPABLE TO

4-KBYTE BOUNDARY

$E000

$C000

$8000

PSIZ[A:B] = 0:0

64K

PSIZ[A:B] = 0:1

32K

PSIZ[A:B] = 1:0

16K

PSIZ[A:B] = 1:1

8K

PROGRAM CHIP SELECT

(CSPROG)

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

OPERATING MODES AND ON-CHIP MEMORY MC68HC11F1

4-20 TECHNICAL DATA

Figure 4-4 Address Map for General-Purpose Chip Select

IO1SA–IO1SB — I/O Chip Select 1 Clock Stretch Select

Refer to Table 4-8.

IO2SA–IO2SB — I/O Chip Select 2 Clock Stretch Select

Refer to Table 4-8.

GSTHA–GSTHB — General-Purpose Chip Select Clock Stretch Select

Refer to Table 4-8.

PSTHA–PSTHB — Program Chip Select Clock Stretch Select

Refer to Table 4-8.

CSSTRH — Chip Select Clock Stretch Select $105C

Bit 7 6 5 4 3 2 1 Bit 0

IO1SA IO1SB IO2SA IO2SB GSTHA GSTHB PSTHA PSTHB

RESET: 0 0 0 0 0 0 0 0

Table 4-8 Chip Select Clock Stretch Control

Bit A Bit B Clock Stretch Selected

0 0 None

0 1 1 cycle

1 0 2 cycles

1 1 3 cycles

$0000

$FFFF

EXP

MODE

ADDR

SPACE

N/A GA15 GA[15:14] GA[15:13] GA[15:12] GA[15:11] GA[15:10]

0 : 0 : 0 0 : 0 : 1 0 : 1 : 0 0 : 1 : 1 1 : 0 : 0 1 : 0 : 1 1 : 1 : 0

VALID BASE ADDR BITS:

GSIZA : GSIZB : GSIZC:

SIZE: 64K 32K 16K 8K 4K 2K 1K

NOTE: These examples assume a starting address of $0000.

GA[15:10]

1 : 1 : 1

DISABLED

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

OPERATING MODES AND ON-CHIP MEMORY

TECHNICAL DATA 4-21

*PCSEN is set out of reset in expanded modes and cleared in single-chip modes.

IO1EN — I/O Chip Select 1 Enable

0 = CSIO1 is disabled and port G bit 5 is general-purpose I/O.

1 = CSIO1 is enabled and uses port G bit 5.

IO1PL — I/O Chip Select 1 Polarity Select

0 = CSIO1 active low

1 = CSIO1 active high

IO2EN — I/O Chip Select 2 Enable

0 = CSIO2 is disabled and port G bit 4 is general-purpose I/O.

1 = CSIO2 is enabled and uses port G bit 4.

IO2PL — I/O Chip Select 2 Polarity Select

0 = CSIO2 active low

1 = CSIO2 active high

GCSPR — General-Purpose Chip Select Priority

0 = Program chip select has priority over general-purpose chip select

1 = General-purpose chip select has priority over program chip select

PCSEN — Program Chip Select Enable

This bit is set out of reset in expanded modes and cleared in single-chip modes.

0 = CSPROG disabled and port G bit 7 available as general-purpose I/O

1 = CSPROG enabled out of reset and uses port G bit 7 pin

PSIZA, PSIZB — Program Chip Select Size (A or B)

GA[15:10] — General-Purpose Chip Select Base Address

GA[15:10] correspond to MCU address bits ADDR[15:10] and select the starting address

of the general-purpose chip select's address range. Which bits are valid depends

upon the size selected by GSIZA–GSIZC in CSGSIZ register. Refer to the

following table and to Figure 4-4.

CSCTL — Chip Select Control $105D

Bit 7 6 5 4 3 2 1 Bit 0

IO1EN IO1PL IO2EN IO2PL GCSPR PCSEN* PSIZA PSIZB

RESET: 0 0 0 0 0 — 0 0

Table 4-9 Program Chip Select Size Control

PSIZA PSIZB Size (Bytes) Address Range

0 0 64 K $0000–$FFFF

0 1 32 K $8000–$FFFF

1 0 16 K $C000–$FFFF

1 1 8 K $E000–$FFFF

CSGADR — General-Purpose Chip Select Address Register $105E

Bit 7 6 5 4 3 2 1 Bit 0

GA15 GA14 GA13 GA12 GA11 GA10 — —

RESET: 0 0 0 0 0 0 0 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

OPERATING MODES AND ON-CHIP MEMORY MC68HC11F1

4-22 TECHNICAL DATA

IO1AV — I/O Chip Select 1 Address Valid

0 = I/O chip select 1 is active during E-clock valid time (E-clock high)

1 = I/O chip select 1 is active during address valid time

IO2AV — I/O Chip Select 2 Address Valid

0 = I/O chip select 1 is active during E-clock valid time (E-clock high)

1 = I/O chip select 1 is active during address valid time

GNPOL — General-Purpose Chip Select Polarity Select

0 = CSGEN is active low

1 = CSGEN is active high

GAVLD — General-Purpose Chip Select Address Valid Select

0 = CSGEN is valid during E-clock valid time (E-clock high)

1 = CSGEN is valid during address valid time

G1SZA–G1SZC — General-Purpose Chip Select Size

Refer to Table 4-11.

Table 4-10 General-Purpose Chip Select Starting Address

Size (Bytes) Valid Starting Address

Bits

0 K (Disabled) None

1 K GA[15:10]

2 K GA[15:11]

4 K GA[15:12]

8 K GA[15:13]

16 K GA[15:14]

32 K GA15

64 K None

CSGSIZ — General-Purpose Chip Select Size Control $105F

Bit 7 6 5 4 3 2 1 Bit 0

IO1AV IO2AV — GNPOL GAVLD GSIZA GSIZB GSIZC

RESET: 0 0 0 0 0 1 1 1

Table 4-11 General-Purpose Chip Select Size Control

GSIZA GSIZB GSIZC Size (Bytes)

0 0 0 64 K

0 0 1 32 K

0 1 0 16 K

0 1 1 8 K

1 0 0 4 K

1 0 1 2 K

1 1 0 1 K

1 1 1 0 K (Disabled)

Table 4-12 Chip Select Control Parameter Summary

CSIO1 Enable IO1EN in CSCTL — 1 = On, off at reset (0)

Valid IO1AV in CSGSIZ — 1 = Address valid, 0 = E valid

Polarity IO1PL in CSCTL — 1 = Active high, 0 = Active low

Size Fixed — ($x060–$x7FF)

Start Address $x060 — “x” is determined by REG[3:0] in INIT

Stretch IO1SA–IO1SB in CSSTRH — 0, 1, 2, or 3 E clocks

CSIO2 Enable IO2EN in CSCTL — 1 = On, off at reset (0)

Valid IO2AV in CSGSIZ — 1 = Address valid, 0 = E valid

Polarity IO2PL in CSCTL — 1 = Active high, 0 = Active low

Size Fixed — ($x800–$xFFF)

Start Address $x800 — “x” is determined by REG[3:0] in INIT

Stretch IO2SA–IO2SB in CSSTRH — 0, 1, 2, or 3 E clocks

CSPROG Enable PCSEN in CSCTL — 1 = On, on after reset in expanded modes

off after reset in single-chip modes

Valid Fixed (Address valid)

Polarity Fixed (Active low)

Size PSIZA–PSIZB — 0:0 = 64K ($0000–$FFFF)

in CSCTL 0:1 = 32K ($8000–$FFFF)

1:0 = 16K ($C000–$FFFF)

1:1 = 8K ($E000–$FFFF)

Start Address Fixed (determined by size)

Stretch PSTHA–PSTHB in CSSTRH — 0, 1, 2, or 3 E clocks

1 cycle after reset in expanded mode

no delay after reset in all other modes

Priority GCSPR in CSCTL — 1 = CSGEN above CSPROG

0 = CSPROG above CSGEN

CSGEN Enable Set size to 0K to disable — 1 = CSGEN above CSPROG

0 = CSPROG above CSGEN

Valid GAVLD in CSGSIZ — Address valid or E valid

Polarity GNPOL in CSGSIZ — Active high or low

Size GSIZA–GSIZC in CSGSIZ — Refer to Table 4–12

Start Address GA[15:10] in CSGADR

Stretch GSTHA–GSTHB in CSSTRH — 0, 1, 2, or 3 E clocks

SECTION 5 RESETS AND INTERRUPTS

Resets and interrupt operations load the program counter with a vector that points to a new location from which instructions are to be fetched. A reset causes the internal control registers to be initialized to a known state. The program counter is loaded with a known starting address and execution of instructions begins. An interrupt temporarily suspends normal program execution while an interrupt service routine is being executed. After an interrupt has been serviced, the main program resumes as if there had been no interruption.

5.1 Resets

There are four possible sources of reset. Power-on reset (POR) and external reset share the normal reset vector. The computer operating properly (COP) reset and the clock monitor reset each has its own vector.

5.1.1 Power-On Reset

A positive transition on VDD generates a power-on reset (POR), which is used only for power-up conditions. POR cannot be used to detect drops in power supply voltages. A 4064 tcyc (internal clock cycle) delay after the oscillator becomes active allows the clock generator to stabilize. If RESET is at logical zero at the end of 4064 tcyc, the CPU remains in the reset condition until RESET goes to logical one.

It is important to protect the MCU during power transitions. To protect data in EEPROM, M68HC11 systems need an external circuit that holds the RESET pin low whenever VDD is below the minimum operating level. This external voltage level detector, or other external reset circuits, are the usual source of reset in a system. The POR circuit only initializes internal circuitry during cold starts. Refer to Figure 2–3.

5.1.2 External Reset (RESET)

The CPU distinguishes between internal and external reset conditions by sensing whether the reset pin rises to a logic one in less than two E-clock cycles after an internal device releases reset. When a reset condition is sensed, the RESET pin is driven low by an internal device for four E-clock cycles, then released. Two E-clock cycles later it is sampled. If the pin is still held low, the CPU assumes that an external reset has occurred. If the pin is high, it indicates that the reset was initiated internally by either the COP system or the clock monitor. It is not advisable to connect an external resistor capacitor (RC) power-up delay circuit to the reset pin of M68HC11 devices because the circuit charge time constant can cause the device to misinterpret the type of reset that occurred.

5.1.3 Computer Operating Properly (COP) Reset

The MCU includes a COP system to help protect against software failures. When the COP is enabled, the software is responsible for keeping a free-running watchdog timer from timing out. When the software is no longer being executed in the intended sequence, a system reset is initiated.

The state of the NOCOP bit in the CONFIG register determines whether the COP system is enabled or disabled. To change the enable status of the COP system, change the contents of the CONFIG register and then perform a system reset. In the special test and bootstrap operating modes, the COP system is initially inhibited by the disable resets (DISR) control bit in the TEST1 register. The DISR bit can subsequently be written to zero to enable COP resets.

The COP timer rate control bits CR[1:0] in the OPTION register determine the COP time-out period. The system E clock is divided by the values shown in Table 5-1. After reset, these bits are zero, which selects the fastest time-out period. In normal operating modes, these bits can only be written once within 64 bus cycles after reset. Complete the following reset sequence to service the COP timer. Write $55 to COPRST to arm the COP timer clearing mechanism. Then write $AA to COPRST to clear the COP timer. Performing instructions between these two steps is possible as long as both steps are completed in the correct sequence before the timer times out.

5.1.4 Clock Monitor Reset

The clock monitor circuit is based on an internal RC time delay. If no MCU clock edges are detected within this RC time delay, the clock monitor can optionally generate a system reset. The clock monitor function is enabled or disabled by the CME and FCME control bits in the OPTION register. The presence of a time-out is determined by the RC delay, which allows the clock monitor to operate without any MCU clocks. Clock monitor is used as a backup for the COP system. Because the COP needs a clock to function, it is disabled when the clocks stop. Therefore, the clock monitor system can detect clock failures not detected by the COP system.

Table 5-1 COP Timer Rate Selection

Semiconductor wafer processing causes variations of the RC time-out values between individual devices. An E-clock frequency below 10 kHz is detected as a clock monitor error. An E-clock frequency of 200 kHz or more prevents clock monitor errors. Using the clock monitor function when the E-clock is below 200 kHz is not recommended. Special considerations are needed when a STOP instruction is executed and the clock monitor is enabled. Because the STOP function causes the clocks to be halted, the clock monitor function generates a reset sequence if it is enabled at the time the STOP mode was initiated. Before executing a STOP instruction, clear to zero the CME bit in the OPTION register to disable the clock monitor. After recovery from STOP, set the CME bit to logic one to enable the clock monitor.

5.1.5 OPTION Register

*Can be written only once in first 64 cycles out of reset in normal modes, or at any time in special modes.

ADPU — Analog-to-Digital Converter Power-Up

Refer to SECTION 10 ANALOG-TO-DIGITAL CONVERTER.

CSEL — Clock Select

Refer to SECTION 10 ANALOG-TO-DIGITAL CONVERTER.

IRQE — Configure IRQ for Edge-Sensitive Only Operation

0 = Low level sensitive operation.

1 = Falling edge sensitive only operation.

DLY — Enable Oscillator Start-up Delay

0 = The oscillator start-up delay coming out of STOP is bypassed and the MCU resumes processing within about four bus cycles.

1 = A delay of approximately 4000 E-clock cycles is imposed as the MCU is started up from the STOP power-saving mode.

CME — Clock Monitor Enable

This control bit can be read or written at any time and controls whether or not the internal clock monitor circuit triggers a reset sequence when the system clock is slow or absent. When it is clear, the clock monitor circuit is disabled, and when it is set, the clock monitor circuit is enabled. Reset clears the CME bit.

FCME — Force Clock Monitor Enable

To use STOP mode, the FCME bit must equal zero.

0 = Clock monitor follows the state of the CME bit.

1 = Clock monitor circuit is enabled until next reset

OPTION — System Configuration Options $1039

Bit 7 6 5 4 3 2 1 Bit 0

ADPU CSEL IRQE* DLY* CME FCME* CR1* CR0*

RESET: 0 0 0 1 0 0 0 0

CR[1:0] — COP Timer Rate Select

The internal E clock is first divided by 215 before it enters the COP watchdog system. These control bits determine a scaling factor for the watchdog timer. Refer to Table 5-1.

5.1.6 CONFIG Register

P indicates a previously programmed bit. P(L) indicates that the bit resets to the logic level held in the latch prior to reset, but the function of COP is controlled by DISR in TEST1 register.

EE[3:0] — EEPROM Mapping Control

Refer to SECTION 4 OPERATING MODES AND ON-CHIP MEMORY.

Bit 3 — Not implemented

Always reads one

NOCOP — COP System Disable

0 = COP system enabled (forces reset on time-out)

1 = COP system disabled

Bit 1 — Not implemented

Always reads one

EEON — EEPROM Enable

Refer to SECTION 4 OPERATING MODES AND ON-CHIP MEMORY.

5.2 Effects of Reset

When a reset condition is recognized, the internal registers and control bits are forced to an initial state. Depending on the cause of the reset and the operating mode, the reset vector can be fetched from any of six possible locations. Refer to Table 5-2. These initial states then control on-chip peripheral systems to force them to known start-up states, as follows:

CONFIG — System Configuration Register $103F

Bit 7 6 5 4 3 2 1 Bit 0

EE3 EE2 EE1 EE0 — NOCOP — EEON

RESET: 1 1 1 1 1 P 1 1 Single Chip

1 1 1 1 1 P(L) 1 1 Bootstrap

P P P P 1 P 1 P Expanded

P P P P 1 P(L) 1 0 Special Test

Table 5-2 Reset Cause, Operating Mode, and Reset Vector

Cause of Reset Normal Mode Vector Special Test or Bootstrap

POR or RESET Pin $FFFE, FFFF $BFFE, $BFFF

Clock Monitor Failure $FFFC, FFFD $BFFC, $BFFD

COP Watchdog Time-out $FFFA, FFFB $BFFA, $BFFB

5.2.1 Central Processing Unit

After reset, the CPU fetches the reset vector from the appropriate address during the first three cycles, and begins executing instructions. The stack pointer and other CPU registers are indeterminate immediately after reset; however, the X and I interrupt mask bits in the condition code register (CCR) are set to mask any interrupt requests. Also, the S bit in the CCR is set to inhibit the STOP mode.

5.2.2 Memory Map

After reset, the INIT register is initialized to $01, putting the 1024 bytes of RAM at locations $0000 through $03FF, and the control registers at locations $1000 through $105F. The EE[3:0] bits in the CONFIG register control the location of the 512-byte EEPROM array.

5.2.3 Parallel I/O

When a reset occurs in expanded operating modes, port B, C, and F pins used for parallel I/O are dedicated to the expansion bus. If a reset occurs during a single-chip operating mode, all ports are configured as general-purpose high-impedance inputs.

NOTE

Do not confuse pin function with the electrical state of the pin at reset. All general-purpose I/O pins configured as inputs at reset are in a high-impedance state. Port data registers reflect the port's functional state at reset. The pin function is mode dependent.

5.2.4 Timer

During reset, the timer system is initialized to a count of $0000. The prescaler bits are cleared, and all output compare registers are initialized to $FFFF. All input capture registers are indeterminate after reset. The output compare 1 mask (OC1M) register is cleared so that successful OC1 compares do not affect any I/O pins. The other four output compares are configured so that they do not affect any I/O pins on successful compares. All input capture edge-detector circuits are configured for capture disabled operation. The timer overflow interrupt flag and all eight timer function interrupt flags are cleared. All nine timer interrupts are disabled because their mask bits have been cleared.

The I4/O5 bit in the PACTL register is cleared to configure the I4/O5 function as OC5; however, the OM5–OL5 control bits in the TCTL1 register are clear so OC5 does not control the PA3 pin.

5.2.5 Real-Time Interrupt (RTI)

The real-time interrupt flag (RTIF) is cleared and automatic hardware interrupts are masked. The rate control bits are cleared after reset and can be initialized by software before the real-time interrupt (RTI) system is used.

5.2.6 Pulse Accumulator

The pulse accumulator system is disabled at reset so that the pulse accumulator input (PAI) pin defaults to being a general-purpose input pin.

5.2.7 Computer Operating Properly (COP)

The COP watchdog system is enabled if the NOCOP control bit in the CONFIG register is cleared, and disabled if NOCOP is set. The COP rate is set for the shortest duration time-out.

5.2.8 Serial Communications Interface (SCI)

The reset condition of the SCI system is independent of the operating mode. All transmit and receive interrupts are masked and both the transmitter and receiver are disabled so the port pins default to being general-purpose I/O lines. The SCI frame format is initialized to an 8-bit character size. The send break and receiver wakeup functions are disabled. The TDRE and TC status bits in the SCI status register are both set, indicating that there is no transmit data in either the transmit data register or the transmit serial shift register. The RDRF, IDLE, OR, NF, FE, PF, and RAF receive-related status bits are cleared.

5.2.9 Serial Peripheral Interface (SPI)

The SPI system is disabled by reset. The port pins associated with this function default to being general-purpose I/O lines.

5.2.10 Analog-to-Digital Converter

The A/D converter configuration is indeterminate after reset. The ADPU bit is cleared by reset, which disables the A/D system. The conversion complete flag is cleared by reset.

5.2.11 System

The EEPROM programming controls are disabled, so the memory system is configured for normal read operation. PSEL[3:0] are initialized with the binary value %0101, causing the external IRQ pin to have the highest I-bit interrupt priority. The IRQ pin is configured for level-sensitive operation (for wired-OR systems). The RBOOT, SMOD, and MDA bits in the HPRIO register reflect the status of the MODB and MODA inputs at the rising edge of reset. The DLY control bit is set to specify that an oscillator startup delay is imposed upon recovery from STOP mode. The clock monitor system is disabled because CME and FCME are cleared.

5.3 Reset and Interrupt Priority

Resets and interrupts have a hardware priority that determines which reset or interrupt is serviced first when simultaneous requests occur. Any maskable interrupt can be given priority over other maskable interrupts.

The first six interrupt sources are not maskable. The priority arrangement for these sources is as follows:

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

RESETS AND INTERRUPTS

TECHNICAL DATA 5-7

1. POR or RESET pin

2. Clock monitor reset

3. COP watchdog reset

4. XIRQ interrupt

5. Illegal opcode interrupt

6. Software interrupt (SWI)

The maskable interrupt sources have the following priority arrangement:

1. IRQ

2. Real-time interrupt

3. Timer input capture 1

4. Timer input capture 2

5. Timer input capture 3

6. Timer output compare 1

7. Timer output compare 2

8. Timer output compare 3

9. Timer output compare 4

10.Timer input capture 4/output compare 5

11.Timer overflow

12.Pulse accumulator overflow

13.Pulse accumulator input edge

14.SPI transfer complete

15.SCI system (refer to Figure 5-5)

Any one of these interrupts can be assigned the highest maskable interrupt priority by writing the appropriate value to the PSEL bits in the HPRIO register. Otherwise, the priority arrangement remains the same. An interrupt that is assigned highest priority is still subject to global masking by the I bit in the CCR, or by any associated local bits. Interrupt vectors are not affected by priority assignment. To avoid race conditions, HPRIO can only be written while I-bit interrupts are inhibited.

5.3.1 Highest Priority Interrupt and Miscellaneous Register

*The values of the RBOOT, SMOD, MDA, and IRV reset bits depend on the operating mode selected during powerup.

Refer to Table 4–3.

RBOOT — Read Bootstrap ROM

Set to one out of reset in bootstrap mode. Valid while in special modes only. Can be

read any time. Can only be written in special modes. Refer to SECTION 4 OPERATING

MODES AND ON-CHIP MEMORY for more information.

HPRIO — Highest Priority I-Bit Interrupt and Miscellaneous $103C

Bit 7 6 5 4 3 2 1 Bit 0

RBOOT* SMOD* MDA* IRV PSEL3 PSEL2 PSEL1 PSEL0

RESET: 0 0 0 0 0 1 0 1 Single Chip

0 0 1 1 0 1 0 1 Expanded

1 1 0 0 0 1 0 1 Bootstrap

0 1 1 1 0 1 0 1 Special Test

SMOD — Special Mode Select

Can be read any time. Can only be written in special modes (SMOD = 1). Can only be written to zero. Refer to SECTION 4 OPERATING MODES AND ON-CHIP MEMORY for more information.

MDA — Mode Select A

Can be read any time. Can only be written in special modes (SMOD = 1). Refer to SECTION 4 OPERATING MODES AND ON-CHIP MEMORY for more information.

IRV — Internal Read Visibility

The IRV control bit allows internal read accesses to be available on the external data bus during operation in expanded modes. In special modes (SMOD = 1), IRV resets to one (enabled) and can be written any time. In normal modes (SMOD = 0), IRV resets to zero (disabled) and only one write is allowed.

PSEL[3:0] — Priority Select Bits

These bits select one interrupt source to be elevated above all other I-bit-related sources and can only be written while the I bit in the CCR is set (interrupts disabled).

5.4 Interrupts

The MCU has 18 interrupt vectors that support 22 interrupt sources. The 15 maskable interrupts are generated by on-chip peripheral systems. These interrupts are recognized when the global interrupt mask bit (I) in the condition code register (CCR) is clear. The three non-maskable interrupt sources are illegal opcode trap, software interrupt, and XIRQ pin. Refer to Table 5-4, which shows the interrupt sources and vector assignments for each source.

Table 5-3 Highest Priority Interrupt Selection

PSEL3 PSEL2 PSEL1 PSEL0 Interrupt Source Promoted

0 0 0 0 Timer Overflow

0 0 0 1 Pulse Accumulator Overflow

0 0 1 0 Pulse Accumulator Input Edge

0 0 1 1 SPI Serial Transfer Complete

0 1 0 0 SCI Serial System

0 1 0 1 Reserved (Default to IRQ)

0 1 1 0 IRQ

0 1 1 1 Real-Time Interrupt

1 0 0 0 Timer Input Capture 1

1 0 0 1 Timer Input Capture 2

1 0 1 0 Timer Input Capture 3

1 0 1 1 Timer Output Compare 1

1 1 0 0 Timer Output Compare 2

1 1 0 1 Timer Output Compare 3

1 1 1 0 Timer Output Compare 4

1 1 1 1 Timer Output Compare 5/Input Capture 4

For some interrupt sources, such as the SCI interrupts, the flags are automatically cleared during the normal course of responding to the interrupt requests. For example, the RDRF flag in the SCI system is cleared by the automatic clearing mechanism consisting of a read of the SCI status register while RDRF is set, followed by a read of the SCI data register. The normal response to an RDRF interrupt request would be to read the SCI status register to check for receive errors, then to read the received data from the SCI data register. These two steps satisfy the automatic clearing mechanism without requiring any special instructions.

5.4.1 Interrupt Recognition and Register Stacking

An interrupt can be recognized at any time after it is enabled by its local mask, if any, and by the global mask bit in the CCR. Once an interrupt source is recognized, the CPU responds at the completion of the instruction being executed. Interrupt latency varies according to the number of cycles required to complete the current instruction.

When the CPU begins to service an interrupt, the contents of the CPU registers are pushed onto the stack in the order shown in Table 5-5. After the CCR value is stacked, the I bit and the X bit (if XIRQ is pending) are set to inhibit further interrupts. The interrupt vector for the highest priority pending source is fetched, and execution continues at the address specified by the vector. At the end of the interrupt service routine, the return from interrupt instruction is executed and the saved registers are pulled from the stack in reverse order so that normal program execution can resume. Refer to SECTION 3 CENTRAL PROCESSING UNIT for further information.

Table 5-4 Interrupt and Reset Vector Assignments

Vector Address Interrupt Source CCR

Mask Bit

Local Mask

FFC0, C1 – FFD4, D5 Reserved — —

FFD6, D7 SCI Serial System I

• SCI Receive Data Register Full RIE

• SCI Receiver Overrun RIE

• SCI Transmit Data Register Empty TIE

• SCI Transmit Complete TCIE

• SCI Idle Line Detect ILIE

FFD8, D9 SPI Serial Transfer Complete I SPIE

FFDA, DB Pulse Accumulator Input Edge I PAII

FFDC, DD Pulse Accumulator Overflow I PAOVI

FFDE, DF Timer Overflow I TOI

FFE0, E1 Timer Input Capture 4/Output Compare 5 I I4/O5I

FFE2, E3 Timer Output Compare 4 I OC4I

FFE4, E5 Timer Output Compare 3 I OC3I

FFE6, E7 Timer Output Compare 2 I OC2I

FFE8, E9 Timer Output Compare 1 I OC1I

FFEA, EB Timer Input Capture 3 I IC3I

FFEC, ED Timer Input Capture 2 I IC2I

FFEE, EF Timer Input Capture 1 I IC1I

FFF0, F1 Real-Time Interrupt I RTII

FFF2, F3 IRQ I None

FFF4, F5 XIRQ Pin X None

FFF6, F7 Software Interrupt None None

FFF8, F9 Illegal Opcode Trap None None

FFFA, FB COP Failure None NOCOP

FFFC, FD Clock Monitor Fail None CME

FFFE, FF RESET None None

5.4.2 Non-Maskable Interrupt Request (XIRQ)

Non-maskable interrupts are useful because they can always interrupt CPU operations. The most common use for such an interrupt is for serious system problems, such as program runaway or power failure. The XIRQ input is an updated version of the NMI input of earlier MCUs.

Upon reset, both the X bit and I bit of the CCR are set to inhibit all maskable interrupts and XIRQ. After minimum system initialization, software can clear the X bit by a TAP instruction, enabling XIRQ interrupts. Thereafter, software cannot set the X bit. Thus, an XIRQ interrupt is a nonmaskable interrupt. Because the operation of the I-bit-related interrupt structure has no effect on the X bit, the internal XIRQ pin remains nonmasked. In the interrupt priority logic, the XIRQ interrupt has a higher priority than any source that is maskable by the I bit. All I-bit-related interrupts operate normally with their own priority relationship.

When an I-bit-related interrupt occurs, the I bit is automatically set by hardware after stacking the CCR byte. The X bit is not affected. When an X-bit-related interrupt occurs, both the X and I bits are automatically set by hardware after stacking the CCR. A return from interrupt instruction restores the X and I bits to their pre-interrupt request state.

5.4.3 Illegal Opcode Trap

Because not all possible opcodes or opcode sequences are defined, the MCU includes an illegal opcode detection circuit, which generates an interrupt request. When an illegal opcode is detected and the interrupt is recognized, the current value of the program counter is stacked. After interrupt service is complete, reinitialize the stack pointer so repeated execution of illegal opcodes does not cause stack underflow. Left uninitialized, the illegal opcode vector can point to a memory location that contains an illegal opcode. This condition causes an infinite loop that causes stack underflow. The stack grows until the system crashes.

Table 5-5 Stacking Order on Entry to Interrupts

Memory Location CPU Registers

SP PCL

SP – 1 PCH

SP – 2 IYL

SP – 3 IYH

SP – 4 IXL

SP – 5 IXH

SP – 6 ACCA

SP – 7 ACCB

SP – 8 CCR

The illegal opcode trap mechanism works for all unimplemented opcodes on all four opcode map pages. The address stacked as the return address for the illegal opcode interrupt is the address of the first byte of the illegal opcode. Otherwise, it would be almost impossible to determine whether the illegal opcode had been one or two bytes. The stacked return address can be used as a pointer to the illegal opcode so the illegal opcode service routine can evaluate the offending opcode.

5.4.4 Software Interrupt

SWI is an instruction, and thus cannot be interrupted until complete. SWI is not inhibited by the global mask bits in the CCR. Because execution of SWI sets the I mask bit, once an SWI interrupt begins, other interrupts are inhibited until SWI is complete, or until user software clears the I bit in the CCR.

5.4.5 Maskable Interrupts

The maskable interrupt structure of the MCU can be extended to include additional external interrupt sources through the IRQ pin. The default configuration of this pin is a low-level sensitive wired-OR network. When an event triggers an interrupt, a software accessible interrupt flag is set. When enabled, this flag causes a constant request for interrupt service. After the flag is cleared, the service request is released.

5.4.6 Reset and Interrupt Processing

Figure 5-1 and Figure 5-3 illustrate the reset and interrupt process. Figure 5-1 illustrates how the CPU begins from a reset and how interrupt detection relates to normal opcode fetches. Figure 5-3 is an expansion of a block in Figure 5-1 and illustrates interrupt priorities. Figure 5-5 shows the resolution of interrupt sources within the SCI subsystem.

Figure 5-1 Processing Flow Out of Reset (1 of 2)

Figure 5-2 Processing Flow Out of Reset (2 of 2)

Figure 5-3 Interrupt Priority Resolution (1 of 2)

Figure 5-4 Interrupt Priority Resolution (2 of 2)

Figure 5-5 Interrupt Source Resolution Within SCI

5.5 Low Power Operation

Both STOP and WAIT suspend CPU operation until a reset or interrupt occurs. The WAIT condition suspends processing and reduces power consumption to an intermediate level. The STOP condition turns off all on-chip clocks and reduces power consumption to an absolute minimum while retaining the contents of all 1024 bytes of RAM.

5.5.1 WAIT

The WAI opcode places the MCU in the WAIT condition, during which the CPU registers are stacked and CPU processing is suspended until a qualified interrupt is detected. The interrupt can be an external IRQ, an XIRQ, or any of the internally generated interrupts, such as the timer or serial interrupts. The on-chip crystal oscillator remains active throughout the WAIT standby period.

The reduction of power in the WAIT condition depends on how many internal clock signals driving on-chip peripheral functions can be shut down. The CPU is always shut down during WAIT. While in the wait state, the address/data bus repeatedly runs read cycles to the address where the CCR contents were stacked. Ensuring that the stack contents are placed in internal RAM will further reduce power consumption. The MCU leaves the wait state when it senses any interrupt that has not been masked.

The free-running timer system is shut down only if the I bit is set to one and the COP system is disabled by NOCOP being set to one. Several other systems can also be in a reduced power consumption state depending on the state of software-controlled configuration control bits. Power consumption by the analog-to-digital (A/D) converter is not affected significantly by the WAIT condition. However, the A/D converter current can be eliminated by writing the ADPU bit to zero. The SPI system is enabled or disabled by the SPE control bit. The SCI transmitter is enabled or disabled by the TE bit, and the SCI receiver is enabled or disabled by the RE bit. Therefore the power consumption in WAIT is dependent on the particular application.

5.5.2 STOP

Executing the STOP instruction while the S bit in the CCR is equal to zero places the MCU in the STOP condition. If the S bit is not zero, the STOP opcode is treated as a no-op (NOP). The STOP condition offers minimum power consumption because all clocks, including the crystal oscillator, are stopped while in this mode. To exit STOP and resume normal processing, a logic low level must be applied to one of the external interrupts (IRQ or XIRQ) or to the RESET pin. A pending edge-triggered IRQ can also bring the CPU out of STOP.

Because all clocks are stopped in this mode, all internal peripheral functions also stop. The data in the internal RAM is retained as long as VDD power is maintained. The CPU state and I/O pin levels are static and are unchanged by STOP. Therefore, when an interrupt restarts the system, the MCU resumes processing as if there were no interruption. If reset is used to restart the system a normal reset sequence results where all I/O pins and functions are also restored to their initial states.

To use the IRQ pin as a means of recovering from STOP, the I bit in the CCR must be clear (IRQ not masked). The XIRQ pin can be used to wake up the MCU from STOP regardless of the state of the X bit in the CCR, although the recovery sequence depends on the state of the X bit. If X is set to zero (XIRQ not masked), the MCU starts up, beginning with the stacking sequence leading to normal service of the XIRQ request. If X is set to one (XIRQ masked or inhibited), then processing continues with the instruction that immediately follows the STOP instruction, and no XIRQ interrupt service is requested or pending.

Because the oscillator is stopped in STOP mode, a restart delay may be imposed to allow oscillator stabilization upon leaving STOP. If the internal oscillator is being used, this delay is required; however, if a stable external oscillator is being used, the DLY control bit can be used to bypass this start-up delay. The DLY control bit is set by reset and can be optionally cleared during initialization. If the DLY equal to zero option is used to avoid start-up delay on recovery from STOP, then reset should not be used as the means of recovering from STOP, as this causes DLY to be set again by reset, imposing the restart delay. This same delay also applies to power-on reset, regardless of the state of the DLY control bit, but does not apply to a reset while the clocks are running.

SECTION 6 PARALLEL INPUT/OUTPUT

The MC68HC11F1 MCU has up to 54 input/output lines, depending on the operating mode. The data bus of this microcontroller is nonmultiplexed. I/O lines are organized into seven parallel ports. Ports with bidirectional pins have an associated data direction control register. This register (DDRx) contains a data direction control bit for each bidirectional port line. The following table is a summary of the configuration and features of each port.

Port pin function is mode dependent. Do not confuse pin function with the electrical state of the pin at reset. Port pins are either driven to a specified logic level or are configured as high impedance inputs. I/O pins configured as high-impedance inputs have port data that is indeterminate. The contents of the corresponding latches are dependent upon the electrical state of the pins during reset. In port descriptions, an “I” indicates this condition. Port pins that are driven to a known logic level during reset are shown with a value of either one or zero. Some control bits are unaffected by reset. Reset states for these bits are indicated with a “U”.

6.1 Port A

Port A has eight bidirectional I/O pins and shares functions with the timer system.

Table 6-1 I/O Port Configuration

Port Input Pins Output Pins Bidirectional Pins Shared Functions

Port A — — 8 Timer

Port B — 8 — High-Order Address

Port C — — 8 Data Bus

Port D — — 6 SCI and SPI

Port E 8 — — A/D Converter

Port F — 8 — Low-Order Address

Port G — — 8 Chip Select Outputs

PORTA — Port A Data $1000

Bit 7 6 5 4 3 2 1 Bit 0

PA7 PA6 PA5 PA4 PA3 PA2 PA1 PA0

RESET: I I I I I I I I

Alt. Pin

Func.: PAI OC2 OC3 OC4 IC4/OC5 IC1 IC2 IC3

And/or: OC1 OC1 OC1 OC1 OC1 — — —

DDA[7:0] — Data Direction for Port A

0 = Input

1 = Output

NOTE

To enable PA3 as fourth input capture, set the I4/O5 bit in the PACTL register. Otherwise, PA3 is configured as a fifth output compare out of reset, with bit I4/O5 being cleared. If the DDA3 bit is set (configuring PA3 as an output), and IC4 is enabled, writes to PA3 cause edges on the pin to result in input captures. Writing to TI4/O5 has no effect when the TI4/O5 register is acting as IC4. PA7 drives the pulse accumulator input but also can be configured for general-purpose I/O, or output compare. Note that even when PA7 is configured as an output, the pin still drives the pulse accumulator input.

6.2 Port B

Reset state is mode dependent. In single-chip or bootstrap modes, port B pins are general-purpose outputs. In expanded and test modes, port B pins are high-order address outputs and PORTB is not in the memory map.

6.3 Port C

Reset state is mode dependent. In single-chip and bootstrap modes, port C pins are high-impedance inputs. It is customary to have an external pull-up resistor on lines that are driven by open-drain devices. In expanded or test modes, port C pins are data bus inputs/outputs and PORTC is not in the memory map. The R/W signal is used to control the direction of data transfers.

The CWOM control bit in the OPT2 register disables port C's P-channel output drivers. Because the N-channel driver is not affected by CWOM, setting CWOM causes port C to become an open-drain-type output port suitable for wired-OR operation. In wired-OR mode, (PORTC bits are at logic level zero), pins are actively driven low by the Nchannel driver. When a port C bit is at logic level one, the associated pin is in a high- impedance state, as neither the N-channel nor the P-channel devices are active. It is customary to have an external pull-up resistor on lines that are driven by open-drain devices. Port C can only be configured for wired-OR operation when the MCU is in single- chip or bootstrap modes.

DDRA — Data Direction Register for Port A $1001

Bit 7 6 5 4 3 2 1 Bit 0

DDA7 DDA6 DDA5 DDA4 DDA3 DDA2 DDA1 DDA0

RESET: 0 0 0 0 0 0 0 0

PORTB — Port B Data $1004

Bit 7 6 5 4 3 2 1 Bit 0

PB7 PB6 PB5 PB4 PB3 PB2 PB1 PB0

S. Chip or

Boot: PB7 PB6 PB5 PB4 PB3 PB2 PB1 PB0

RESET: 0 0 0 0 0 0 0 0

Expan. or

Test: ADDR15 ADDR14 ADDR13 ADDR12 ADDR11 ADDR10 ADDR9 ADDR8

DDC[7:0] — Data Direction for Port C

0 = Input

1 = Output

6.4 Port D

In all modes, port D bits [5:0] can be used either for general-purpose I/O, or with the SCI and SPI subsystems. During reset, port D pins are configured as high impedance inputs (DDRD bits cleared). The DWOM control bit in the SPCR register disables port D’s P-channel output drivers. Because the N-channel driver is not affected by DWOM, setting DWOM causes port D to become an open-drain-type output port suitable for wired-OR operation. In wired-OR mode, (PORTD bits are at logic level zero), pins are actively driven low by the Nchannel driver. When a port D bit is at logic level one, the associated pin is in a high impedance state, as neither the N-channel nor the P-channel devices are active. It is customary to have an external pull-up resistor on lines that are driven by open-drain devices. Port D can be configured for wired-OR operation in any operating mode.

PORTC — Port C Data $1006

Bit 7 6 5 4 3 2 1 Bit 0

PC7 PC6 PC5 PC4 PC3 PC2 PC1 PC0

S. Chip or

Boot: PC7 PC6 PC5 PC4 PC3 PC2 PC1 PC0

RESET: I I I I I I I I

Expan. or

Test: DATA7 DATA6 DATA5 DATA4 DATA3 DATA2 DATA1 DATA0

DDRC — Data Direction Register for Port C $1007

Bit 7 6 5 4 3 2 1 Bit 0

DDC7 DDC6 DDC5 DDC4 DDC3 DDC2 DDC1 DDC0

RESET: 0 0 0 0 0 0 0 0

PORTD — Port D Data $1008

Bit 7 6 5 4 3 2 1 Bit 0

— — PD5 PD4 PD3 PD2 PD1 PD0

RESET: 0 0 I I I I I I

Alt. Pin

Func.: — — SS SCK MOSI MISO TxD RxD

Bits [7:6] — Not implemented

Always read zero

DDD[5:0] — Data Direction for Port D

0 = Input

1 = Output

NOTE

When the SPI system is in slave mode, DDD5 has no meaning nor effect. When the SPI system is in master mode, DDD5 determines whether bit 5 of PORTD is an error detect input (DDD5 = 0) or a general-purpose output (DDD5 = 1). If the SPI system is enabled and expects any of bits [4:2] to be an input, that bit will be an input regardless of the state of the associated DDR bit. If any of bits [4:2] are expected to be outputs that bit will be an output only if the associated DDR bit is set.

6.5 Port E

Port E has eight general-purpose input pins and shares functions with the A/D converter system. When some port E pins are being used for general-purpose input and others are being used as A/D inputs, PORTE should not be read during the sample portion of an A/D conversion.

6.6 Port F

Reset state is mode dependent. In single-chip or bootstrap modes, port F pins are general-purpose outputs. In expanded and test modes, port F pins are low order address outputs and PORTF is not in the memory map.

DDRD — Data Direction Register for Port D $1009

Bit 7 6 5 4 3 2 1 Bit 0

— — DDD5 DDD4 DDD3 DDD2 DDD1 DDD0

RESET: 0 0 0 0 0 0 0 0

PORTE — Port E Data $100A

Bit 7 6 5 4 3 2 1 Bit 0

PE7 PE6 PE5 PE4 PE3 PE2 PE1 PE0

RESET: I I I I I I I I

Alt. Pin

Func.: AN7 AN6 AN5 AN4 AN3 AN2 AN1 AN0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

PARALLEL INPUT/OUTPUT

TECHNICAL DATA 6-5

6.7 Port G

Port G pins reset to high-impedance inputs except in expanded modes where reset causes PG7 to become the CSPROG output. Alternate functions for port G bits [7:4] are chip select outputs. All port G bits are bidirectional and have corresponding data direction bits.

The GWOM control bit in the OPT2 register disables port G's P-channel output drivers. Because the N-channel driver is not affected by GWOM, setting GWOM causes port G to become an open-drain-type output port suitable for wired-OR operation. In wired-OR mode, (PORTG bits are at logic level zero), pins are actively driven low by the Nchannel driver. When a port G bit is at logic level one, the associated pin is in a high impedance state, as neither the N-channel nor the P-channel devices are active. It is customary to have an external pull-up resistor on lines that are driven by open-drain devices. Port G can be configured for wired-OR operation in any operating mode.

DDG[7:0] — Data Direction for Port G

0 = Input

1 = Output

6.8 System Configuration Options 2

The system configuration options 2 register controls several configuration parameters.

Bit 6, CWOM, is the only bit in this register that directly affects parallel I/O.

PORTF — Port F Data $1005

Bit 7 6 5 4 3 2 1 Bit 0

PF7 PF6 PF5 PF4 PF3 PF2 PF1 PF0

S. Chip

or Boot: PF7 PF6 PF5 PF4 PF3 PF2 PF1 PF0

RESET: 0 0 0 0 0 0 0 0

Expan.

or Test: ADDR7 ADDR6 ADDR5 ADDR4 ADDR3 ADDR2 ADDR1 ADDR0

PORTG — Port G Data $1002

Bit 7 6 5 4 3 2 1 Bit 0

PG7 PG6 PG5 PG4 PG3 PG2 PG1 PG0

RESET: I I I I I I I I

Alt. Pin

Func.: CSPROG CSGEN CSIO1 CSIO2 — — — —

DDRG — Data Direction Register for Port G $1003

Bit 7 6 5 4 3 2 1 Bit 0

DDG7 DDG6 DDG5 DDG4 DDG3 DDG2 DDG1 DDG0

RESET: 0 0 0 0 0 0 0 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

PARALLEL INPUT/OUTPUT MC68HC11F1

6-6 TECHNICAL DATA

GWOM — Port G Wired-OR Mode

0 = Port G operates normally

1 = Port G outputs are open drain

CWOM — Port C Wired-OR Mode

0 = Port C operates normally

1 = Port C outputs are open drain

CLK4X — 4XOUT Clock Enable

Refer to SECTION 2 PIN DESCRIPTIONS.

Bits [4:0] — Not implemented

Always read zero

OPT2 — System Configuration Options 2 $1038

Bit 7 6 5 4 3 2 1 Bit 0

GWOM CWOM CLK4X — — — — —

RESET: 0 0 1 0 0 0 0 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

SERIAL COMMUNICATIONS INTERFACE

TECHNICAL DATA 7-1

SECTION 7 SERIAL COMMUNICATIONS INTERFACE

The serial communications interface (SCI) is a universal asynchronous receiver transmitter

(UART), one of two independent serial I/O subsystems in the MC68HC11F1

MCU. It has a standard nonreturn to zero (NRZ) format (one start bit, eight or nine data

bits, and one stop bit). Several baud rates are available. The SCI transmitter and receiver

are independent, but use the same data format and bit rate.

7.1 Data Format

The serial data format requires the following conditions:

1. An idle-line in the high state before transmission or reception of a message.

2. A start bit, logic zero, transmitted or received, that indicates the start of each

character.

3. Data that is transmitted and received least significant bit (LSB) first.

4. A stop bit, logic one, used to indicate the end of a frame. (A frame consists of a

start bit, a character of eight or nine data bits, and a stop bit.)

5. A break (defined as the transmission or reception of a logic zero for some multiple

number of frames).

Selection of the word length is controlled by the M bit of SCI control register SCCR1.

7.2 Transmit Operation

The SCI transmitter includes a parallel transmit data register (SCDR) and a serial shift

register. The contents of the serial shift register can only be written through the SCDR.

This double buffered operation allows a character to be shifted out serially while another

character is waiting in the SCDR to be transferred into the serial shift register.

The output of the serial shift register is applied to TxD as long as transmission is in

progress or the transmit enable (TE) bit of serial communication control register 2

(SCCR2) is set. The block diagram, Figure 7-1, shows the transmit serial shift register

and the buffer logic at the top of the figure.

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

SERIAL COMMUNICATIONS INTERFACE MC68HC11F1

7-2 TECHNICAL DATA

Figure 7-1 SCI Transmitter Block Diagram

7.3 Receive Operation

During receive operations, the transmit sequence is reversed. The serial shift register

receives data and transfers it to a parallel receive data register (SCDR) as a complete

word. This double buffered operation allows a character to be shifted in serially while

another character is already in the SCDR. An advanced data recovery scheme distinguishes

valid data from noise in the serial data stream. The data input is selectively

sampled to detect receive data, and a majority voting circuit determines the value and

integrity of each bit.

FE

NF

OR

IDLE

RDRF

TC

TDRE

SCSR INTERRUPT STATUS

SBK

RWU

RE

TE

ILIE

RIE

TCIE

TIE

SCCR2 SCI CONTROL 2

TRANSMITTER

CONTROL LOGIC

TCIE

TC

TIE

TDRE

SCI Rx

REQUESTS

SCI INTERRUPT

REQUEST

INTERNAL

DATA BUS

PIN BUFFER

H (8) 7 6 5 4 3 2 1 0 L AND CONTROL

10 (11) - BIT Tx SHIFT REGISTER

PD1/

TxD

SCDR Tx BUFFER

TRANSFER Tx BUFFER

SHIFT ENABLE

JAM ENABLE

PREAMBLE—JAM 1's

BREAK—JAM 0's

(WRITE-ONLY)

FORCE PIN

DIRECTION (OUT)

SIZE 8/9

WAKE

M

T8

R8

SCCR1 SCI CONTROL 1

TRANSMITTER

BAUD RATE

CLOCK

DDD1

8

8

8

8

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

SERIAL COMMUNICATIONS INTERFACE

TECHNICAL DATA 7-3

Figure 7-2 SCI Receiver Block Diagram

FE

NF

OR

IDLE

RDRF

TC

TDRE

SCSR1 SCI STATUS 1

SBK

RWU

RE

TE

ILIE

RIE

TCIE

TIE

SCCR2 SCI CONTROL 2

WAKE

M

T8

R8

WAKE-UP

LOGIC

RIE

OR

ILIE

IDLE

SCI Tx

REQUESTS

SCI INTERRUPT

REQUEST

INTERNAL

DATA BUS

PD0/

RxD

SCDR Rx BUFFER

STOP

(8) 7 6 5 4 3 2 1 0

10 (11) - BIT

Rx SHIFT REGISTER

(READ-ONLY)

SCCR1 SCI CONTROL 1

RIE

RDRF

START

MSB ALL ONES

DATA

RECOVERY

¸16

RWU

RE

M

DISABLE

DRIVER

RECEIVER

BAUD RATE

CLOCK

8

8

8

PIN BUFFER

AND CONTROL

DDD0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

SERIAL COMMUNICATIONS INTERFACE MC68HC11F1

7-4 TECHNICAL DATA

7.4 Wakeup Feature

The wakeup feature reduces SCI service overhead in multiple receiver systems. Software

for each receiver evaluates the first character of each message. The receiver is

placed in wakeup mode by writing a one to the RWU bit in the SCCR2 register. While

RWU is one, all of the receiver-related status flags (RDRF, IDLE, OR, NF, and FE) are

inhibited (cannot become set). Although RWU can be cleared by a software write to

SCCR2, to do so would be unusual. Normally RWU is set by software and is cleared

automatically with hardware. Whenever a new message begins, logic alerts the sleeping

receivers to wake up and evaluate the initial character of the new message.

Two methods of wakeup are available: idle-line wakeup and address-mark wakeup.

During idle-line wakeup, a sleeping receiver awakens as soon as the RxD line becomes

idle. In the address-mark wakeup, logic one in the most significant bit (MSB) of

a character wakes up all sleeping receivers.

7.4.1 Idle-Line Wakeup

To use the receiver wakeup method, establish a software addressing scheme to allow

the transmitting devices to direct a message to individual receivers or to groups of receivers.

This addressing scheme can take any form as long as all transmitting and receiving

devices are programmed to understand the same scheme. Because the

addressing information is usually the first frame(s) in a message, receivers that are not

part of the current task do not become burdened with the entire set of addressing

frames. All receivers are awake (RWU = 0) when each message begins. As soon as

a receiver determines that the message is not intended for it, software sets the RWU

bit (RWU = 1), which inhibits further flag setting until the RxD line goes idle at the end

of the message. As soon as an idle line is detected by receiver logic, hardware automatically

clears the RWU bit so that the first frame of the next message can be received.

This type of receiver wakeup requires a minimum of one idle-line frame time

between messages, and no idle time between frames in a message.

7.4.2 Address-Mark Wakeup

The serial characters in this type of wakeup consist of seven (eight if M = 1) information

bits and an MSB, which indicates an address character (when set to one, or mark).

The first character of each message is an addressing character (MSB = 1). All receivers

in the system evaluate this character to determine if the remainder of the message

is directed toward this particular receiver. As soon as a receiver determines that a

message is not intended for it, the receiver activates the RWU function by using a software

write to set the RWU bit. Because setting RWU inhibits receiver-related flags,

there is no further software overhead for the rest of this message.

When the next message begins, its first character has its MSB set, which automatically

clears the RWU bit and enables normal character reception. The first character whose

MSB is set is also the first character to be received after wakeup because RWU gets

cleared before the stop bit for that frame is serially received. This type of wakeup allows

messages to include gaps of idle time, unlike the idle-line method, but there is a

loss of efficiency because of the extra bit time for each character (address bit) required

for all characters.

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

SERIAL COMMUNICATIONS INTERFACE

TECHNICAL DATA 7-5

7.5 SCI Error Detection

Three error conditions, SCDR overrun, received bit noise, and framing can occur during

generation of SCI system interrupts. Three bits (OR, NF, and FE) in the serial communications

status register (SCSR) indicate if one of these error conditions exists.

The overrun error (OR) bit is set when the next byte is ready to be transferred from the

receive shift register to the SCDR and the SCDR is already full (RDRF bit is set). When

an overrun error occurs, the data that caused the overrun is lost and the data that was

already in SCDR is not disturbed. The OR is cleared when the SCSR is read (with OR

set), followed by a read of the SCDR.

The noise flag (NF) bit is set if there is noise on any of the received bits, including the

start and stop bits. The NF bit is not set until the RDRF flag is set. The NF bit is cleared

when the SCSR is read (with FE equal to one) followed by a read of the SCDR.

When no stop bit is detected in the received data character, the framing error (FE) bit

is set. FE is set at the same time as the RDRF. If the byte received causes both framing

and overrun errors, the processor only recognizes the overrun error. The framing

error flag inhibits further transfer of data into the SCDR until it is cleared. The FE bit is

cleared when the SCSR is read (with FE equal to one) followed by a read of the SCDR.

7.6 SCI Registers

There are five addressable registers associated with the SCI. SCCR1, SCCR2, and

BAUD are control registers. SCDR is the SCI data register and SCSR is the SCI status

register. Refer to the BAUD register description as well as the block diagram for the

baud rate generator.

7.6.1 Serial Communications Data Register

SCDR is a parallel register that performs two functions. It is the receive data register

when it is read, and the transmit data register when it is written. Reads access the receive

data buffer and writes access the transmit data buffer. Receive and transmit are

double buffered.

7.6.2 Serial Communications Control Register 1

The SCCR1 register provides the control bits that determine word length and select

the method used for the wakeup feature.

SCDR — SCI Data Register $102F

Bit 7 6 5 4 3 2 1 Bit 0

R7/T7 R6/T6 R5/T5 R4/T4 R3/T3 R2/T2 R1/T1 R0/T0

RESET: I I I I I I I I

SCCR1 — SCI Control Register 1 $102C

Bit 7 6 5 4 3 2 1 Bit 0

R8 T8 — M WAKE — — —

RESET: I I 0 0 0 0 0 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

SERIAL COMMUNICATIONS INTERFACE MC68HC11F1

7-6 TECHNICAL DATA

R8 — Receive Data Bit 8

If M bit is set, R8 stores the ninth bit in the receive data character.

T8 — Transmit Data Bit 8

If M bit is set, T8 stores the ninth bit in the transmit data character.

M — Mode (Select Character Format)

0 = Start bit, 8 data bits, 1 stop bit

1 = Start bit, 9 data bits, 1 stop bit

WAKE — Wakeup by Address Mark/Idle

0 = Wakeup by IDLE line recognition

1 = Wakeup by address mark (most significant data bit set)

7.6.3 Serial Communications Control Register 2

The SCCR2 register provides the control bits that enable or disable individual SCI

functions.

TIE — Transmit Interrupt Enable

0 = TDRE interrupts disabled

1 = SCI interrupt requested when TDRE status flag is set

TCIE — Transmit Complete Interrupt Enable

0 = TC interrupts disabled

1 = SCI interrupt requested when TC status flag is set

RIE — Receiver Interrupt Enable

0 = RDRF and OR interrupts disabled

1 = SCI interrupt requested when RDRF flag or the OR status flag is set

ILIE — Idle-Line Interrupt Enable

0 = IDLE interrupts disabled

1 = SCI interrupt requested when IDLE status flag is set

TE — Transmitter Enable

When TE goes from zero to one, one unit of idle character time (logic one) is queued

as a preamble.

0 = Transmitter disabled

1 = Transmitter enabled

RE — Receiver Enable

0 = Receiver disabled

1 = Receiver enabled

SCCR2 — SCI Control Register 2 $102D

Bit 7 6 5 4 3 2 1 Bit 0

TIE TCIE RIE ILIE TE RE RWU SBK

RESET: 0 0 0 0 0 0 0 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

SERIAL COMMUNICATIONS INTERFACE

TECHNICAL DATA 7-7

RWU — Receiver Wakeup Control

0 = Normal SCI receiver

1 = Wakeup enabled and receiver interrupts inhibited

SBK — Send Break

At least one character time of break is queued and sent each time SBK is written to

one. As long as the SBK bit is set, break characters are queued and sent. More than

one break may be sent if the transmitter is idle at the time the SBK bit is toggled on

and off, as the baud rate clock edge could occur between writing the one and writing

the zero to SBK.

0 = Break generator off

1 = Break codes generated

7.6.4 Serial Communication Status Register

The SCSR provides inputs to the interrupt logic circuits for generation of the SCI system

interrupt.

TDRE — Transmit Data Register Empty Flag

This flag is set when SCDR is empty. Clear the TDRE flag by reading SCSR and then

writing to SCDR.

0 = SCDR busy

1 = SCDR empty

TC — Transmit Complete Flag

This flag is set when the transmitter is idle (no data, preamble, or break transmission

in progress). Clear the TC flag by reading SCSR and then writing to SCDR.

0 = Transmitter busy

1 = Transmitter idle

RDRF — Receive Data Register Full Flag

This flag is set if a received character is ready to be read from SCDR. Clear the RDRF

flag by reading SCSR and then reading SCDR.

0 = SCDR empty

1 = SCDR full

IDLE — Idle Line Detected Flag

This flag is set if the RxD line is idle. Once cleared, IDLE is not set again until the RxD

line has been active and becomes idle again. The IDLE flag is inhibited when RWU =

1. Clear IDLE by reading SCSR and then reading SCDR.

0 = RxD line is active

1 = RxD line is idle

SCSR — SCI Status Register $102E

Bit 7 6 5 4 3 2 1 Bit 0

TDRE TC RDRF IDLE OR NF FE —

RESET: 1 1 0 0 0 0 0 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

SERIAL COMMUNICATIONS INTERFACE MC68HC11F1

7-8 TECHNICAL DATA

OR — Overrun Error Flag

OR is set if a new character is received before a previously received character is read

from SCDR. Clear the OR flag by reading SCSR and then reading SCDR.

0 = No overrun

1 = Overrun detected

NF — Noise Error Flag

NF is set if majority sample logic detects anything other than a unanimous decision.

Clear NF by reading SCSR and then reading SCDR.

0 = Unanimous decision

1 = Noise detected

FE — Framing Error

FE is set when a zero is detected where a stop bit was expected. Clear the FE flag by

reading SCSR and then reading SCDR.

0 = Stop bit detected

1 = Zero detected

Bit 0 — Not implemented

Always reads zero

7.6.5 Baud Rate Register

Use this register to select different baud rates for the SCI system. The SCP[1:0] bits

select the prescaler rate for the SCR[2:0] bits. Together, these five bits provide multiple

baud rate combinations for a given crystal frequency. Normally, this register is written

once during initialization. The prescaler is set to its fastest rate by default out of

reset, and can be changed at any time. Refer to Table 7-1 and Table 7-2 for normal

baud rate selections.

TCLR — Clear Baud Rate Counters (Test)

SCP[1:0] — SCI Baud Rate Prescaler Selects

Refer to the SCI baud rate generator block diagram.

BAUD — Baud Rate $102B

Bit 7 6 5 4 3 2 1 Bit 0

TCLR — SCP1 SCP0 RCKB SCR2 SCR1 SCR0

RESET: 0 0 0 0 0 U U U

Table 7-1 Baud Rate Prescaler Selection

Prescaler Divide Internal Crystal Frequency (MHz)

SCP1 SCP0 Clock By 4.0 4.9152 8.0 12.0 16.0 20.0

0 0 1 62500 76800 125000 187500 25000 312500

0 1 3 20833 25600 41667 62500 83332 104165

1 0 4 15625 19200 31250 46875 62500 78125

1 1 13 4800 5907 9600 14423 19200 24000

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

SERIAL COMMUNICATIONS INTERFACE

TECHNICAL DATA 7-9

RCKB — SCI Baud Rate Clock Check (Test)

SCR[2:0] — SCI Baud Rate Selects

Selects receiver and transmitter bit rate based on output from baud rate prescaler

stage. Refer to the SCI baud rate generator block diagram.

The prescaler bits, SCP[2:0], determine the highest baud rate, and the SCR[2:0] bits

select an additional binary submultiple (³1, ³2, ³4, through ³128) of this highest baud

rate. The result of these two dividers in series is the 16X receiver baud rate clock. The

SCR[2:0] bits are not affected by reset and can be changed at any time, although they

should not be changed when any SCI transfer is in progress.

Figure 7-3 and Figure 7-4 illustrate the SCI baud rate timing chain. The prescaler select

bits determine the highest baud rate. The rate select bits determine additional divide

by two stages to arrive at the receiver timing (RT) clock rate. The baud rate clock

is the result of dividing the RT clock by 16.

Table 7-2 Baud Rate Selection

Divide

Prescaler

Highest Baud Rate

(Prescaler Output from Previous Table)

SCR[2:0] By 4800 19200 76800 312500

0 0 0 1 4800 19200 76800 312500

0 0 1 2 2400 9600 38400 156250

0 1 0 4 1200 4800 19200 78125

0 1 1 8 600 2400 9600 39063

1 0 0 16 300 1200 4800 19531

1 0 1 32 150 600 2400 9766

1 1 0 64 75 300 1200 4883

1 1 1 128 — 150 600 2441

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

SERIAL COMMUNICATIONS INTERFACE MC68HC11F1

7-10 TECHNICAL DATA

Figure 7-3 SCI Baud Rate Generator Block Diagram

7.7 Status Flags and Interrupts

The SCI transmitter has two status flags. These status flags can be read by software

(polled) to tell when the corresponding condition exists. Alternatively, a local interrupt

enable bit can be set to enable each of these status conditions to generate interrupt

requests when the corresponding condition is present. Status flags are automatically

set by hardware logic conditions, but must be cleared by software, which provides an

interlock mechanism that enables logic to know when software has noticed the status

indication. The software clearing sequence for these flags is automatic — functions

that are normally performed in response to the status flags also satisfy the conditions

of the clearing sequence.

OSCILLATOR

AND

CLOCK GENERATOR

(¸4)

XTAL

EXTAL

E

AS

SCP1:SCP0

SCI

RECEIVE

BAUD RATE

(16X)

SCI

TRANSMIT

BAUD RATE

(1X)

SCR2:SCR1:SCR0

¸2

¸2

¸2

¸2

¸2

¸2

0:0:1

0:1:0

0:1:1

1:0:0

1:0:1

1:1:0

1:1:1

0:0:0

¸2

¸16

¸3 ¸4 ¸13

0:0 0:1 1:0 1:1

INTERNAL

BUS

CLOCK

(PH2)

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

SERIAL COMMUNICATIONS INTERFACE

TECHNICAL DATA 7-11

TDRE and TC flags are normally set when the transmitter is first enabled (TE set to

one). The TDRE flag indicates there is room in the transmit queue to store another

data character in the TDR. The TIE bit is the local interrupt mask for TDRE. When TIE

is zero, TDRE must be polled. When TIE and TDRE are one, an interrupt is requested.

The TC flag indicates the transmitter has completed the queue. The TCIE bit is the local

interrupt mask for TC. When TCIE is zero, TC must be polled; when TCIE is one

and TC is one, an interrupt is requested.

Writing a zero to TE requests that the transmitter stop when it can. The transmitter

completes any transmission in progress before actually shutting down. Only an MCU

reset can cause the transmitter to stop and shut down immediately. If TE is written to

zero when the transmitter is already idle, the pin reverts to its general-purpose I/O

function (synchronized to the bit-rate clock). If anything is being transmitted when TE

is written to zero, that character is completed before the pin reverts to general-purpose

I/O, but any other characters waiting in the transmit queue are lost. The TC and TDRE

flags are set at the completion of this last character, even though TE has been disabled.

7.7.1 Receiver Flags

The SCI receiver has five status flags, three of which can generate interrupt requests.

The status flags are set by the SCI logic in response to specific conditions in the receiver.

These flags can be read (polled) at any time by software. Refer to Figure 7–4,

which shows SCI interrupt arbitration.

When an overrun takes place, the new character is lost, and the character that was in

its way in the parallel RDR is undisturbed. RDRF is set when a character has been

received and transferred into the parallel RDR. The OR flag is set instead of RDRF if

overrun occurs. A new character is ready to be transferred into RDR before a previous

character is read from RDR.

The NF and FE flags provide additional information about the character in the RDR,

but do not generate interrupt requests.

The last receiver status flag and interrupt source come from the IDLE flag. The RxD

line is idle if it has constantly been at logic one for a full character time. The IDLE flag

is set only after the RxD line has been busy and becomes idle, which prevents repeated

interrupts for the whole time RxD remains idle.

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

SERIAL COMMUNICATIONS INTERFACE MC68HC11F1

7-12 TECHNICAL DATA

Figure 7-4 Interrupt Source Resolution Within SCI

YES

NO

RDRF = 1

?

NO – VALID SCI

REQUEST

BEGIN

YES

NO

YES

NO

OR = 1

?

RIE = 1

?

YES

NO

RE = 1

?

YES

NO

YES

NO

TDRE = 1

?

TIE = 1

?

YES

NO

TE = 1

?

YES

NO

YES

NO

TC = 1

?

TCIE = 1

?

YES

NO

YES

NO

IDLE = 1

?

ILIE = 1

?

YES

NO

RE = 1

?

YES – VALID SCI

REQUEST

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

SERIAL PERIPHERAL INTERFACE

TECHNICAL DATA 8-1

SECTION 8 SERIAL PERIPHERAL INTERFACE

The serial peripheral interface (SPI), an independent serial communications subsystem,

allows the MCU to communicate synchronously with peripheral devices, such

as transistor-transistor logic (TTL) shift registers, liquid crystal display (LCD) drivers,

analog-to-digital converter subsystems, and other microprocessors. The SPI is also

capable of inter-processor communication in a multiple master system. The SPI system

can be configured as either a master or a slave device. When configured as a

master, data transfer rates can be as high as one-half the E-clock rate (2.5 Mbits per

second for a 5-MHz bus frequency). When configured as a slave, data transfers can

be as fast as the E-clock rate (5 Mbits per second for a 5-MHz bus frequency).

8.1 Functional Description

The central element in the SPI system is the block containing the shift register and the

read data buffer. The system is single buffered in the transmit direction and double

buffered in the receive direction. This means that new data for transmission cannot be

written to the shifter until the previous transfer is complete; however, received data is

transferred into a parallel read data buffer so the shifter is free to accept a second serial

character. As long as the first character is read out of the read data buffer before

the next serial character is ready to be transferred, no overrun condition occurs. A single

MCU register address is used for reading data from the read data buffer and for

writing data to the shifter.

The SPI status block represents the SPI status flags (transfer complete, write collision,

and mode fault) located in the SPI status register (SPSR). The SPI control block represents

those functions that control the SPI system through the serial peripheral control

register (SPCR).

Refer to Figure 8-1, which shows the SPI block diagram.

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

SERIAL PERIPHERAL INTERFACE MC68HC11F1

8-2 TECHNICAL DATA

Figure 8-1 SPI Block Diagram

8.2 SPI Transfer Formats

During an SPI transfer, data is simultaneously transmitted and received. A serial clock

line synchronizes shifting and sampling of the information on the two serial data lines.

A slave select line allows individual selection of a slave SPI device; slave devices that

are not selected do not interfere with SPI bus activities. On a master SPI device, the

select line can optionally be used to indicate a multiple master bus contention. Refer

to Figure 8-2.

INTERNAL

MCU CLOCK

DIVIDER

¸2 ¸4 ¸16 ¸32

SELECT

SPI CLOCK (MASTER)

8-BIT SHIFT REGISTER

READ DATA BUFFER

CLOCK

CLOCK

LOGIC

S

M

M

S

PIN

CONTROL

LOGIC

MISO/

PD2

MOSI/

PD3

SCK/

PD4

SS/

PD5

M

S

MSTR

SPE

DWOM

SPR1

SPR0

MSTR

SPE

SPIE

SPIE

SPE

DWOM

MSTR

CPHA

CPOL

SPR1

SPR0

SPCR SPI CONTROL REGISTER

SPIF

WCOL

MODF

SPSR SPI STATUS REGISTER

SPI CONTROL

INTERNAL

DATA BUS

SPI INTERRUPT

REQUEST

MSB LSB

8

8

8

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

SERIAL PERIPHERAL INTERFACE

TECHNICAL DATA 8-3

Figure 8-2 SPI Transfer Format

8.2.1 Clock Phase and Polarity Controls

Software can select one of four combinations of serial clock phase and polarity using

two bits in the SPI control register (SPCR). The clock polarity is specified by the CPOL

control bit, which selects an active high or active low clock, and has no significant effect

on the transfer format. The clock phase (CPHA) control bit selects one of two different

transfer formats. The clock phase and polarity should be identical for the master

SPI device and the communicating slave device. In some cases, the phase and polarity

are changed between transfers to allow a master device to communicate with peripheral

slaves having different requirements.

When CPHA equals zero, the SS line must be negated and reasserted between each

successive serial byte. Also, if the slave writes data to the SPI data register (SPDR)

while SS is low, a write collision error results.

When CPHA equals one, the SS line can remain low between successive transfers.

8.3 SPI Signals

The following paragraphs contain descriptions of the four SPI signals: master in slave

out (MISO), master out slave in (MOSI), serial clock (SCK), and slave select (SS).

Any SPI output line must have its corresponding data direction bit in DDRD register

set. If the DDR bit is clear, that line is disconnected from the SPI logic and becomes a

general-purpose input. All SPI input lines are forced to act as inputs regardless of the

state of the corresponding DDR bits in DDRD register.

1 2 3 4 5 6 7 8

SCK (CPOL = 1)

SCK (CPOL = 0)

SCK CYCLE #

SS (TO SLAVE)

MSB 6 5 4 3 2 1 LSB

MSB 6 5 4 3 2 1 LSB

SAMPLE INPUT

DATA OUT

SAMPLE INPUT

(CPHA = 1) DATA OUT

(CPHA = 0)

SLAVE CPHA = 1 TRANSFER IN PROGRESS

MASTER TRANSFER IN PROGRESS

SLAVE CPHA = 0 TRANSFER IN PROGRESS

3

2

1 5

SS ASSERTED 4

MASTER WRITES

FIRST SCK EDGE

SPIF SET

SS NEGATED

1

2

3

4

5

TO SPDR

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

SERIAL PERIPHERAL INTERFACE MC68HC11F1

8-4 TECHNICAL DATA

8.3.1 Master In Slave Out

MISO is one of two unidirectional serial data signals. It is an input to a master device and an output from a slave device. The MISO line of a slave device is placed in the high-impedance state if the slave device is not selected.

8.3.2 Master Out Slave In

The MOSI line is the second of the two unidirectional serial data signals. It is an output from a master device and an input to a slave device. The master device places data on the MOSI line a half-cycle before the clock edge that the slave device uses to latch the data.

8.3.3 Serial Clock

SCK, an input to a slave device, is generated by the master device and synchronizes data movement in and out of the device through the MOSI and MISO lines. Master and slave devices are capable of exchanging a byte of information during a sequence of eight clock cycles.

There are four possible timing relationships that can be chosen by using control bits CPOL and CPHA in the serial peripheral control register (SPCR). Both master and slave devices must operate with the same timing. The SPI clock rate select bits, SPR[1:0], in the SPCR of the master device, select the clock rate. In a slave device, SPR[1:0] have no effect on the operation of the SPI.

8.3.4 Slave Select

The slave select (SS) input of a slave device must be externally asserted before a master device can exchange data with the slave device. SS must be low before data transactions and must stay low for the duration of the transaction.

The SS line of the master must be held high. If it goes low, a mode fault error flag (MODF) is set in the serial peripheral status register (SPSR). To disable the mode fault circuit, write a one in bit 5 of the port D data direction register. This sets the SS pin to act as a general-purpose output rather than the dedicated input to the slave select circuit, thus inhibiting the mode fault flag. The other three lines are dedicated to the SPI whenever the serial peripheral interface is on.

The state of the master and slave CPHA bits affects the operation of SS. CPHA settings should be identical for master and slave. When CPHA = 0, the shift clock is the OR of SS with SCK. In this clock phase mode, SS must go high between successive characters in an SPI message. When CPHA = 1, SS can be left low between successive SPI characters. In cases where there is only one SPI slave MCU, its SS line can be tied to Vss as long as only CPHA = 1 clock mode is used.

8.4 SPI System Errors

Two system errors can be detected by the SPI system. The first type of error arises in a multiple-master system when more than one SPI device simultaneously tries to be a master. This error is called a mode fault. The second type of error, write collision, indicates that an attempt was made to write data to the SPDR while a transfer was in progress.

When the SPI system is configured as a master and the SS input line goes to active low, a mode fault error has occurred — usually because two devices have attempted to act as master at the same time. In cases where more than one device is concurrently configured as a master, there is a chance of contention between two pin drivers. For push-pull CMOS drivers, this contention can cause permanent damage. The mode fault mechanism attempts to protect the device by disabling the drivers. The MSTR control bit in the SPCR and all four DDRD control bits associated with the SPI are cleared and an interrupt is generated subject to masking by the SPIE control bit and the I bit in the CCR.

Other precautions may need to be taken to prevent driver damage. If two devices are made masters at the same time, mode fault does not help protect either one unless one of them selects the other as slave. The amount of damage possible depends on the length of time both devices attempt to act as master.

A write collision error occurs if the SPDR is written while a transfer is in progress. Because the SPDR is not double buffered in the transmit direction, writes to SPDR cause data to be written directly into the SPI shift register. Because this write corrupts any transfer in progress, a write collision error is generated. The transfer continues undisturbed, and the write data that caused the error is not written to the shifter.

A write collision is normally a slave error because a slave has no control over when a master initiates a transfer. A master knows when a transfer is in progress, so there is no reason for a master to generate a write-collision error, although the SPI logic can detect write collisions in both master and slave devices.

The SPI configuration determines the characteristics of a transfer in progress. For a master, a transfer begins when data is written to SPDR and ends when SPIF is set.

For a slave with CPHA equal to zero, a transfer starts when SS goes low and ends when SS returns high. In this case, SPIF is set at the middle of the eighth SCK cycle when data is transferred from the shifter to the parallel data register, but the transfer is still in progress until SS goes high. For a slave with CPHA equal to one, transfer begins when the SCK line goes to its active level, which is the edge at the beginning of the first SCK cycle. The transfer ends in a slave in which CPHA equals one when SPIF is set.

8.5 SPI Registers

The three SPI registers, SPCR, SPSR, and SPDR, provide control, status, and data storage functions. Refer to the following information for a description of how these registers are organized.

8.5.1 Serial Peripheral Control

SPCR — Serial Peripheral Control Register $1028

Bit 7 6 5 4 3 2 1 Bit 0

SPIE SPE DWOM MSTR CPOL CPHA SPR1 SPR0

RESET: 0 0 0 0 0 1 U U

SPIE — Serial Peripheral Interrupt Enable

Set the SPE bit to one to request a hardware interrupt sequence each time the SPIF or MODF status flag is set. SPI interrupts are inhibited if this bit is clear or if the I bit in the condition code register is one.

0 = SPI system interrupts disabled

1 = SPI system interrupts enabled

SPE — Serial Peripheral System Enable

When the SPE bit is set, the port D bit 2, 3, 4, and 5 pins are dedicated to the SPI function.

If the SPI is in the master mode and DDRD bit 5 is set, then the port D bit 5 pin becomes a general-purpose output instead of the SS input.

0 = SPI system disabled

1 = SPI system enabled

DWOM — Port D Wired-OR Mode

DWOM affects all port D pins.

0 = Normal CMOS outputs

1 = Open-drain outputs

MSTR — Master Mode Select

0 = Slave mode

1 = Master mode

CPOL — Clock Polarity

When the clock polarity bit is cleared and data is not being transferred, the SCK pin of

the master device has a steady state low value. When CPOL is set, SCK idles high.

Refer to Figure 8-2 and 8.2.1 Clock Phase and Polarity Controls.

CPHA — Clock Phase

The clock phase bit, in conjunction with the CPOL bit, controls the clock-data relationship

between master and slave. The CPHA bit selects one of two different clocking

protocols. Refer to Figure 8-2 and 8.2.1 Clock Phase and Polarity Controls.

SPR[1:0] — SPI Clock Rate Selects

These two bits select the SPI clock (SCK) rate when the device is configured as master.

When the device is configured as slave, these bits have no effect. Refer to Table

8-1.

Table 8-1 SPI Clock Rates

SPR[1:0]

E Clock

Divide By

Frequency at

E = 2 MHz

Frequency at

E = 3 MHz

Frequency at

E = 4 MHz

Frequency at

E = 5 MHz

0 0 2 1.0 MHz 1.5 MHz 2.0 MHz 2.5 MHz

0 1 4 500 kHz 750 kHz 1.0 MHz 625 kHz

1 0 16 125 kHz 187.5 kHz 250 kHz 156.25 kHz

1 1 32 62.5 kHz 93.7 kHz 125 kHz 78.125 kHz

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

SERIAL PERIPHERAL INTERFACE

TECHNICAL DATA 8-7

8.5.2 Serial Peripheral Status

SPIF — SPI Interrupt Complete Flag

SPIF is set upon completion of data transfer between the processor and the external

device. If SPIF goes high, and if SPIE is set, a serial peripheral interrupt is generated.

To clear the SPIF bit, read the SPSR then access the SPDR. Unless SPSR is read

(with SPIF set) first, attempts to write SPDR are inhibited.

WCOL — Write Collision

Clearing the WCOL bit is accomplished by reading the SPSR followed by an access of

SPDR. Refer to 8.3.4 Slave Select and 8.4 SPI System Errors.

0 = No write collision

1 = Write collision

Bit 5 — Not implemented

Always reads zero

MODF — Mode Fault

To clear the MODF bit, read the SPSR then write to the SPCR. Refer to 8.3.4 Slave

Select and 8.4 SPI System Errors.

0 = No mode fault

1 = Mode fault

Bits [3:0] — Not implemented

Always read zero

8.5.3 Serial Peripheral Data Register

The SPDR is used when transmitting or receiving data on the serial bus. Only a write

to this register initiates transmission or reception of a byte, and this only occurs in the

master device. At the completion of transferring a byte of data, the SPIF status bit is

set in both the master and slave devices.

A read of the SPDR is actually a read of a buffer. To prevent an overrun and the loss

of the byte that caused the overrun, the first SPIF must be cleared by the time a second

transfer of data from the shift register to the read buffer is initiated.

SPI is double buffered in and single buffered out.

SPSR — Serial Peripheral Status Register $1029

Bit 7 6 5 4 3 2 1 Bit 0

SPIF WCOL — MODF — — — —

RESET: 0 0 0 0 0 0 0 0

SPDR — SPI Data Register $102A

Bit 7 6 5 4 3 2 1 Bit 0

Bit 7 6 5 4 3 2 1 Bit 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

SERIAL PERIPHERAL INTERFACE MC68HC11F1

8-8 TECHNICAL DATA

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TIMING SYSTEM

TECHNICAL DATA 9-1

SECTION 9 TIMING SYSTEM

The M68HC11 timing system is composed of five clock divider chains. The main clock

divider chain includes a 16-bit free-running counter, which is driven by a programmable

prescaler. The main timer's programmable prescaler provides one of the four

clocking rates to drive the 16-bit counter. Two prescaler control bits select the prescale

rate.

The prescaler output divides the system clock by 1, 4, 8, or 16. Taps off of this main

clocking chain drive circuitry that generates the slower clocks used by the pulse accumulator,

the real-time interrupt (RTI), and the computer operating properly (COP)

watchdog subsystems, also described in this section. Refer to Figure 9-1.

All main timer system activities are referenced to this free-running counter. The

counter begins incrementing from $0000 as the MCU comes out of reset, and continues

to the maximum count, $FFFF. At the maximum count, the counter rolls over to

$0000, sets an overflow flag, and continues to increment. As long as the MCU is running

in a normal operating mode, there is no way to reset, change, or interrupt the

counting. The capture/compare subsystem features three input capture channels, four

output compare channels, and one channel that can be selected to perform either input

capture or output compare. Each of the three input capture functions has its own

16-bit input capture register (time capture latch) and each of the output compare functions

has its own 16-bit compare register. All timer functions, including the timer overflow

and RTI have their own interrupt controls and separate interrupt vectors.

The pulse accumulator contains an 8-bit counter and edge select logic. The pulse accumulator

can operate in either event counting mode or gated time accumulation

mode. During event counting mode, the pulse accumulator's 8-bit counter increments

when a specified edge is detected on an input signal. During gated time accumulation

mode, an internal clock source increments the 8-bit counter while an input signal has

a predetermined logic level.

The real-time interrupt (RTI) is a programmable periodic interrupt circuit that permits

pacing the execution of software routines by selecting one of four interrupt rates.

The COP watchdog clock input (E ¸ 215) is tapped off of the free-running counter

chain. The COP automatically times out unless it is serviced within a specific time by

a program reset sequence. If the COP is allowed to time out, a reset is generated,

which drives the RESET pin low to reset the MCU and the external system. Refer to

Table 9-1 for crystal related frequencies and periods.

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TIMING SYSTEM MC68HC11F1

9-2 TECHNICAL DATA

Figure 9-1 Timer Clock Divider Chains

R

Q

FF2

S

Q

OSCILLATOR AND

CLOCK GENERATOR

AS

E CLOCK

SPI

SCI RECEIVER CLOCK

SCI TRANSMIT CLOCK

E¸26 PULSE ACCUMULATOR

TCNT

E¸213 REAL-TIME INTERRUPT

¸4

E¸215

R

Q

FORCE

COP

RESET

SYSTEM

RESET

CLEAR COP

TIMER

FF1

(DIVIDE BY FOUR)

INTERNAL BUS CLOCK (PH2)

IC/OC

¸16

CR[1:0]

PRESCALER

(¸1, 4, 16, 64)

PRESCALER

(¸ 2, 4, 16, 32)

SPR[1:0]

PRESCALER

(¸ 1, 3, 4, 13)

SCP[1:0]

PRESCALER

(¸ 1, 2, 4, 8)

RTR[1:0]

PRESCALER

(¸ 1, 2, 4,....128)

SCR[2:0]

PRESCALER

(¸ 1, 4, 8, 16)

PR[1:0]

S

Q

TOF

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TIMING SYSTEM

TECHNICAL DATA 9-3

9.1 Timer Structure

Figure 9-2 shows the capture/compare system block diagram. The port A pin control

block includes logic for timer functions and for general-purpose I/O. For pins PA3,

PA2, PA1, and PA0, this block contains both the edge-detection logic and the control

logic that enables the selection of which edge triggers an input capture. The digital level

on PA[3:0] can be read at any time (read PORTA register), even if the pin is being

used for the input capture function. Pins PA[6:3] are used for either general-purpose

I/O, or as output compare pins. When one of these pins is being used for an output

compare function, it cannot be written directly as if it were a general-purpose output.

Each of the output compare functions (OC[5:2]) is related to one of the port A output

pins. Output compare one (OC1) has extra control logic, allowing it optional control of

any combination of the PA[7:3] pins. The PA7 pin can be used as a general-purpose

I/O pin, as an input to the pulse accumulator, or as an OC1 output pin.

Table 9-1 Timer Summary

XTAL Frequencies

4.0 MHz 8.0 MHz 12.0 MHz 16.0 MHz Other Rates

1.0 MHz 2.0 MHz 3.0 MHz 4.0 MHz (E)

Control Bits 1000 ns 500 ns 333 ns 250 ns (1/E)

PR1, PR0 Main Timer Count Rates

0 0

1 count —

overflow —

1.0 ms

65.536 ms

500 ns

32.768 ms

333 ns

21.845 ms

250 ns

16.384 ms

(1/E)

(216/E)

0 1

1 count —

overflow —

4.0 ms

262.14 ms

2.0 ms

131.07 ms

1.333 ms

87.381 ms

1.0 ms

65.536 ms

(4/E)

(218/E)

1 0

1 count —

overflow —

8.0 ms

524.29 ms

4.0 ms

262.14 ms

2.667 ms

174.76 ms

2.0 ms

131.07 ms

(8/E)

(219/E)

1 1

1 count —

overflow —

16.0 ms

1.049 s

8.0 ms

524.29 ms

5.333 ms

349.52 ms

4.0 ms

262.14 ms

(16/E)

(220/E)

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TIMING SYSTEM MC68HC11F1

9-4 TECHNICAL DATA

Figure 9-2 Capture/Compare Block Diagram

PA3/

OC5/

IC4/

OC1

16-BIT LATCH CLK

TIC1 (HI)

16-BIT COMPARATOR =

IC3F

OC2F

OC3F

I4/O5F

TFLG1

TMSK1

IC1F

IC1I

3

IC2F

IC2I

2

IC3I

1

OC4F

I4/O5I

16-BIT TIMER BUS

16-BIT FREE-RUNNING

COUNTER

TCNT (HI)

TOF

TOI

9

PR1 PR0

PRESCALER–DIVIDE BY

1, 4, 8, OR 16

I4/O5

OC1I

8

FOC1

OC2I

7

FOC2

OC3I

6

FOC3

OC4I

5

FOC4

4

FOC5

STATUS

FLAGS

FORCE

OUTPUT

COMPARE

INTERRUPT

ENABLES

PORT A

OC5

IC4

CFORC

16-BIT TIMER BUS

OC1F BIT-7

BIT-6

BIT-5

BIT-4

BIT-3

BIT-2

BIT-1

BIT-0

PIN

FUNCTIONS

TCNT (LO)

PA2/

IC1

TIC1 (LO)

16-BIT LATCH CLK

TIC2 (HI) TIC2 (LO)

16-BIT LATCH CLK

TIC3 (HI) TIC3 (LO)

TOC1 (HI) TOC1 (LO)

TOC2 (HI) TOC2 (LO)

TOC3 (HI) TOC3 (LO)

TOC4 (HI) TOC4 (LO)

16-BIT LATCH CLK

TI4/O5 (HI) TI4/O5 (LO)

16-BIT COMPARATOR =

16-BIT COMPARATOR =

16-BIT COMPARATOR =

16-BIT COMPARATOR =

MCU

ECLK

INTERRUPT REQUESTS

(FURTHER QUALIFIED

BY I-BIT IN CCR)

PIN

CONTROL

PA7/

OC1/

PAI

PA6/

OC2/

OC1

PA5/

OC3/

OC1

PA4/

OC4/

OC1

PA1/

IC2

PA0/

IC3

TO

PULSE

ACCUMULATOR

TAPS FOR RTI, COP

WATCHDOG AND

PULSE ACCUMULATOR

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TIMING SYSTEM

TECHNICAL DATA 9-5

9.2 Input Capture

The input capture function records the time an external event occurs by latching the

value of the free-running counter when a selected edge is detected at the associated

timer input pin. Software can store latched values and use them to compute the periodicity

and duration of events. For example, by storing the times of successive edges

of an incoming signal, software can determine the period and pulse width of a signal.

To measure period, two successive edges of the same polarity are captured. To measure

pulse width, two alternate polarity edges are captured.

In most cases, input capture edges are asynchronous to the internal timer counter,

which is clocked relative to an internal clock (PH2). These asynchronous capture requests

are synchronized to PH2 so that the latching occurs on the opposite half cycle

of PH2 from when the timer counter is being incremented. This synchronization process

introduces a delay from when the edge occurs to when the counter value is detected.

Because these delays offset each other when the time between two edges is

being measured, the delay can be ignored. When an input capture is being used with

an output compare, there is a similar delay between the actual compare point and

when the output pin changes state.

The control and status bits that implement the input capture functions are contained in

the PACTL, TCTL2, TMSK1, and TFLG1 registers.

To configure port A bit 3 as an input capture, clear the DDA3 bit of the DDRA register.

Note that this bit is cleared out of reset. To enable PA3 as the fourth input capture, set

the I4/O5 bit in the PACTL register. Otherwise, PA3 is configured as a fifth output compare

out of reset, with bit I4/O5 being cleared. If the DDA3 bit is set (configuring PA3

as an output), and IC4 is enabled, then writes to PA3 cause edges on the pin to result

in input captures. Writing to TI4/O5 has no effect when the TI4/O5 register is acting as

IC4.

9.2.1 Timer Control Register 2

Use the control bits of this register to program input capture functions to detect a particular

edge polarity on the corresponding timer input pin. Each of the input capture

functions can be independently configured to detect rising edges only, falling edges

only, any edge (rising or falling), or to disable the input capture function. The input capture

functions operate independently of each other and can capture the same TCNT

value if the input edges are detected within the same timer count cycle.

EDGxB and EDGxA — Input Capture Edge Control

There are four pairs of these bits. Each pair is cleared to zero by reset and must be

encoded to configure the corresponding input capture edge detector circuit. IC4 functions

only if the I4/O5 bit in the PACTL register is set. Refer to Table 9-2 for timer control

configuration.

TCTL2 — Timer Control 2 $1021

Bit 7 6 5 4 3 2 1 Bit 0

EDG4B EDG4A EDG1B EDG1A EDG2B EDG2A EDG3B EDG3A

RESET: 0 0 0 0 0 0 0 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TIMING SYSTEM MC68HC11F1

9-6 TECHNICAL DATA

9.2.2 Timer Input Capture Registers

When an edge has been detected and synchronized, the 16-bit free-running counter

value is transferred into the input capture register pair as a single 16-bit parallel transfer.

Timer counter value captures and timer counter incrementing occur on opposite

half-cycles of the phase 2 clock so that the count value is stable whenever a capture

occurs. The TICx registers are not affected by reset. Input capture values can be read

from a pair of 8-bit read-only registers. A read of the high-order byte of an input capture

register pair inhibits a new capture transfer for one bus cycle. If a double-byte read instruction,

such as LDD, is used to read the captured value, coherency is assured.

When a new input capture occurs immediately after a high-order byte read, transfer is

delayed for an additional cycle but the value is not lost.

TICx not affected by reset.

9.2.3 Timer Input Capture 4/Output Compare 5 Register

Use TI4/O5 as either an input capture register or an output compare register, depending

on the function chosen for the PA3 pin. To enable it as an input capture pin, set the

I4/O5 bit in the pulse accumulator control register (PACTL) to logic level one. To use

it as an output compare register, set the I4/O5 bit to a logic level zero. Refer to 9.6

Pulse Accumulator.

The TI4/O5 register pair resets to ones ($FFFF).

9.3 Output Compare

Use the output compare (OC) function to program an action to occur at a specific time

— when the 16-bit counter reaches a specified value. For each of the five output compare

functions, there is a separate 16-bit compare register and a dedicated 16-bit comparator.

The value in the compare register is compared to the value of the free-running

counter on every bus cycle. When the compare register matches the counter value, an

output compare status flag is set. The flag can be used to initiate the automatic actions

for that output compare function.

TIC1–TIC3 — Timer Input Capture $1010–$1015

$1010 Bit 15 14 13 12 11 10 9 Bit 8 TIC1 (High)

$1011 Bit 7 6 5 4 3 2 1 Bit 0 TIC1 (Low)

$1012 Bit 15 14 13 12 11 10 9 Bit 8 TIC2 (High)

$1013 Bit 7 6 5 4 3 2 1 Bit 0 TIC2 (Low)

$1014 Bit 15 14 13 12 11 10 9 Bit 8 TIC3 (High)

$1015 Bit 7 6 5 4 3 2 1 Bit 0 TIC3 (Low)

TI4/O5 — Timer Input Capture 4/Output Compare 5 $101E, $101F

$101E Bit 15 14 13 12 11 10 9 Bit 8 TI4/O5 (High)

$101F Bit 7 6 5 4 3 2 1 Bit 0 TI4/O5 (Low)

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TIMING SYSTEM

TECHNICAL DATA 9-7

To produce a pulse of a specific duration, write a value to the output compare register

that represents the time the leading edge of the pulse is to occur. The output compare

circuit is configured to set the appropriate output either high or low, depending on the

polarity of the pulse being produced. After a match occurs, the output compare register

is reprogrammed to change the output pin back to its inactive level at the next match.

A value representing the width of the pulse is added to the original value, and then written

to the output compare register. Because the pin state changes occur at specific

values of the free-running counter, the pulse width can be controlled accurately at the

resolution of the free-running counter, independent of software latencies. To generate

an output signal of a specific frequency and duty cycle, repeat this pulse-generating

procedure.

There are four 16-bit read/write output compare registers: TOC1, TOC2, TOC3, and

TOC4, and the TI4/O5 register, which functions under software control as either IC4

or OC5. Each of the OC registers is set to $FFFF on reset. A value written to an OC

register is compared to the free-running counter value during each E-clock cycle. If a

match is found, the particular output compare flag is set in timer interrupt flag register

1 (TFLG1). If that particular interrupt is enabled in the timer interrupt mask register 1

(TMSK1), an interrupt is generated. In addition to an interrupt, a specified action can

be initiated at one or more timer output pins. For OC[5:2], the pin action is controlled

by pairs of bits (OMx and OLx) in the TCTL1 register. The output action is taken on

each successful compare, regardless of whether or not the OCxF flag in the TFLG1

register was previously cleared.

OC1 is different from the other output compares in that a successful OC1 compare can

affect any or all five of the OC pins. The OC1 output action taken when a match is

found is controlled by two 8-bit registers with three bits unimplemented: the output

compare 1 mask register, OC1M, and the output compare 1 data register, OC1D.

OC1M specifies which port A outputs are to be used, and OC1D specifies what data

is placed on these port pins.

9.3.1 Timer Output Compare Registers

All output compare registers are 16-bit read-write. Each is initialized to $FFFF at reset.

If an output compare register is not used for an output compare function, it can be used

as a storage location. A write to the high-order byte of an output compare register pair

inhibits the output compare function for one bus cycle. This inhibition prevents inappropriate

subsequent comparisons. Coherency requires a complete 16-bit read or

write. However, if coherency is not needed, byte accesses can be used.

For output compare functions, write a comparison value to output compare registers

TOC1–TOC4 and TI4/O5. When TCNT value matches the comparison value, specified

pin actions occur.

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TIMING SYSTEM MC68HC11F1

9-8 TECHNICAL DATA

All TOCx register pairs reset to ones ($FFFF).

9.3.2 Timer Compare Force Register

The CFORC register allows forced early compares. FOC[1:5] correspond to the five

output compares. These bits are set for each output compare that is to be forced. The

action taken as a result of a forced compare is the same as if there were a match between

the OCx register and the free-running counter, except that the corresponding

interrupt status flag bits are not set. The forced channels trigger their programmed pin

actions to occur at the next timer count transition after the write to CFORC.

The CFORC bits should not be used on an output compare function that is programmed

to toggle its output on a successful compare because a normal compare that

occurs immediately before or after the force can result in an undesirable operation.

FOC[1:5] — Force Output Comparison

When the FOC bit associated with an output compare circuit is set, the output compare

circuit immediately performs the action it is programmed to do when an output match

occurs.

0 = Not affected

1 = Output x action occurs

Bits [2:0] — Not implemented

Always read zero

9.3.3 Output Compare Mask Registers

Use OC1M with OC1 to specify the bits of port A that are affected by a successful OC1

compare. The bits of the OC1M register correspond to PA[7:3].

TOC1–TOC4 — Timer Output Compare $1016–$101D

$1016 Bit 15 14 13 12 11 10 9 Bit 8 TOC1 (High)

$1017 Bit 7 6 5 4 3 2 1 Bit 0 TOC1 (Low)

$1018 Bit 15 14 13 12 11 10 9 Bit 8 TOC2 (High)

$1019 Bit 7 6 5 4 3 2 1 Bit 0 TOC2 (Low)

$101A Bit 15 14 13 12 11 10 9 Bit 8 TOC3 (High)

$101B Bit 7 6 5 4 3 2 1 Bit 0 TOC3 (Low)

$101C Bit 15 14 13 12 11 10 9 Bit 8 TOC4 (High)

$101D Bit 7 6 5 4 3 2 1 Bit 0 TOC4 (Low)

CFORC — Timer Compare Force $100B

Bit 7 6 5 4 3 2 1 Bit 0

FOC1 FOC2 FOC3 FOC4 FOC5 — — —

RESET: 0 0 0 0 0 0 0 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TIMING SYSTEM

TECHNICAL DATA 9-9

OC1M[7:3] — Output Compare Masks

0 = OC1 is disabled.

1 = OC1 is enabled to control the corresponding pin of port A

Bits [2:0] — Not implemented

Always read zero

9.3.4 Output Compare Data Register

Use this register with OC1 to specify the data that is to be stored on the affected pin

of port A after a successful OC1 compare. When a successful OC1 compare occurs,

a data bit in OC1D is stored in the corresponding bit of port A for each bit that is set in

OC1M.

If OC1Mx is set, data in OC1Dx is output to port A bit x on successful OC1 compares.

Bits [2:0] — Not implemented

Always read zero

9.3.5 Timer Counter Register

The 16-bit read-only TCNT register contains the prescaled value of the 16-bit timer. A

full counter read addresses the most significant byte (MSB) first. A read of this address

causes the least significant byte (LSB) to be latched into a buffer for the next CPU cycle

so that a double-byte read returns the full 16-bit state of the counter at the time of

the MSB read cycle.

TCNT resets to $0000. In normal modes, TCNT is a read-only register.

9.3.6 Timer Control Register 1

The bits of this register specify the action taken as a result of a successful OCx compare.

OC1M — Output Compare 1 Mask $100C

Bit 7 6 5 4 3 2 1 Bit 0

OC1M7 OC1M6 OC1M5 OC1M4 OC1M3 — — —

RESET: 0 0 0 0 0 0 0 0

OC1D — Output Compare 1 Data $100D

Bit 7 6 5 4 3 2 1 Bit 0

OC1D7 OC1D6 OC1D5 OC1D4 OC1D3 — — —

RESET: 0 0 0 0 0 0 0 0

TCNT — Timer Counter $100E, $100F

$100E Bit 15 14 13 12 11 10 9 Bit 8 TCNT (High)

$100F Bit 7 6 5 4 3 2 1 Bit 0 TCNT (Low)

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TIMING SYSTEM MC68HC11F1

9-10 TECHNICAL DATA

OM[2:5] — Output Mode

OL[2:5] — Output Level

These control bit pairs are encoded to specify the action taken after a successful OCx

compare. OC5 functions only if the I4/O5 bit in the PACTL register is clear. Refer to

Table 9-3 for the coding.

9.3.7 Timer Interrupt Mask Register 1

Use this 8-bit register to enable or inhibit the timer input capture and output compare

interrupts.

OC1I–OC4I — Output Compare x Interrupt Enable

If the OCxI enable bit is set when the OCxF flag bit is set, a hardware interrupt sequence

is requested.

I4/O5I — Input Capture 4/Output Compare 5 Interrupt Enable

When I4/O5 in PACTL is one, I4/O5I is the input capture 4 interrupt enable bit. When

I4/O5 in PACTL is zero, I4/O5I is the output compare 5 interrupt enable bit.

IC1I–IC3I — Input Capture x Interrupt Enable

If the ICxI enable bit is set when the ICxF flag bit is set, a hardware interrupt sequence

is requested.

NOTE

Bits in TMSK1 correspond bit for bit with flag bits in TFLG1. Ones in

TMSK1 enable the corresponding interrupt sources.

TCTL1 — Timer Control 1 $1020

Bit 7 6 5 4 3 2 1 Bit 0

OM2 OL2 OM3 OL3 OM4 OL4 OM5 OL5

RESET: 0 0 0 0 0 0 0 0

Table 9-2 Timer Output Compare Configuration

OMx OLx Action Taken on Successful Compare

0 0 Timer disconnected from output pin logic

0 1 Toggle OCx output line

1 0 Clear OCx output line to zero

1 1 Set OCx output line to one

TMSK1 — Timer Interrupt Mask 1 $1022

Bit 7 6 5 4 3 2 1 Bit 0

OC1I OC2I OC3I OC4I I4/O5I IC1I IC2I IC3I

RESET: 0 0 0 0 0 0 0 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TIMING SYSTEM

TECHNICAL DATA 9-11

9.3.8 Timer Interrupt Flag Register 1

Bits in this register indicate when timer system events have occurred. Coupled with

the bits of TMSK1, the bits of TFLG1 allow the timer subsystem to operate in either a

polled or interrupt driven system. Each bit of TFLG1 corresponds to a bit in TMSK1 in

the same position.

Clear flags by writing a one to the corresponding bit position(s).

OC1F–OC4F — Output Compare x Flag

Set each time the counter matches output compare x value

I4/O5F — Input Capture 4/Output Compare 5 Flag

Set by IC4 or OC5, depending on the function enabled by I4/O5 bit in PACTL

IC1F–IC3F — Input Capture x Flag

Set each time a selected active edge is detected on the ICx input line

9.3.9 Timer Interrupt Mask Register 2

Use this 8-bit register to enable or inhibit timer overflow and real-time interrupts. The

timer prescaler control bits are included in this register.

TOI — Timer Overflow Interrupt Enable

0 = TOF interrupts disabled

1 = Interrupt requested when TOF is set to one

RTII — Real-Time Interrupt Enable

Refer to 9.4 Real-Time Interrupt.

PAOVI — Pulse Accumulator Overflow Interrupt Enable

Refer to 9.6.3 Pulse Accumulator Status and Interrupt Bits.

PAII — Pulse Accumulator Input Edge Interrupt Enable

Refer to 9.6.3 Pulse Accumulator Status and Interrupt Bits.

PR[1:0] — Timer Prescaler Select

These bits are used to select the prescaler divide-by ratio. In normal modes, PR[1:0]

can only be written once, and the write must be within 64 cycles after reset. Refer to

Table 9-1 and Table 9-4 for specific timing values.

TFLG1 — Timer Interrupt Flag 1 $1023

Bit 7 6 5 4 3 2 1 Bit 0

OC1F OC2F OC3F OC4F I4/O5F IC1F IC2F IC3F

RESET: 0 0 0 0 0 0 0 0

TMSK2 — Timer Interrupt Mask 2 $1024

Bit 7 6 5 4 3 2 1 Bit 0

TOI RTII PAOVI PAII — — PR1 PR0

RESET: 0 0 0 0 0 0 0 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TIMING SYSTEM MC68HC11F1

9-12 TECHNICAL DATA

NOTE

Bits in TMSK2 correspond bit for bit with flag bits in TFLG2. Ones in

TMSK2 enable the corresponding interrupt sources.

9.3.10 Timer Interrupt Flag Register 2

Bits in this register indicate when certain timer system events have occurred. Coupled

with the four high-order bits of TMSK2, the bits of TFLG2 allow the timer subsystem to

operate in either a polled or interrupt driven system. Each bit of TFLG2 corresponds

to a bit in TMSK2 in the same position.

Clear flags by writing a one to the corresponding bit position(s).

TOF — Timer Overflow Interrupt Flag

Set when TCNT changes from $FFFF to $0000

RTIF — Real-Time (Periodic) Interrupt Flag

Refer to 9.4 Real-Time Interrupt.

PAOVF — Pulse Accumulator Overflow Interrupt Flag

Refer to 9.6 Pulse Accumulator.

PAIF — Pulse Accumulator Input Edge Interrupt Flag

Refer to 9.6 Pulse Accumulator.

Bits [3:0] — Not implemented

Always read zero

9.4 Real-Time Interrupt

The real-time interrupt (RTI) feature, used to generate hardware interrupts at a fixed

periodic rate, is controlled and configured by two bits (RTR1 and RTR0) in the pulse

accumulator control (PACTL) register. The RTII bit in the TMSK2 register enables the

interrupt capability. The four different rates available are a product of the MCU oscillator

frequency and the value of bits RTR[1:0]. Refer to Table 9-4, which shows the

periodic real-time interrupt rates.

Table 9-3 Timer Prescaler Selection

PR[1:0] Prescaler

0 0 1

0 1 4

1 0 8

1 1 16

TFLG2 — Timer Interrupt Flag 2 $1025

Bit 7 6 5 4 3 2 1 Bit 0

TOF RTIF PAOVF PAIF — — — —

RESET: 0 0 0 0 0 0 0 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TIMING SYSTEM

TECHNICAL DATA 9-13

The clock source for the RTI function is a free-running clock that cannot be stopped or

interrupted except by reset. This clock causes the time between successive RTI timeouts

to be a constant that is independent of the software latencies associated with flag

clearing and service. For this reason, an RTI period starts from the previous time-out,

not from when RTIF is cleared.

Every time-out causes the RTIF bit in TFLG2 to be set, and if RTII is set, an interrupt

request is generated. After reset, one entire RTI period elapses before the RTIF flag

is set for the first time. Refer to the TMSK2, TFLG2, and PACTL registers.

9.4.1 Timer Interrupt Mask Register 2

This register contains the real-time interrupt enable bits.

TOI — Timer Overflow Interrupt Enable

0 = TOF interrupts disabled

1 = Interrupt requested when TOF is set to one

RTII — Real-Time Interrupt Enable

0 = RTIF interrupts disabled

1 = Interrupt requested when RTIF set to one

PAOVI — Pulse Accumulator Overflow Interrupt Enable

Refer to 9.6 Pulse Accumulator.

PAII — Pulse Accumulator Input Edge

Refer to 9.6 Pulse Accumulator.

PR[1:0] — Timer Prescaler Select

Refer to Table 9-4.

NOTE

Bits in TMSK2 correspond bit for bit with flag bits in TFLG2. Ones in

TMSK2 enable the corresponding interrupt sources.

Table 9-4 RTI Rate Selection

RTR[1:0] E = 1 MHz E = 2 MHz E = 3 MHz E = 4 MHz E = X MHz

0 0

0 1

1 0

1 1

8.192 ms

16.384 ms

32.768 ms

65.536 ms

4.096 ms

8.192 ms

16.384 ms

32.768 ms

2.731 ms

5.461 ms

10.923 ms

21.845 ms

2.048 ms

4.096 ms

8.192 ms

16.384 ms

(213/E)

(214/E)

(215/E)

(216/E)

TMSK2 — Timer Interrupt Mask Register 2 $1024

Bit 7 6 5 4 3 2 1 Bit 0

TOI RTII PAOVI PAII — — PR1 PR0

RESET: 0 0 0 0 0 0 0 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TIMING SYSTEM MC68HC11F1

9-14 TECHNICAL DATA

9.4.2 Timer Interrupt Flag Register 2

Bits of this register indicate the occurrence of timer system events. Coupled with the

four high-order bits of TMSK2, the bits of TFLG2 allow the timer subsystem to operate

in either a polled or interrupt driven system. Each bit of TFLG2 corresponds to a bit in

TMSK2 in the same position.

Clear flags by writing a one to the corresponding bit position(s).

TOF — Timer Overflow Interrupt Flag

Set when TCNT changes from $FFFF to $0000

RTIF — Real-Time Interrupt Flag

The RTIF status bit is automatically set to one at the end of every RTI period. To clear

RTIF, write a byte to TFLG2 with bit 6 set.

PAOVF — Pulse Accumulator Overflow Interrupt Flag

Refer to 9.6 Pulse Accumulator.

PAIF — Pulse Accumulator Input Edge Interrupt Flag

Refer to 9.6 Pulse Accumulator.

Bits [3:0] — Not implemented

Always read zero

9.4.3 Pulse Accumulator Control Register

Bits RTR[1:0] of this register select the rate for the RTI system. The remaining bits control

the pulse accumulator and IC4/OC5 functions.

Bit 7 — Not implemented

Always reads zero

PAEN — Pulse Accumulator System Enable

Refer to 9.6 Pulse Accumulator.

PAMOD — Pulse Accumulator Mode

Refer to 9.6 Pulse Accumulator.

TFLG2 — Timer Interrupt Flag 2 $1025

Bit 7 6 5 4 3 2 1 Bit 0

TOF RTIF PAOVF PAIF — — — —

RESET: 0 0 0 0 0 0 0 0

PACTL — Pulse Accumulator Control $1026

Bit 7 6 5 4 3 2 1 Bit 0

— PAEN PAMOD PEDGE — I4/O5 RTR1 RTR0

RESET: 0 0 0 0 0 0 0 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TIMING SYSTEM

TECHNICAL DATA 9-15

PEDGE — Pulse Accumulator Edge Control

Refer to 9.6 Pulse Accumulator.

Bit 3 — Not implemented

Always reads zero

I4/O5 — Input Capture 4/Output Compare

Refer to 9.6 Pulse Accumulator.

RTR[1:0] — RTI Interrupt Rate Select

These two bits determine the rate at which the RTI system requests interrupts. The

RTI system is driven by an E divided by 213 rate clock that is compensated so it is independent

of the timer prescaler. These two control bits select an additional division

factor. Refer to Table 9-5.

9.5 Computer Operating Properly Watchdog Function

The clocking chain for the COP function, tapped off of the main timer divider chain, is

only superficially related to the main timer system. The CR[1:0] bits in the OPTION

register and the NOCOP bit in the CONFIG register determine the status of the COP

function. One additional register, COPRST, is used to arm and clear the COP watchdog

reset system. Refer to SECTION 5 RESETS AND INTERRUPTS for a more detailed

discussion of the COP function.

9.6 Pulse Accumulator

The MC68HC11F1 MCUs have an 8-bit counter that can be configured to operate either

as a simple event counter, or for gated time accumulation, depending on the state

of the PAMOD bit in the PACTL register. Refer to the pulse accumulator block diagram,

Figure 9-3.

In the event counting mode, the 8-bit counter is incremented by pulses on an external

pin (PAI). The maximum clocking rate for the external event counting mode is the E

clock divided by two. In gated time accumulation mode, a free-running E-clock ¸ 64

signal drives the 8-bit counter, but only while the external PAI pin is activated. Refer

to Table 9-6. The pulse accumulator counter can be read or written at any time.

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TIMING SYSTEM MC68HC11F1

9-16 TECHNICAL DATA

Figure 9-3 Pulse Accumulator

Pulse accumulator control bits are also located within two timer registers, TMSK2 and

TFLG2, as described in the following paragraphs.

9.6.1 Pulse Accumulator Control Register

Four of this register's bits control an 8-bit pulse accumulator system. Another bit enables

either the OC5 function or the IC4 function, while two other bits select the rate

for the real-time interrupt system.

Table 9-5 Pulse Accumulator Timing

Crystal Frequency

(4*E)

E Clock

(E)

Cycle Time

(1/E)

26/E

(64/E)

PACNT Overflow

(16384/E)

4.0 MHz 1.0 MHz 1000 ns 64 ms 16.384 ms

8.0 MHz 2.0 MHz 500 ns 32 ms 8.192 ms

12.0 MHz 3.0 MHz 333 ns 21.33 ms 5.461 ms

16.0 MHz 4.0 MHz 250 ns 16.0 ms 4.096 ms

PEDGE

PAMOD

PAEN

PACTL CONTROL

INTERNAL

DATA BUS

PA7/ PACNT 8-BIT COUNTER

PAI/

OC1

INTERRUPT

REQUESTS

PAIF

PAOVF

TFLG2 INTERRUPT STATUS

PAOVI

PAII

PAOVF

PAOVI

PAIF

PAII

TMSK2 INT ENABLES

1

2

OVERFLOW

ENABLE

DISABLE

FLAG SETTING

CLOCK

PAI EDGE

PAEN

PAEN

2:1

MUX

OUTPUT

BUFFER

INPUT BUFFER

AND

EDGE DETECTOR

FROM

MAIN TIMER

OC1

DATA BUS

PIN

E ¸ 64 CLOCK

(FROM MAIN TIMER)

FROM DATA

DIRECTION

BIT FOR

PORT A PIN 7

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TIMING SYSTEM

TECHNICAL DATA 9-17

Bit 7 — Not implemented

Always reads zero

PAEN — Pulse Accumulator System Enable

0 = Pulse accumulator disabled

1 = Pulse accumulator enabled

PAMOD — Pulse Accumulator Mode

0 = Event counter

1 = Gated time accumulation

PEDGE — Pulse Accumulator Edge Control

This bit has different meanings depending on the state of the PAMOD bit, as shown in

Table 9-6.

Bit 3 — Not implemented

Always reads zero

I4/O5 — Input Capture 4/Output Compare 5

0 = Output compare 5 function enable (No IC4)

1 = Input capture 4 function enable (No OC5)

RTR[1:0] — RTI Interrupt Rate Selects

Refer to 9.4 Real-Time Interrupt.

9.6.2 Pulse Accumulator Count Register

This 8-bit read/write register contains the count of external input events at the PAI input,

or the accumulated count. The PACNT is readable even if PAI is not active in gated

time accumulation mode. The counter is not affected by reset and can be read or

written at any time. Counting is synchronized to the internal PH2 clock so that incrementing

and reading occur during opposite half cycles.

PACTL — Pulse Accumulator Control $1026

Bit 7 6 5 4 3 2 1 Bit 0

— PAEN PAMOD PEDGE — I4/O5 RTR1 RTR0

RESET: 0 0 0 0 0 0 0 0

Table 9-6 Pulse Accumulator Edge Detection Control

PAMOD PEDGE Action on Clock

0 0 PAI falling edge increments the counter.

0 1 PAI rising edge increments the counter.

1 0 A zero on PAI inhibits counting.

1 1 A one on PAI inhibits counting.

PACNT — Pulse Accumulator Count $1027

Bit 7 6 5 4 3 2 1 Bit 0

Bit 7 6 5 4 3 2 1 Bit 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TIMING SYSTEM MC68HC11F1

9-18 TECHNICAL DATA

9.6.3 Pulse Accumulator Status and Interrupt Bits

The pulse accumulator control bits, PAOVI, PAII, PAOVF, and PAIF are located within

timer registers TMSK2 and TFLG2.

PAOVI and PAOVF — Pulse Accumulator Interrupt Enable and Overflow Flag

The PAOVF status bit is set each time the pulse accumulator count rolls over from $FF

to $00. To clear this status bit, write a one in the corresponding data bit position (bit 5)

of the TFLG2 register. The PAOVI control bit allows configuring the pulse accumulator

overflow for polled or interrupt-driven operation and does not affect the state of

PAOVF. When PAOVI is zero, pulse accumulator overflow interrupts are inhibited, and

the system operates in a polled mode, which requires that PAOVF be polled by user

software to determine when an overflow has occurred. When the PAOVI control bit is

set, a hardware interrupt request is generated each time PAOVF is set. Before leaving

the interrupt service routine, software must clear PAOVF by writing to the TFLG2 register.

PAII and PAIF — Pulse Accumulator Input Edge Interrupt Enable and Flag

The PAIF status bit is automatically set each time a selected edge is detected at the

PA7/PAI/OC1 pin. To clear this status bit, write to the TFLG2 register with a one in the

corresponding data bit position (bit 4). The PAII control bit allows configuring the pulse

accumulator input edge detect for polled or interrupt-driven operation but does not affect

setting or clearing the PAIF bit. When PAII is zero, pulse accumulator input interrupts

are inhibited, and the system operates in a polled mode. In this mode, the PAIF

bit must be polled by user software to determine when an edge has occurred. When

the PAII control bit is set, a hardware interrupt request is generated each time PAIF is

set. Before leaving the interrupt service routine, software must clear PAIF by writing to

the TFLG2 register.

TMSK2 — Timer Interrupt Mask 2 Register $1024

Bit 7 6 5 4 3 2 1 Bit 0

TOI RTII PAOVI PAII — — PR1 PR0

RESET: 0 0 0 0 0 0 0 0

TFLG2 — Timer Interrupt Flag 2 Register $1025

Bit 7 6 5 4 3 2 1 Bit 0

TOF RTIF PAOVF PAIF — — — —

RESET: 0 0 0 0 0 0 0 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ANALOG-TO-DIGITAL CONVERTER

TECHNICAL DATA 10-1

SECTION 10 ANALOG-TO-DIGITAL CONVERTER

The analog-to-digital (A/D) system, a successive approximation converter, uses an allcapacitive

charge redistribution technique to convert analog signals to digital values.

10.1 Overview

The A/D system is an 8-channel, 8-bit, multiplexed-input converter. The AVDD pin is

used to input supply voltage to the A/D converter. This allows the supply voltage to be

bypassed independently. The converter does not require external sample and hold circuits

because of the type of charge redistribution technique used. A/D converter timing

can be synchronized to the system E clock, or to an internal resistor capacitor (RC)

oscillator. The A/D converter system consists of four functional blocks: multiplexer, analog

converter, digital control, and result storage. Refer to Figure 10-1.

10.1.1 Multiplexer

The multiplexer selects one of 16 inputs for conversion. Input selection is controlled by

the value of bits CD–CA in the ADCTL register. The eight port E pins are fixed-direction

analog inputs to the multiplexer, and internal analog signal lines are routed to it.

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ANALOG-TO-DIGITAL CONVERTER MC68HC11F1

10-2 TECHNICAL DATA

Figure 10-1 A/D Converter Block Diagram

Port E pins can also be used as digital inputs. Reads of port E pins are not recommended

during the sample portion of an A/D conversion cycle, when the gate signal

to the N-channel input gate is on. Because no P-channel devices are directly connected

to either input pins or reference voltage pins, voltages above VDD do not cause a

latchup problem, although current should be limited according to maximum ratings.

Refer to Figure 10-2, which is a functional diagram of an input pin.

PE0/

AN0

PE1/

AN1

PE2/

AN2

PE3/

AN3

PE4/

AN4

PE5/

AN5

PE6/

AN6

PE7/

AN7

ANALOG

MUX

8-BIT CAPACITIVE DAC

WITH SAMPLE AND HOLD

SUCCESSIVE APPROXIMATION

REGISTER AND CONTROL

ADCTL A/D CONTROL

CB

CC

CD

MULT

SCAN

CCF

CA

ADDR 3 A/D RESULT 3

RESULT REGISTER INTERFACE

RESULT

INTERNAL

DATA BUS

VRH

VRL

ADDR 1 A/D RESULT 1 ADDR 2 A/D RESULT 2 ADDR 4 A/D RESULT 4

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ANALOG-TO-DIGITAL CONVERTER

TECHNICAL DATA 10-3

Figure 10-2 Electrical Model of an A/D Input Pin (Sample Mode)

10.1.2 Analog Converter

Conversion of an analog input selected by the multiplexer occurs in this block. It contains

a digital-to-analog capacitor (DAC) array, a comparator, and a successive approximation

register (SAR). Each conversion is a sequence of eight comparison

operations, beginning with the most significant bit (MSB). Each comparison determines

the value of a bit in the successive approximation register.

The DAC array performs two functions. It acts as a sample and hold circuit during the

entire conversion sequence, and provides comparison voltage to the comparator during

each successive comparison.

The result of each successive comparison is stored in the SAR. When a conversion

sequence is complete, the contents of the SAR are transferred to the appropriate result

register.

A charge pump provides switching voltage to the gates of analog switches in the multiplexer.

Charge pump output must stabilize between 7 and 8 volts, thus a delay of up

to 100 ms must be imposed after setting ADPU before the converter can be used. The

charge pump is enabled by the ADPU bit in the OPTION register.

Power is provided to the A/D converter system through the AVDD and AVSS pins.

10.1.3 Digital Control

All A/D converter operations are controlled by bits in register ADCTL. In addition to selecting

the analog input to be converted, ADCTL bits indicate conversion status, and

control whether single or continuous conversions are performed. Finally, the ADCTL

bits determine whether conversions are performed on single or multiple channels.

10.1.4 Result Registers

Four 8-bit registers (ADR1–ADR4) store conversion results. Each of these registers

can be accessed by the processor in the CPU. The conversion complete flag (CCF)

DAC

CAPACITANCE

VRL

ANALOG

INPUT

PIN

~ 20 pF

+ ~ 20 V

– ~ 0.7 V

< 2 pF

INPUT

PROTECTION

DEVICE

400 nA

JUNCTION

LEAKAGE

DIFFUSION AND

POLY COUPLER

£ 4 kW

* This analog switch is closed only during the 12-cycle sample time.

*

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ANALOG-TO-DIGITAL CONVERTER MC68HC11F1

10-4 TECHNICAL DATA

indicates when valid data is present in the result registers. The result registers are written

during a portion of the system clock cycle when reads do not occur, so there is no

conflict.

10.1.5 A/D Converter Clocks

The CSEL bit in the OPTION register selects whether the A/D converter uses the system

E clock or an internal RC oscillator for synchronization. When the A/D system is

operating with the MCU E clock, all switching and comparator functions are synchronized

to the MCU clocks. This allows the comparator results to be sampled at relatively

quiet clock times to minimize noise errors.

When E-clock frequency is below 750 kHz, charge leakage in the capacitor array can

cause errors, and the internal oscillator should be used. The RC clock is asynchronous

to the MCU internal E clock. Therefore, when the RC clock is used, additional errors

can occur because the comparator is sensitive to the additional system clock noise.

10.1.6 Conversion Sequence

A/D converter operations are performed in sequences of four conversions each. A

conversion sequence can repeat continuously or stop after one iteration. The conversion

complete flag (CCF) is set after the fourth conversion in a sequence to show the

availability of data in the result registers. Figure 10-3 shows the timing of a typical sequence.

Synchronization is referenced to the system E clock.

Figure 10-3 A/D Conversion Sequence

MSB

4

CYCLES

E CLOCK

WRITE

BIT 6

2

CYC

BIT 5

2

CYC

BIT 4

2

CYC

BIT 3

2

CYC

BIT 2

2

CYC

BIT 1

2

CYC

LSB

2

CYC

END

12 E CYCLES

SAMPLE ANALOG INPUT SUCCESSIVE APPROXIMATION SEQUENCE

2

CYC

CONVERT FOURTH

CHANNEL

AND UPDATE ADDR4

CONVERT THIRD

CHANNEL

AND UPDATE ADDR3

CONVERT SECOND

CHANNEL

AND UPDATE ADDR2

CONVERT FIRST

CHANNEL

AND UPDATE ADDR1

SET

REPEAT

E

CYCLES

0 32 64 96 128

TO

ADCTL

SEQUENCE

IF

SCAN = 1

CCF

FLAG

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ANALOG-TO-DIGITAL CONVERTER

TECHNICAL DATA 10-5

10.2 A/D Converter Power-Up and Clock Select

Bit 7 of the OPTION register controls A/D converter power up. Clearing ADPU removes

power from and disables the A/D converter system. Setting ADPU enables the

A/D converter system. Stabilization of the analog bias voltages requires a delay of as

much as 100 ms after turning on the A/D converter. When the A/D converter system is

operating with the MCU E clock, all switching and comparator operations are synchronized

to the MCU clocks. This allows the comparator results to be sampled at quiet

times, which minimizes noise errors. The internal RC oscillator is asynchronous to the

MCU clock, so noise affects A/D converter results, which lowers accuracy slightly

while CSEL = 1.

*Can be written only once in first 64 cycles out of reset in normal modes, or at any time in special modes

ADPU — A/D Power-Up

0 = A/D powered down

1 = A/D powered up

CSEL — Clock Select

0 = A/D and EEPROM use system E clock

1 = A/D and EEPROM use internal RC clock

IRQE — Configure IRQ for Edge-Sensitive Only Operation

Refer to SECTION 5 RESETS AND INTERRUPTS.

DLY — Enable Oscillator Start-up Delay

Refer to SECTION 5 RESETS AND INTERRUPTS.

CME — Clock Monitor Enable

Refer to SECTION 5 RESETS AND INTERRUPTS.

FCME — Force Clock Monitor Enable

Refer to SECTION 5 RESETS AND INTERRUPTS.

CR[1:0] — COP Timer Rate Select Bits

Refer to SECTION 5 RESETS AND INTERRUPTS and SECTION 9 TIMING SYSTEM.

10.3 Conversion Process

The A/D conversion sequence begins one E-clock cycle after a write to the A/D control/

status register, ADCTL. The bits in ADCTL select the channel and the mode of conversion.

An input voltage equal to VRL converts to $00 and an input voltage equal to VRH converts

to $FF (full scale), with no overflow indication. For ratiometric conversions of this

type, the source of each analog input should use VRH as the supply voltage and be

referenced to VRL.

OPTION — System Configuration Options $1039

Bit 7 6 5 4 3 2 1 Bit 0

ADPU CSEL IRQE* DLY* CME FCME* CR1* CR0*

RESET: 0 0 0 1 0 0 0 0

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ANALOG-TO-DIGITAL CONVERTER MC68HC11F1

10-6 TECHNICAL DATA

10.4 Channel Assignments

The multiplexer allows the A/D converter to select one of sixteen analog signals. Eight

of these channels correspond to port E input lines, four of the channels are internal

reference points or test functions, and four channels are reserved. Refer to Table 10-

1.

*Used for factory testing

10.5 Single-Channel Operation

There are two types of single-channel operation. When SCAN = 0, the first type, the

single selected channel is converted four consecutive times. The first result is stored

in A/D result register 1 (ADR1), and the fourth result is stored in ADR4. After the fourth

conversion is complete, all conversion activity is halted until a new conversion command

is written to the ADCTL register. In the second type of single-channel operation,

SCAN = 1, conversions continue to be performed on the selected channel with the fifth

conversion being stored in register ADR1 (overwriting the first conversion result), the

sixth conversion overwriting ADR2, and so on.

10.6 Multiple-Channel Operation

There are two types of multiple-channel operation. When SCAN = 0, the first type, a

selected group of four channels is converted one time each. The first result is stored

in A/D result register 1 (ADR1), and the fourth result is stored in ADR4. After the fourth

conversion is complete, all conversion activity is halted until a new conversion command

is written to the ADCTL register. In the second type of multiple-channel operation,

SCAN = 1, conversions continue to be performed on the selected group of

channels with the fifth conversion being stored in register ADR1 (replacing the earlier

conversion result for the first channel in the group), the sixth conversion overwriting

ADR2, and so on.

Table 10-1 A/D Converter Channel Assignments

Channel

Number

Channel

Signal

Result in ADRx if

MULT = 1

1 AN0 ADR1

2 AN1 ADR2

3 AN2 ADR3

4 AN3 ADR4

5 AN4 ADR1

6 AN5 ADR2

7 AN6 ADR3

8 AN7 ADR4

9 Reserved —

10 Reserved —

11 Reserved —

12 Reserved —

13 VRH* ADR1

14 VRL* ADR2

15 (VRH)/2* ADR3

16 Reserved* ADR4

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ANALOG-TO-DIGITAL CONVERTER

TECHNICAL DATA 10-7

10.7 Operation in STOP and WAIT Modes

If a conversion sequence is in progress when either the STOP or WAIT mode is entered,

the conversion of the current channel is suspended. When the MCU resumes

normal operation, that channel is resampled and the conversion sequence is resumed.

As the MCU exits the WAIT mode, the A/D circuits are stable and valid results can be

obtained on the first conversion. However, in STOP mode, all analog bias currents are

disabled and it is necessary to allow a stabilization period when leaving the STOP

mode. If the STOP mode is exited with a delay (DLY = 1), there is enough time for

these circuits to stabilize before the first conversion. If the STOP mode is exited with

no delay (DLY bit in OPTION register = 0), allow 10 ms for the A/D circuitry to stabilize

to avoid invalid results.

10.8 A/D Control/Status Registers

All bits in this register can be read or written, except CCF (bit 7), which is a read-only

status indicator, and bit 6, which always reads as zero. Write to ADCTL to initiate a

conversion. To quit a conversion in progress, write to this register and a new conversion

sequence begins immediately.

CCF — Conversions Complete Flag

A read-only status indicator, this bit is set when all four A/D result registers contain valid

conversion results. Each time the ADCTL register is overwritten, this bit is automatically

cleared to zero and a conversion sequence is started. In the continuous mode,

CCF is set at the end of the first conversion sequence.

Bit 6 — Not implemented

Always reads zero

SCAN — Continuous Scan Control

When this control bit is clear, the four requested conversions are performed once to

fill the four result registers. When this control bit is set, conversions continue in a

round-robin fashion with the result registers updated as data becomes available.

MULT — Multiple Channel/Single Channel Control

When this bit is clear, the A/D converter system is configured to perform four consecutive

conversions on the single channel specified by the four channel select bits CD–

CA (bits [3:0] of the ADCTL register). When this bit is set, the A/D system is configured

to perform a conversion on each of four channels where each result register corresponds

to one channel.

ADCTL — A/D Control/Status $1030

Bit 7 6 5 4 3 2 1 Bit 0

CCF — SCAN MULT CD CC CB CA

RESET: 1 0 I I I I I I

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ANALOG-TO-DIGITAL CONVERTER MC68HC11F1

10-8 TECHNICAL DATA

NOTE

When the multiple-channel continuous scan mode is used, extra care

is needed in the design of circuitry driving the A/D inputs. The charge

on the capacitive DAC array before the sample time is related to the

voltage on the previously converted channel. A charge share situation

exists between the internal DAC capacitance and the external

circuit capacitance. Although the amount of charge involved is small,

the rate at which it is repeated is every 64 ms for an E clock of 2 MHz.

The RC charging rate of the external circuit must be balanced against

this charge sharing effect to avoid errors in accuracy. Refer to

M68HC11 Reference Manual (M68HC11RM/AD) for further information.

CD–CA — Channel Selects D–A

Refer to Table 10-2. When a multiple channel mode is selected (MULT = 1), the two

least significant channel select bits (CB and CA) have no meaning and the CD and CC

bits specify which group of four channels is to be converted.

*Used for factory testing

10.9 A/D Converter Result Registers

These read-only registers hold an 8-bit conversion result. Writes to these registers

have no effect. Data in the A/D converter result registers is valid when the CCF flag in

the ADCTL register is set, indicating a conversion sequence is complete. If conversion

results are needed sooner, refer to Figure 10-3, which shows the A/D conversion sequence

diagram.

Table 10-2 A/D Converter Channel Selection

Channel Select

Control Bits

Channel Signal Result in ADRx if

MULT = 1

CD:CC:CB:CA

0000 AN0 ADR1

0001 AN1 ADR2

0010 AN2 ADR3

0011 AN3 ADR4

0100 AN4 ADR1

0101 AN5 ADR2

0110 AN6 ADR3

0111 AN7 ADR4

1000 Reserved —

1001 Reserved —

1010 Reserved —

1011 Reserved —

1100 VRH* ADR1

1101 VRL* ADR2

1110 (VRH)/2* ADR3

1111 Reserved* ADR4

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ANALOG-TO-DIGITAL CONVERTER

TECHNICAL DATA 10-9

ADR1–ADR4 — A/D Results $1031–$1034

$1031 Bit 7 6 5 4 3 2 1 Bit 0 ADR1

$1032 Bit 7 6 5 4 3 2 1 Bit 0 ADR2

$1033 Bit 7 6 5 4 3 2 1 Bit 0 ADR3

$1034 Bit 7 6 5 4 3 2 1 Bit 0 ADR4

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ANALOG-TO-DIGITAL CONVERTER MC68HC11F1

10-10 TECHNICAL DATA

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ELECTRICAL CHARACTERISTICS

TECHNICAL DATA A-1

APPENDIX A ELECTRICAL CHARACTERISTICS

This appendix contains electrical parameters for the MC68HC11F1 microcontroller.

*One pin at a time, observing maximum power dissipation limits.

Internal circuitry protects the inputs against damage caused by high static voltages or

electric fields; however, normal precautions are necessary to avoid application of any

voltage higher than maximum-rated voltages to this high-impedance circuit. Extended

operation at the maximum ratings can adversely affect device reliability. Tying unused

inputs to an appropriate logic voltage level (either GND or VDD) enhances reliability of

operation.

Table A-1 Maximum Ratings

Rating Symbol Value Unit

Supply Voltage VDD – 0.3 to + 7.0 V

Input Voltage Vin – 0.3 to + 7.0 V

Operating Temperature Range

MC68HC11F1

MC68HC11F1C

MC68HC11F1V

MC68HC11F1M

TA TL to TH

0 to + 70

– 40 to + 85

– 40 to + 105

– 40 to + 125

°C

Storage Temperature Range Tstg – 55 to + 150 °C

Current Drain per Pin*

Excluding VDD, VSS, AVDD, VRH, and VRL

ID 25 mA

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ELECTRICAL CHARACTERISTICS MC68HC11F1

A-2 TECHNICAL DATA

NOTES:

1 This is an approximate value, neglecting PI/O.

2. For most applications PI/O « PINT and can be neglected.

3. K is a constant pertaining to the device. Solve for K with a known TA and a measured PD (at equilibrium. Use

this value of K to solve for PD and TJ iteratively for any value of TA

Table A-2 Thermal Characteristics

Characteristic Symbol Value Unit

Average Junction Temperature TJ TA + (PD x QJA) °C

Ambient Temperature TA User-determined °C

Package Thermal Resistance (Junction-to-Ambient)

68-Pin Plastic Leaded Chip Carrier

80-Pin Low Profile Quad Flat Pack (LQFP, 1.4 mm Thick)

QJA

50

80

°C/W

°C/W

Total Power Dissipation (Note 1) PD PINT + PI/O

K / (TJ + 273°C)

W

Device Internal Power Dissipation PINT IDD x VDD W

I/O Pin Power Dissipation (Note 2) PI/O User-determined W

A Constant (Note 3) K PD x (TA + 273°C) +

QJA x PD

2

W x °C

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ELECTRICAL CHARACTERISTICS

TECHNICAL DATA A-3

NOTES:

1. VOH specification for RESET and MODA is not applicable because they are open-drain pins. VOH specification

not applicable to ports C and D in wired-OR mode.

2. Refer to A/D specification for leakage current for port E.

3. EXTAL is driven with a square wave, and

tcyc = 500 ns for 2 MHz rating;

tcyc = 333 ns for 3 MHz rating;

tcyc = 250 ns for 4 MHz rating;

VIL £ 0.2 V; VIH ³ VDD – 0.2 V; No dc loads.

Table A-3 DC Electrical Characteristics

VDD = 5.0 Vdc ± 5%, VSS = 0 Vdc, TA = TL to TH, unless otherwise noted

Characteristic Symbol Min Max Unit

Output Voltage (Note 1) All Outputs except XTAL

All Outputs Except XTAL, RESET, and MODA

ILoad = ± 10.0 mA

VOL

VOH

—

VDD – 0.1

0.1

—

VV

Output High Voltage (Note 1)

All Outputs Except XTAL, RESET, and MODA

ILoad = – 0.8 mA, VDD = 4.5 V

VOH VDD – 0.8 — V

Output Low Voltage All Outputs Except XTAL

ILoad = 1.6 mA

VOL — 0.4 V

Input High Voltage All Inputs Except RESET

RESET

VIH 0.7 x VDD

0.8 x VDD

VDD + 0.3

VDD + 0.3

VV

Input Low Voltage All Inputs VIL VSS – 0.3 0.2 x VDD V

I/O Ports, Three-State Leakage Ports A, B, C, D, F, G

Vin = VIH or VIL MODA/LIR, RESET

IOZ — ±10 mA

Input Leakage Current (Note 2)

Vin = VDD or VSS IRQ, XIRQ on standard devices

Vin = VDD or VSS MODB/VSTBY,

XIRQ on EPROM devices

Iin

—

—

±1

±10

mA

mA

Input Current with Pull-Up Resistors

Vin = VIL Ports B, F, and G

lipr 100 500 mA

RAM Standby Voltage Power down VSB 4.0 VDD V

RAM Standby Current Power down ISB — 20 mA

Input Capacitance PE[7:0], IRQ, XIRQ, EXTAL

Ports A, B, C, D, F, G, MODA/LIR, RESET

Cin —

—

8

12

pF

pF

Output Load Capacitance All Outputs Except PD[4:1],

4XOUT, XTAL, MODA/LIR

PD[4:1]

4XOUT

CL

—

—

—

90

200

30

pF

pF

pF

Characteristic Symbol 2 MHz 3 MHz 4 MHz Unit

Maximum Total Supply Current (Note 3)

RUN:

Expanded Mode

WAIT: (All Peripheral Functions Shut Down)

Expanded Mode

STOP:

No Clocks, Expanded Mode

IDD

WIDD

SIDD

27

15

50

38

20

50

50

25

50

mA

mA

mA

Maximum Power Dissipation

Expanded Mode

PD

149 209 275 mW

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ELECTRICAL CHARACTERISTICS MC68HC11F1

A-4 TECHNICAL DATA

Figure A-1 Test Methods

20% OF VDD

70% OF VDD

VDD – 0.8 V

CLOCKS/ 70% OF VDD

STROBES

INPUTS

SPEC TIMING

~VDD

SPEC

OUTPUTS

20% OF VDD

AC TESTING

~VDD

~VSS

~VSS

SPEC

NOTES:

1. Full test loads are applied during all DC electrical tests and AC timing measurements.

2. During AC timing measurements, inputs are driven to 0.4 volts and VDD – 0.8 volts while timing measurements are

taken at the 20% and 70% of VDD points.

CLOCKS/

STROBES

INPUTS

VDD – 0.8 V

0.4 V

NOMINAL TIMING

20% OF VDD

70% OF VDD

VDD – 0.8 V

0.4 V

~VDD

NOM

OUTPUTS

0.4 V

DC TESTING

~VDD

~VSS

~VSS

NOM

20% OF VDD

0.4 V

20% OF VDD

70% OF VDD

20% OF VDD

70% OF VDD

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ELECTRICAL CHARACTERISTICS

TECHNICAL DATA A-5

NOTES:

1. All timing is shown with respect to 20% VDD and 70% VDD, unless otherwise noted.

2. RESET is recognized during the first clock cycle it is held low. Internal circuitry then drives the pin low for four

clock cycles, releases the pin, and samples the pin level two cycles later to determine the source of the interrupt.

Refer to SECTION 5 RESETS AND INTERRUPTS for further detail.

3. PWRSTL = 8 tcyc minimum on mask set C94R only.

Figure A-2 Timer Inputs

Table A-4 Control Timing

VDD = 5.0 Vdc ± 5%, VSS = 0 Vdc, TA = TL to TH

Characteristic Symbol 2.0 MHz 3.0 MHz 4.0 MHz Unit

Min Max Min Max Min Max

Frequency of Operation fo dc 2.0 dc 3.0 dc 4.0 MHz

E-Clock Period tcyc 500 — 333 — 250 — ns

Crystal Frequency fXTAL — 8.0 — 12.0 — 16.0 MHz

External Oscillator Frequency 4 fo dc 8.0 dc 12.0 dc 16.0 MHz

Processor Control Setup Time

tPCSU = 1/4 tcyc + 50 ns

tPCSU 175 — 133 — 113 — ns

Reset Input Pulse Width (Notes 2, 3)

(To Guarantee External Reset Vector)

(Minimum Input Time;

Can Be Preempted by Internal Reset)

PWRSTL

16

1

——

16

1

——

16

1

——

tcyc

tcyc

Mode Programming Setup Time tMPS 2 — 2 — 2 — tcyc

Mode Programming Hold Time tMPH 10 — 10 — 10 — ns

Interrupt Pulse Width, IRQ Edge-Sensitive Mode

PWIRQ = tcyc + 20 ns

PWIRQ 520 — 353 — 270 — ns

Wait Recovery Startup Time tWRS — 4 — 4 — 4 tcyc

Timer Pulse Width, Input Capture Pulse Accumulator

Input PWTIM = tcyc + 20 ns

PWTIM 520 — 353 — 270 — ns

NOTES:

1. Rising edge sensitive input.

2. Falling edge sensitive input.

3. Maximum pulse accumulator clocking rate is E-clock frequency divided by 2.

PA72,3

PA71,3

PA[3:0]2

PA[3:0]1

PWTIM

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ELECTRICAL CHARACTERISTICS MC68HC11F1

A-6 TECHNICAL DATA

Figure A-3 POR External Reset Timing Diagram

tPCSU

ADDRESS

MODA, MODB

E

EXTAL

VDD

RESET

4064 tCYC

FFFE FFFE FFFE NEW

PC FFFE FFFF FFFE FFFE FFFE NEW

PC FFFE FFFF FFFE

tMPH

PWRSTL

tMPS

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ELECTRICAL CHARACTERISTICS

TECHNICAL DATA A-7

Figure A-4 STOP Recovery Timing Diagram

PWIRQ

tSTOPDELAY

3

IRQ1

IRQ2

OR XIRQ

E

SP – 8 SP – 8 FFF2

(FFF4)

NEW

PC

ADDRESS4 STOP

ADDR

STOP

ADDR + 1

STOP

ADDR + 1

STOP

ADDR + 1

STOP

ADDR+2

SP…SP–7 FFF3

(FFF5)

OPCODE

RESUME PROGRAM WITH INSTR WHICH FOLLOWS THE STOP INSTR

NOTES:

1. Edge Sensitive IRQ pin (IRQE bit = 1)

2. Level sensitive IRQ pin (IRQE bit = 0)

3. tSTOPDELAY = 4064 tCYC if DLY bit = 1 or 4 tCYC if DLY = 0.

4. XIRQ with X bit in CCR = 1.

5. IRQ or (XIRQ with X bit in CCR = 0).

INTERNAL

CLOCKS

ADDRESS5 STOP

ADDR + 1

STOP

ADDR

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ELECTRICAL CHARACTERISTICS MC68HC11F1

A-8 TECHNICAL DATA

Figure A-5 WAIT Recovery from Interrupt Timing Diagram

tPCSU

PCL PCH, YL, YH, XL, XH, A, B, CCR

STACK REGISTERS

E

R/W

ADDRESS WAIT

ADDR

WAIT

ADDR+1

IRQ, XIRQ,

OR INTERNAL

INTERRUPTS

NOTE: RESET also causes recovery from WAIT.

SP SP – 1 SP – 2…SP – 8 SP – 8 SP – 8…SP – 8 SP – 8 SP – 8 SP – 8 VECTOR

ADDR

VECTOR

ADDR+1

NEW

PC

tWRS

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ELECTRICAL CHARACTERISTICS

TECHNICAL DATA A-9

Figure A-6 Interrupt Timing Diagram

E

PWIRQ

SP – 8 SP – 8 ADDRESS NEW

PC

NEXT

OPCODE

NEXT

OP + 1

IRQ1

VECTOR

ADDR SP – 7

tPCSU

IRQ2, XIRQ

OR INTERNAL

INTERRUPT

SP SP – 1 SP – 2 SP – 3 SP – 4 SP – 5 SP – 6 VECTOR

ADDR+1

OP

CODE

– –

NOTES:

1. Edge sensitive IRQ pin (IRQE bit = 1).

2. Level sensitive IRQ pin (IRQE bit = 0).

DATA PCL PCH IYL IYH IXL IXH B A CCR – – VECT

MSB

VECT

LSB

OP

CODE

R/W

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ELECTRICAL CHARACTERISTICS MC68HC11F1

A-10 TECHNICAL DATA

NOTES:

1. Ports C, D, and G timing is valid for active drive (CWOM, DWOM, and GWOM bits cleared).

2. All timing is shown with respect to 20% VDD and 70% VDD, unless otherwise noted.

Figure A-7 Port Read Timing Diagram

Figure A-8 Port Write Timing Diagram

Table A-5 Peripheral Port Timing

VDD = 5.0 Vdc ± 5%, VSS = 0 Vdc, TA = TL to TH

Characteristic Symbol 2.0 MHz 3.0 MHz 4.0 MHz Unit

Min Max Min Max Min Max

Frequency of Operation (E-Clock Frequency) fo dc 2.0 dc 3.0 dc 4.0 MHz

E-Clock Period tcyc 500 — 333 — 250 — ns

Peripheral Data Setup Time

(MCU Read of Ports A, C, D, E, G)

tPDSU 100 — 100 — 100 — ns

Peripheral Data Hold Time

(MCU Read of Ports A, C, D, E, G)

tPDH 50 — 50 — 50 — ns

Delay Time, Peripheral Data Write

(MCU Write to Port A)

(MCU Write to Ports B, C, D, F, and G

tPWD = 1/4 tcyc + 100 ns)

tPWD

——

200

225

——

200

183

——

200

162

ns

E

MCU READ OF PORT

tPDSU

PORTS A, C, D, F

PORTS B, E, G

tPDH

tPDSU tPDH

tPWD

E

MCU WRITE TO PORT

PREVIOUS PORT DATA

PREVIOUS PORT DATA

NEW DATA VALID

NEW DATA VALID

PORTS C, D, F

PORTS A, B, G

tPWD

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ELECTRICAL CHARACTERISTICS

TECHNICAL DATA A-11

NOTES:

1. For fop < 2 MHz, source impedances should equal approximately 10 kW. For fop ³ 2 MHz, source impedances

should equal approximately 5 kW – 10 kW. Source impedances greater than 10 kW affect accuracy adversely because

of input leakage.

2. Performance verified down to 2.5 V DVR, but accuracy is tested and guaranteed at DVR = 5 V ± 10%

Table A-6 Analog-To-Digital Converter Characteristics

VDD = 5.0 Vdc ± 5%, VSS = 0 Vdc, TA = TL to TH, 750 kHz £ E £ 3.0 MHz, unless otherwise noted

Characteristic Parameter Min Absolute 2.0 MHz 3.0 MHz 4.0 MHz Unit

Max Max Max

Resolution Number of Bits Resolved by A/D Converter — 8 — — — Bits

Non-Linearity Maximum Deviation from the Ideal A/D Transfer

Characteristics

— — ± 1 ± 1 ± 1 LSB

Zero Error Difference Between the Output of an Ideal and

an Actual for Zero Input Voltage

— — ± 1 ± 1 ± 1 LSB

Full Scale Error Difference Between the Output of an Ideal and

an Actual A/D for Full-Scale Input Voltage

— — ± 1 ± 1 ± 1 LSB

Total Unadjusted

Error

Maximum Sum of Non-Linearity, Zero Error, and

Full-Scale Error

— — ± 1/2 ± 1 1/2 ± 1 1/2 LSB

Quantization

Error

Uncertainty Because of Converter Resolution — — ± 1/2 ± 1/2 ± 1/2 LSB

Absolute

Accuracy

Difference Between the Actual Input Voltage and

the Full-Scale Weighted Equivalent of the Binary

Output Code, All Error Sources Included

— — ± 1 ± 2 ± 2 LSB

Conversion

Range

Analog Input Voltage Range VRL — VRH VRH VRH V

VRH Maximum Analog Reference Voltage

(Note 2)

VRL — VDD +

0.1

VDD +

0.1

VDD +

0.1

V

VRL Minimum Analog Reference Voltage

(Note 2)

VSS –0.1 — VRH VRH VRH V

DVR Minimum Difference between VRH and VRL

(Note 2)

3 — — — — V

Conversion

Time

Total Time to Perform a Single

Analog-to-Digital Conversion:

E Clock

Internal RC Oscillator

——

32

—

—

tcyc + 32

—

tcyc + 32

—

tcyc + 32

tcyc

ms

Monotonicity Conversion Result Never Decreases with an

Increase in Input Voltage and has no Missing

Codes

Guaranteed

Zero Input

Reading

Conversion Result when Vin = VRL 00 — — — — Hex

Full Scale

Reading

Conversion Result when Vin = VRH — — FF FF FF Hex

Sample

Acquisition Time

Analog Input Acquisition Sampling Time:

E Clock

Internal RC Oscillator

——

12

—

—

12

—

12

—

12

tcyc

ms

Sample/Hold

Capacitance

Input Capacitance during Sample PE[7:0] — 20 (Typ) — — — pF

Input Leakage Input Leakage on A/D Pins PE[7:0]

VRL, VRH

——

——

400

1.0

400

1.0

400

1.0

nA

mA

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ELECTRICAL CHARACTERISTICS MC68HC11F1

A-12 TECHNICAL DATA

NOTES:

1. Input clocks with duty cycles other than 50% affect bus performance.

2. Indicates a parameter affected by clock stretching. Add n(tcyc) to parameter value, where:

n = 1, 2, or 3 depending on values written to CSSTRH register.

3. All timing is shown with respect to 20% VDD and 70% VDD, unless otherwise noted.

Table A-7 Expansion Bus Timing

VDD = 5.0 Vdc ± 5%, VSS = 0 Vdc, TA = TL to TH

Num Characteristic Symbol 2.0 MHz 3.0 MHz 4.0 MHz Unit

Min Max Min Max Min Max

Frequency of Operation (E-Clock Frequency) fo dc 2.0 dc 3.0 dc 4.0 MHz

1 Cycle Time tcyc = 1/fo tcyc 500 — 333 — 250 — ns

2 Pulse Width, E Low

PWEL = 1/2 tcyc – 20 ns

PWEL 230 — 147 — 105 — ns

3 Pulse Width, E High

PWEH = 1/2 tcyc – 25 ns (Note 2)

PWEH 225 — 142 — 100 — ns

4A

4B

E Clock Rise Time

Fall Time

tr

tf

——

20

20

——

20

18

——

20

15

ns

ns

9 Address Hold Time

tAH = 1/8 tcyc – 10 ns

tAH 53 — 32 — 21 — ns

11 Address Delay Time

tAD = 1/8 tcyc + 40 ns

tAD — 103 — 82 — 71 ns

12 Address Valid Time to E Rise

tAV = PWEL – tAD

tAV 128 — 65 — 34 — ns

17 Read Data Setup Time tDSR 30 — 30 — 20 — ns

18 Read Data Hold Time tDHR 0 — 0 — 0 — ns

19 Write Data Delay Time tDDW — 40 — 40 — 40 ns

21 Write Data Hold Time

tDHW = 1/8 tcyc

tDHW 63 — 42 — 31 — ns

29 MPU Address Access Time

tACCA = tcyc – tf – tDSR – tAD (Note 2)

tACCA 348 — 203 — 144 — ns

39 Write Data Setup Time

tDSW = PWEH – tDDW (Note 2)

tDSW 185 — 102 — 60 — ns

50 E Valid Chip Select Delay Time tECSD — 40 — 40 — 40 ns

51 E Valid Chip Select Access Time

tECSA = PWEH – tECSD – tDSR (Note 2)

tECSA 155 — 72 — 40 — ns

52 Chip Select Hold Time tCH 0 20 0 20 0 20 ns

54 Address Valid Chip Select Delay Time

tACSD = 1/4 tcyc + 40 ns

tACSD — 165 — 123 — 103 ns

55 Address Valid Chip Select Access Time

tACSA = tcyc – tf – tDSR – tACSD (Note 2)

tACSA 285 — 162 — 113 — ns

56 Address Valid to Chip Select Time tAVCS 10 — 10 — 10 — ns

57 Address Valid to Data Three-State Time tAVDZ — 10 — 10 — 10 ns

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ELECTRICAL CHARACTERISTICS

TECHNICAL DATA A-13

Figure A-9 Expansion Bus Timing

E

CS E VALID

READ DATA

WRITE DATA

R/W, ADDRESS

CS AD VALID

11 12

4A

2

1

3 4B

9

17

18

39 21

29

19

57

56

54

55

50 51 52

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ELECTRICAL CHARACTERISTICS MC68HC11F1

A-14 TECHNICAL DATA

NOTES:

1. All timing is shown with respect to 20% VDD and 70% VDD, unless otherwise noted.

2. Signal production depends on software.

3. Assumes 200 pF load on all SPI pins.

Table A-8 Serial Peripheral Interface Timing

VDD = 5.0 Vdc ± 5%, VSS = 0 Vdc, TA = TL to TH

Num Characteristic Symbol 2.0 MHz 3.0 MHz 4.0 MHz Unit

Min Max Min Max Min Max

Operating Frequency

Master

Slave

fop(m)

fop(s)

dc

dc

1.0

2.0

dc

dc

1.5

3.0

dc

dc

2.0

4.0

MHz

MHz

1 Cycle Time

Master

Slave

tcyc(m)

tcyc(s)

2.0

500

——

2.0

333

——

2.0

250

——

tcyc

ns

2 Enable Lead Time

Master (Note 2)

Slave

tlead(m)

tlead(s)

—

250

——

—

240

——

—

200

——

ns

ns

3 Enable Lag Time

Master (Note 2)

Slave

tlag(m)

tlag(s)

—

250

——

—

240

——

—

200

——

ns

ns

4 Clock (SCK) High Time

Master

Slave

tw(SCKH)m

tw(SCKH)s

340

190

——

227

127

——

130

85

——

ns

ns

5 Clock (SCK) Low Time

Master

Slave

tw(SCKL)m

tw(SCKL)s

340

190

——

227

127

——

130

85

——

ns

ns

6 Data Setup Time (Inputs)

Master

Slave

tsu(m)

tsu(s)

100

100

——

100

100

——100

100

——

ns

ns

7 Data Hold Time (Inputs)

Master

Slave

th(m)

th(s)

100

100

——

100

100

——

100

100

——

ns

ns

8 Access Time (Time to Data Active from

High-Impedance State)

Slave ta 0 120 0 120 0 120 ns

9 Disable Time (Hold Time to High-Impedance State)

Slave tdis — 240 — 167 — 125 ns

10 Data Valid (After Enable Edge) (Note 3) tv(s) — 240 — 167 — 125 ns

11 Data Hold Time (Outputs) (After Enable Edge) tho 0 — 0 — 0 — ns

12 Rise Time (20% VDD to 70% VDD, CL = 200 pF)

SPI Outputs (SCK, MOSI, and MISO)

SPI Inputs (SCK, MOSI, MISO, and SS)

trm

trs

——

100

2.0

——

100

2.0

——

100

2.0

ns

ms

13 Fall Time (70% VDD to 20% VDD, CL = 200 pF)

SPI Outputs (SCK, MOSI, and MISO)

SPI Inputs (SCK, MOSI, MISO, and SS)

tfm

tfs

——

100

2.0

——

100

2.0

——

100

2.0

ns

ms

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ELECTRICAL CHARACTERISTICS

TECHNICAL DATA A-15

Figure A-10 SPI Master Timing (CPHA = 0)

Figure A-11 SPI Master Timing (CPHA = 1)

SEE

NOTE

NOTE: This first clock edge is generated internally but is not seen at the SCK pin.

SCK (CPOL = 0)

(OUTPUT)

SCK (CPOL = 1)

(OUTPUT)

MISO

(INPUT)

MOSI

(OUTPUT)

SS

(INPUT)

SEE

NOTE

MSB IN

BIT 6 - - - -1

LSB IN

MASTER MSB OUT MASTER LSB OUT

BIT 6 - - - -1

SS IS HELD HIGH ON MASTER

10 (ref)

5

1 13

4

5

12

13 12

4

6 7

11 10 11 (ref)

13 12

NOTE: This last clock edge is generated internally but is not seen at the SCK pin.

SCK (CPOL = 0)

(OUTPUT)

SCK (CPOL = 1)

(OUTPUT)

MISO

(INPUT)

MOSI

(OUTPUT)

SS

(INPUT)

MSB IN

BIT 6 - - - -1

LSB IN

MASTER MSB OUT MASTER LSB OUT

BIT 6 - - - -1

SS IS HELD HIGH ON MASTER

10 (ref)

1 13

4

5

12

12

13

4

6 7

11 10 11 (ref)

13 12

5

SEE

NOTE

SEE

NOTE

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ELECTRICAL CHARACTERISTICS MC68HC11F1

A-16 TECHNICAL DATA

Figure A-12 SPI Slave Timing (CPHA = 0)

Figure A-13 SPI Slave Timing (CPHA = 1)

NOTE: Not defined but normally MSB of character just received.

SCK (CPOL = 0)

(INPUT)

SCK (CPOL = 1)

(INPUT)

MISO

(OUTPUT)

MOSI

(INPUT)

SS

(INPUT)

MSB IN

BIT 6 - - - -1

LSB IN

SLAVE MSB OUT SLAVE LSB OUT

BIT 6 - - - -1

1 13

4

5

12

12

4

13 9

10 11

5

3

6 7 11

8

SEE

NOTE

2

NOTE: Not defined but normally LSB of character previously transmitted.

SCK (CPOL = 0)

(INPUT)

SCK (CPOL = 1)

(INPUT)

MISO

(OUTPUT)

MOSI

(INPUT)

SS

(INPUT)

MSB IN

BIT 6 - - - -1

LSB IN

SLAVE MSB OUT SLAVE LSB OUT

BIT 6 - - - -1

1 13

4

5

12

12

4

13 9

10 11

5

6 7

8

SEE

NOTE

2

10

3

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

ELECTRICAL CHARACTERISTICS

TECHNICAL DATA A-17

NOTES:

1. The RC oscillator (RCO) must be enabled (by setting the CSEL bit in the OPTION register) for EEPROM programming

and erasure when the E-clock frequency is below 1.0 MHz.

2. Refer to Reliability Monitor Report (current quarterly issue) for current failure rate information.

Table A-9 EEPROM Characteristics

VDD = 5.0 Vdc ± 10%, VSS = 0 Vdc, TA = TL to TH

Characteristic Temperature Range Unit

0 to 70,

– 40 to 85 –40 to 105 –40 to 125 ° C

Programming Time <1.0 MHz, RCO Enabled

(Note 1) 1.0 to 2.0 MHz, RCO Disabled

³ 2.0 MHz (or Anytime RCO Enabled)

10

20

10

15

Must use RCO

15

20

Must use RCO

20

ms

Erase Time (Note 1) Byte, Row and Bulk 10 10 10 ms

Write/Erase Endurance (Note 2) 10,000 10,000 10,000 Cycles

Data Retention (Note 2) 10 10 10 Years

A-18 TECHNICAL DATA

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TECHNICAL DATA B-1

APPENDIX BMECHANICAL DATA AND ORDERING INFORMATION

B.1 Pin Assignments

The MC68HC11F1 is available in the 80-pin plastic low profile quad flat pack (LQFP)

or the 68-pin plastic leaded chip carrier (PLCC). Refer to Table B-1 for ordering information.

Figure B-1 MC68HC11F1 68-Pin PLCC

Figure B-2 MC68HC11F1 80-Pin Quad Flat Pack

B.2 Package Dimensions

For case outlines please visit our website at http://design-net.sps.mot.com.

B.3 Ordering Information

Use the information in Table B-1 to specify the appropriate device when placing an

order.

Table B-1 Device Ordering Information

Description Package Temperature Frequency MC Order Number

NO ROM, 80-Pin LQFP – 40° to + 85° C 2 MHz MC68HC11F1CPU2

512 Bytes EEPROM, (14 mm X 14 mm, 3 MHz MC68HC11F1CPU3

1024 Bytes RAM 1.4 mm thick) 4 MHz MC68HC11F1CPU4

– 40° to + 105° C 2 MHz MC68HC11F1VPU2

3 MHz MC68HC11F1VPU3

4 MHz MC68HC11F1VPU4

– 40° to + 125° C 2 MHz MC68HC11F1MPU2

3 MHz MC68HC11F1MPU3

68-Pin PLCC – 40° to + 85° C 2 MHz MC68HC11F1CFN2

3 MHz MC68HC11F1CFN3

4 MHz MC68HC11F1CFN4

– 40° to + 105° C 2 MHz MC68HC11F1VFN2

3 MHz MC68HC11F1VFN3

4 MHz MC68HC11F1VFN4

– 40° to + 125° C 2 MHz MC68HC11F1MFN2

3 MHz MC68HC11F1MFN3

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

MC68HC11F1

B-4 TECHNICAL DATA

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

TECHNICAL DATA C-1

APPENDIX CDEVELOPMENT SUPPORT

C.1 MC68HC11F1 Development Tools

The following table and text provide a reference to development tools for the

MC68HC11F1 microcontrollers. For more complete information refer to the appropriate

manual for each system.

* For MC68HC11F1 support, the MMDS11 must be used with an MC68HC11F1 emulator module.

C.2 MC68HC11EVS — Evaluation System

The EVS is an economical tool for designing, debugging, and evaluating target systems

based on the MC68HC11F1 MCU. The two printed circuit boards that comprise

the EVS are the MC68HC11F1EM emulator module (EM) and the M68HC11PFB platform

board (PFB).

• Monitor/debugger firmware

• One-line assembler/disassembler

• Host computer download capability

• Dual memory maps:

— 64 Kbyte monitor map that includes 16 Kbytes of monitor EPROM

— MC68HC11F1 user map that includes 64 Kbytes of emulation RAM

• OTPROM, EPROM, and EEPROM MCU programmer

• MCU extension I/O port for single-chip, expanded, and special-test operation

modes

• RS-232C terminal and host I/O ports

• Logic analyzer connector

C.3 M68MMDS11 — Modular Development System for M68HC11 Devices

The M68MMDS11 Freescale Modular Development System (MMDS11) is a tool for developing

embedded systems based on M68HC11 MCUs. The MMDS11 is an emulator

system that provides an on-screen bus state analyzer and real-time memory monitoring

windows. An integrated design environment includes an editor, an assembler, the

user interface, and source-level debug capability. These features significantly reduce

the time necessary to develop and debug an embedded MCU system. The compact

unit requires minimum space.

• Real-time, non-intrusive, in-circuit emulation

• Assembly-language source-level debugging

Table C-1 MC68HC11F1 Development Tools

Device Evaluation Systems Modular Development Systems

MC68HC11F1 M68HC11F1EVS MMDS11*

Freescale Semiconductor, I

Freescale Semiconductor, Inc.

For More Information On This Product,

Go to: www.freescale.com

nc...

MC68HC11F1

C-2 TECHNICAL DATA

• Built-in real-time bus state analyzer:

— 8 Kbyte x 64 real-time trace buffer

— Four hardware triggers control real-time bus analysis, provide breakpoints

— Nine triggering modes

— Display of real-time trace data as raw data, disassembled instructions, raw

data and disassembled instructions, or assembly-language source code

— As many as 8190 pre- or post-trigger points

— Trace buffer can be filled while single-stepping through user software

— 16-bit or 24-bit time tag

— Programmable time tag clock source

— 16 general-purpose logic clips, four can trigger bus state analyzer sequencer

• 64 Kbytes of emulation memory

• 32-byte block of real-time memory, can be mapped within a 1 Kbyte window anywhere

in the 64 Kbyte M68HC11 memory map.

• Six software-selectable oscillator clock sources: five internally generated frequencies

or an external frequency on a logic clip

• 64 possible hardware instruction breakpoints over the 64 Kbyte M68HC11 memory

map or a one megabyte bank selected memory map.

• Four data breakpoints (hardware breakpoints). A data breakpoint can be qualified

by an address, an address range, data, or clips.

• SCRIPT command for automatic execution of MMDS11 command sequences

• Command and response logging

• A DOS personality file for each EM that the MMDS supports. Each personality file

provides a foreground memory-map description and a chip information file.

• CHIPINFO command provides memory-map, vectors, registers, and pin-out information

contained in the personality file

• Latch-up resistant design makes power-up sequencing unimportant.

• RS-232 operation speeds as high as 57.6 Kbaud

• On-screen, context-sensitive help

• Mouse or keyboard control of software

• Built-in power supply

• Compact size: 15.38 inches (390.6 mm) deep, 10.19 inches (258.83 mm) wide,

and 2.75 inches (69.85 mm) high. Station module weighs 6.0 pounds (2.72 kg).